3GPP TSG SA WG2 Meeting #155	S2-2302637

# February 20 - 24, 2023, Athens, Greece									(Revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1268 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS DC capability discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following conclusions are agreed for IMS DC capability discovery in the TR 23.700-87: - The network discovers the UE support of DC based on existing mechanism as specified in TS 26.114 [6] and TS 24.229 [8]. - The UE discovers home network support for DC via the Feature-Caps header field in 200 OK during registration, as specified in solution #4. - The discovery of UE capabilities required to use data channel applications is done via application level communication through the bootstrap data channel and is transparent to IMS. TS 23.228 needs to be updated to support mutual IMS DC capability discovery based on the above conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is proposed to document how UE and IMS network discovers the DC capability of each other. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

Annex XX (normative):
Support of Data Channel services in IMS

XX.Y	Procedures

XX.Y.Z	Procedures to support IMS DC capability discovery

The IMS network and the UE needs to discover the capability of supporting IMS data channel mutually.

IMS data channel capabilitiy discovery includes two aspects:

-	The network discovers the data channel capability of the UE, including the local UE and the remote UE.

-	The UE discovers the data channel capability of the network, including the local network and the remote network.

When the UE supporting IMS data channel registers on the IMS network, it includes a +sip.app-subtype media feature tag with a value of "webrtc-datachannel" as specified in TS 26.114 [76] in the Contact header field of initial Register request to allow the home IMS network discovers its IMS data channel capability.

When the UE supporting IMS data channel initiates a IMS session,  it includes a +sip.app-subtype media feature tag with a value of "webrtc-datachannel" as specified in TS 26.114 [76] in the Contact header field of initial INVITE request to allow the remote IMS network discovers its IMS data channel capability.

If the IMS network supports IMS data channel, the S-CSCF includes a Feature-Caps header field defined in TS 24.229 [10a], with the "+g.3gpp.webrtc-datachannel" header field parameter in the 200 OK response to initial registration, which is used by the UE to discver the IMS data channel capability of its home IMS network.

**** End of Changes ****