3GPP TSG-WG SA2 Meeting #155	S2- 2302661

# Athens, Greece, February 20 – 24, 2023		(revision of S2-22xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0301 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for MO-LR Ranging and Sidelink Positioning service exposure to UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure for Ranging/SL Positioning service exposure to UE is necessary and shall be specified in the normative phase, based on the conclusion of KI#6 in TR 23.700-86. From the perspective of “functional description and information flows”, it adds the MO-LR procedure with Sidelink Positioning for relative positioning as one basic procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the MO-LR procedure with Sidelink Positioning for relative positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MO-LR procedure with Sidelink Positioning for relative positioning is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new) * * * *

6.2.X	MO-LR procedure with Sidelink Positioning for relative positioning

SL Positioning Client UE

NG-RAN

AMF1

LMF

2. UL NAS TRANSPORT (MO-LR Request)

GMLC

UE1

UE2

6. DL NAS TRANSPORT

1. UE Triggered Service Request

AMF2

UDM

NRF

3. Ngmlc_Ranging_ProvideLocation_Request

4. Steps 4-13 of MT-LR with Sidelink Positioning for relative positioning in clause 6.1.x

5. Ngmlc_ Ranging _ProvideLocation_Response

Figure 6.2.X-1: MO-LR procedure with Sidelink Positioning for relative positioning

1.	The application layer of the SL Positioning Client UE triggers the Ranging/SL positioning, and then requests the network to perform the Ranging/SL positioning between the UE1 and UE2. If the SL Positioning Client UE in CM-IDLE state, it firstly triggers the service request procedure to enter CM-CONNECTED state.

2.	The SL Positioning Client UE sends the MO-LR request to the AMF1 for the Ranging/SL Positioning operation between the UE1 and UE2.

The MO-LR request includes the following parameters for the Ranging/SL positioning service requirement: Reference UE ID (e.g. UE1), Target UE ID (e.g. UE2), Ranging/SL Positioning service type, Requested Quality, etc. The Ranging/SL Positioning service type indicates the distance between two UEs or more UEs and/or the direction and/or relative positioning of one UE (i.e.Target UE) from another UE (i.e. Reference UE) via PC5 interface. The Requested Quality may include delay, accuracy, etc. The SL Positioning Client UE may suggest the initiator UE (i.e. which is used to perform signalling exchange with the LMF) based on implementation and indicate which UE is the initiator UE to the AMF1.

3.	The AMF1 may select a GMLC based on NRF query or configuration in AMF1, and then acts as an LCS client to request the GMLC for Ranging/SL Positioning result via Ngmlc_Ranging_ProvideLocation_Request, the AMF includes the SL Positioning Client UE ID in the request.

4.	Steps 4-13 of MT-LR with Sidelink Positioning for relative positioning in clause 6.1.x are performed. In this step GMLC selects the AMF (i.e., AMF2) serving either UE1 or UE2.

5.	The GMLC returns the result to the AMF1 via Ngmlc_ Ranging _ProvideLocation_Response.

6.	The AMF1 returns the result to the SL Positioning Client UE.

# * * * * End of changes * * * *