SA WG2 Meeting #S2-154AH-e	S2-2302719

16 - 20 January 2023, Electronic, Elbonia	(revision of S2-2302096/S2-2300678)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0170 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification shared NG-U Termination among gNBs for Broadcast MBS session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Nokia, Nokia Shanghai-Bell, Ericson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2023-02-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the SA2#153E, CR#0141 was approved on shared delivery tunnel management for common gNB UP case in multicast MBS case to align with RAN based on RAN input in LS S2-2208148. And the LS reply S2-2209285 was sent back to CT4 and RAN3. In this meeting, CT4 asks SA2 in their LS response S2-2300035/C4-225556 whether it is correct to assume the shared NG-U termination also applies to broadcast MBS Session (see below): CT4 would like to apprise SA2 that CT4 has assumed that the shared NG-U terminations handling is applicable for a Broadcast MBS Session as well, and has agreed a CR related to the impact of shared NG-U terminations on Broadcast MBS Session restoration. The common gNB UP could apply to broadcast MBS Sessions, however, it is our view that RAN3 input/confirmation is required (as for alignment in SA2 was done for common gNB UP for multicast MBS Session). Considering that RAN3’s next meeting is in Feb, if SA2 waits for RAN3’s input, the update in SA2 will happen in April the earliest. To avoid any potential delay, this CR (i.e., CR0169) could be conditionally approved in SA2 and the approval in SA plenary in March will require RAN3’s confirmation. In addition, in clause 6.9.1, there is inconsistency between multicast MBS Session Context and broadcast MBS Session regarding MBS IP distribution information, e.g. “TEID for data distribution” refers to TEID of MB-UPF for multicast N3mb in multicast MBS Session Context, but refer to TEID at NG-RAN in broadcast MBS Session Context which is incorrect. In the Rev1, Step 12 was mistakenly changed to Step 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Broadcast MBS session shared delivery tunnel management for common gNB UP. Correct the MBS Session context in clause 6.9.1 to align the parameters related unicast N3mb transport or multicast N3mb transport between the multicast and broradcast MBS Session context. Rev2: Recover the step number in section 7.3.1 (step 12). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to handle common gNB UP for broadcast MBS Session. Incorrect stage 2 requirement for N3mb transport in MBS Session Context and inconsistency between multicast MBS Session context and broadcast MBS Session context |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.1, 7.3.1, 7.3.3, 7.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR could be conditionally approved in SA2 and the approval in SA plenary in March will require RAN3’s confirmation. This CR (if approved) needs to be implemented before CR#0157 which update the same step in clause 7.3.6. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 6.9.1	MBS Session Context

The MBS Session Context contains all information describing a particular MBS session in the 5GS and is created in each node involved in the delivery of the MBS data.

The content of the Multicast MBS Session Context is described in Table 6.9.1-1.

Table 6.9.1-1: Multicast MBS Session Context

| Parameter | Description | NG-RAN | AMF | SMF | MB-SMF |
| --- | --- | --- | --- | --- | --- |
| State | State of MBS session ('Active' or 'Inactive' or 'Configured') | X (note 2) |  | X (note 2) | X |
| SSM (source specific IP multicast address) | IP multicast address identifying the MBS session. |  |  | X (note 1) | X (note 1) |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | x | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X | X (note 1) | X (note 1) |
| MB-SMF | The MB-SMF that handles the MBS session. |  | X | X |  |
| QoS information | QoS information of the MBS session. | X |  | X | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X (note 1) |  | X (note 1) | X (note 1) |
| NG-RAN Node ID(s) | NG-RAN nodes which are involved in the Multicast MBS session |  | X |  | X (note 1, note 4) |
| AMF | The AMF(s) which are selected for the MBS session | X |  |  | X |
| IP multicast and source address for data distribution | IP addresses identifying the SSM user plane transport for shared delivery from MB-UPF to NG-RAN and for individual delivery from MB-UPF to UPF when the IP multicast transport is used. | X (note 1) |  | X (note 1) | X (note 1) |
| TEID for IP multicast distribution | Tunnel ID allocated by MB-UPF used for receiving the multicast data for shared delivery by NG-RAN and for individual delivery by UPF when the IP multicast transport is used. | X |  | X | X (note 1) |
| SMF | The SMF(s) that manages the associated PDU session. |  |  |  | X |
| UE ID | ID identifying the UE that successfully join the Multicast MBS session. For NG-RAN it is NGAP UE ID and for SMF it is SUPI. | X (note 3) |  | X (note 3) |  |
| NG-RAN IP unicast distribution | The IP addresses and TEID of NG-RAN used for the user plane between NG-RAN and MB-UPF and between MB-UPF and UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) | X (note 1, note 4) |
| PCF | The MB-PCF that provides policy control for the MBS session. |  |  |  | X (note 1) |
| NOTE 1: It is an optional parameter. NOTE 2: The value 'Configured' is not applicable for NG-RAN and SMF. NOTE 3: the UE ID is available within the UE Context which contains the MBS information. NOTE 4: The Parameter needs to be stored in deployments with shared NG-U termination(s) if unicast transport is used. |  |  |  |  |  |

In Broadcast MBMS mode, an MBS Session Context is created in the NG-RAN, AMF, MB-SMF and MBSF as a result of the MBS Session Start procedure.

The content of the Broadcast MBS Session Context is described in Table 6.9.1-2.

Table 6.9.1-2: Broadcast MBS Session context

| Parameter | Description | NG-RAN | AMF | MB-SMF |
| --- | --- | --- | --- | --- |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) |
| AMF | The AMF(s) which are selected for the MBS session | X |  | X |
| MB-SMF | The MB-SMF that handles the MBS session. |  | X |  |
| QoS information | QoS information for the MBS Session, including the QoS parameters of QoS flows. | X |  | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X | X | X |
| NG-RAN Node ID(s) | NG-RAN nodes which are selected for the Broadcast MBS session |  | X | X (note 1, note 2) |
| IP multicast address for data distribution | IP addresses identifying the SSM user plane transport used for shared delivery from MB-UPF to NG-RAN when the IP multicast transport is used. | X (note 1) |  | X (note 1) |
| TEID for IP multicast distribution | Tunnel ID allocated by MB-UPF used for receiving the broadcast data for shared delivery by NG-RAN when the IP multicast transport is used. | X |  | X (note 1) |
| NG-RAN IP unicast distribution | IP address and TEID of NG-RAN used for the user plane from NG-RAN to MB-UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1, note 2) |
| PCF | The PCF that provides policy control for the MBS session. |  |  | X (note 1) |
| MBS FSA ID | MBS Frequency Selection Area (FSA) ID is used for broadcast MBS sessions to guide the frequency selection of the UE. | X |  | X |
| NOTE 1: It is an optional parameter. NOTE 2: The Parameter needs to be stored in deployments with shared NG-U termination(s) if unicast transport is used. |  |  |  |  |

Next CHANGE

### 7.3.1	MBS Session Start for Broadcast

The Broadcast MBS Session Start follows the common procedure specified in clause 7.1.1.2 or clause 7.1.1.3, which consist of TMGI Allocation and MBS Session Create. It is possible for AF to allocate TMGI once but create the MBS Session for multiple times. A combined procedure to perform both TMGI allocation and MBS Session Create is available.

The TMGI Allocation is used by AF to obtain the TMGI as MBS Session ID (i.e. TMGI) and perform service announcement towards UEs.

The MBS Session Create (with MBS service type set to broadcast service) is used by the AF to indicate the impending start of the transmission of MBS data, and to provide the session attributes, so that resources for the MBS Session are set up in the MB-UPF and in the NG-RAN for 5GC Shared MBS traffic delivery. The MBS Session Create can be used if TMGI has not been allocated. In this case, MB-SMF will allocate a unique TMGI for the AF and then start the MBS Session.

NOTE 1:	When the multicast transport between NG-RAN and MB-UPF is described below, source specific multicasting is assumed.

To receive the data of broadcast communication service, the UE is either preconfigured with needed configuration (e.g. USD as defined in TS 26.346 [13]) for the UE to receive MBS service, or provisioned with the configuration of Broadcast MBS session on application level (service announcement; the configuration may for instance be performed using SIP signalling, or methods described in TS 26.346 [13]). If the needed configuration is pre-configured, the UE does not need to interact with network.

Figure 7.3.1-1: MBS Session Establishment for Broadcast

0	Based on OAM configuration, RAN nodes announce in SIBs over the radio interface information about the MBS FSA IDs and frequencies of neighbouring cells.

1.	To establish broadcast MBS session, the AF performs TMGI allocation and MBS session creation as specified in clause 7.1.1.2 or 7.1.1.3. The MBS service type indicates to be broadcast service. The MBS FSA ID(s) of a broadcast MBS session are communicated in the service announcement towards the UE. The UE compares those MBS FSA IDs(s) with the MBS FSA ID(s) in SIBs for frequency selection.

2.	The MB-SMF may use NRF to discover the AMF(s) supporting MBS based on the MBS service area and select the appropriate one(s). Then the MB-SMF sends the Namf_MBSBroadcast_ContextCreate (TMGI, N2 SM information ([LL SSM], 5G QoS Profile), MBS service area, [MBS FSA ID(s)]) messages to the selected AMF(s) in parallel if the service type is broadcast service. The MB-SMF may include a maximum response time in the request.

3.	The AMF transfers the MBS Session Resource Setup Request message, which contains the N2 SM information in the received Namf_MBSBroadcast_ContextCreate Request to all NG-RANs which support MBS in the MBS service area. The AMF includes the MBS service area.

4.	NG-RAN creates a Broadcast MBS Session Context and stores the TMGI and the QoS Profile in the MBS Session Context. The LL SSM are optional parameters and only provided by MB-SMF to NG-RAN if N3mb multicast transport is configured to be used in the 5GC. If MBS FSA ID(s) were received, the NG-RAN may use those MBS FSA ID(s)s to determine cells/frequencies within the MBS service area to broadcast MBS session data based on OAM configuration about the MBS FSA IDs and related frequencies.

5.	If NG-RAN prefers to use N3mb multicast transport (and if LL SSM is available in NG-RAN), the NG-RAN joins the multicast group (i.e. LL SSM).

If NG-RAN prefers to use N3mb unicast transport (or if the LL SSM is not available in NG-RAN) between the NG-RAN and MB-UPF, NG-RAN provides its N3mb DL Tunnel Info.

6.	The NG-RAN reports successful establishment of the MBS Session resources (which may include multiple MBS QoS Flows) by sending MBS Session Resource Setup Response (TMGI, N2 SM information ([N3mb DL Tunnel Info])) message(s) to the AMF. N3mb DL Tunnel Info is only available when unicast transport applies between MB-UPF and NG-RAN. For more details, refer to TS 38.413 [15].

7.	The AMF transfers the Namf_MBSBroadcast_ContextCreate Response () to the MB-SMF. The AMF should respond success when it receives the first success response from the NG-RAN(s). And if all NG-RAN(s) report failure, the AMF should respond failure. The MB-SMF stores the AMF(s) which responds success in the MBS Session Context as the downstream nodes. If the AMF receives the NG-RAN response(s) from all involved NG-RAN(s), the AMF should include an indication of completion of the operation in all NG-RANs. If received from NG-RAN node, the N3mb DL Tunnel Info and NG-RAN ID are included in the response.

8.	[Conditional] If N3mb unicast transport is to be used (i.e. N3mb DL Tunnel Info is present in the Namf_MBSBroadcast_ContextCreate Response message from AMF) in a deployment where NG-RAN nodes share a common user plane entity, the MB-SMF only establishes the shared tunnel towards the DL GTP tunnel endpoint if the shared tunnel has not yet been established (as determined based on the stored DL GTP Tunnel endpoint(s) for the MBS session). The MB-SMF also stores the received DL GTP Tunnel and corresponding NG-RAN Node ID for the MBS session. For the deployment where NG-RAN nodes do not share a common user plane entity, the DL GTP tunnel will not be used by other NG-RAN nodes. In a deployment where NG-RAN nodes do not share a common user plane entity, the MB-SMF always establishes the shared tunnel towards the DL GTP tunnel endpoint.

To establish the shared tunnel towards the DL GTP tunnel endpoint, the MB-SMF sends an N4mb Session Modification Request to the MB-UPF to allocate the N3mb unicast transport tunnel for a replicated MBS stream for the MBS Session.

9.	NG-RAN broadcasts the TMGI representing the MBS service over radio interface. Step 9 can take place in parallel with step 6.

10.	Another NG-RAN may report successful establishment of the MBS Session resources (which may include multiple MBS QoS Flows) by sending MBS Session Resource Setup Response (TMGI, N2 SM information ([N3mb DL Tunnel Info])) message after the AMF transferred the Namf_MBSBroadcast_ContextCreate Response () to the MB-SMF.

11.	The AMF transfers the Namf_MBSBroadcast_ContextStatusNotify request () to the MB-SMF. When the AMF receives the response from all NG-RAN nodes, the AMF includes an indication of the completion of the operation. If the AMF does not receive responses from all NG-RAN nodes before the maximum response time elapses since the reception of the Namf_MBSBroadcast_ContextCreate Request, then the AMF should transfer the Namf_MBSBroadcast_ContextStatusNotify request () which indicates partial success or failure.

12.	[Conditional] If N3mb unicast transport is to be used (i.e. N3mb DL Tunnel Info is present in the MBS Session Start Response message from AMF), the handling in MB-SMF and MB-UPF is the same as specified in step 8

13.	The AF starts transmitting the DL media stream to MB-UPF using the N6mb Tunnel, or optionally un-tunnelled i.e. as an IP multicast stream using the HL MC address.

14.	The MB-UPF transmits the media stream to NG-RAN via N3mb multicast transport or unicast transport.

15.	The NG-RAN transmits the received DL media stream using DL PTM resources.

NOTE 2:	Step 6-8 and 2-4 are comparable to step 2-5 and 6-7 in clause 7.2.1.4, respectively.

Next CHANGE

### 7.3.3	MBS Session Update for Broadcast

The MBS Session Update for broadcast is used by the AF to update the broadcast area or service requirements of the MBS Session which may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s).

Figure 7.3.3-1: MBS Session Update for Broadcast

1.	The AF starts MBS session update procedure by sending Nnef_MBSSession_Update Request to the NEF/MBSF with TMGI. The AF may adjust service requirement and/or broadcast area. The service requirements adjustment may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s).

2.	The MB-SMF sends Namf_MBSBroadcast_ContextUpdate Request (TMGI, N2 SM information (MBS Session ID, [MBS QoS profile], [service area(s)], [Area Session ID(s)], [MBS IP Multicast Tunnel Info(s)], [MBS FSA ID(s)]), [updated MBS service area]) to the AMFs. For a location dependent service, the MB-SMF may provide information about several location areas. If the broadcast area is updated, the MB-SMF may use NRF to discover the AMF(s) based on the new broadcast area and select the appropriate one(s). The MB-SMF may include a maximum response time in the request.

Depending on the change of the MBS service area, the MB-SMF may send Namf_MBSBroadcast_ContextCreate to some AMFs in the new MBS service area, Namf_MBSBroadcast_ContextRelease to some other AMFs in the old MBS service area.

3.	The AMF sends MBS Session Resource Update to NG-RANs with TMGI, the updated 5G QoS Profile and the updated MBS service area.

Depending on the change of the MBS service area, the AMF may send MBS Session Resource Setup to some NG-RANs in new MBS service area (see clause 7.3.1) and MBS Session Resource Release to some other NG-RANs in old MBS service area.

4.	The NG-RAN updates MBS Session Context.

5.	The NG-RAN reports successful update of the MBS Session resources (which may include multiple MBS QoS Flows) by sending MBS Session Resource Update Response (TMGI, N2 SM information ([N3mb DL Tunnel Info])) message(s) to the AMF. N3mb DL Tunnel Info is only available when unicast transport applies between MB-UPF and NG-RAN and the NG-RAN wants the transport to be changed. The NG-RAN should be ready to receive using the N3mb DL tunnel. For more details, refer to TS 38.413 [15].

6.	The AMF sends Namf_MBSBroadcast_ContextUpdate Response to the MB-SMF. If the AMF received the NG-RAN responses from all involved NG-RAN(s), the AMF should include an indication of completion of the operation in all NG-RANs. If recieved from NG-RAN node, the N3mb DL Tunnel Info and corresponding NG-RAN ID are included in the response.

6a.	[Conditional] If MBS service area is changed in such a manner that NG-RAN nodes are added or removed from handling the MBS session, and N3mb unicast transport is used.

-	In a deployment where NG-RAN nodes share a common user plane entity, the MB-SMF perform as following:

-	For the NG-RAN node to be added, the the MB-SMF only establishes the shared tunnel towards the DL GTP tunnel endpoint if the shared tunnel has not yet been established (as determined based on the stored DL GTP Tunnel endpoint(s) for the MBS session). The MB-SMF also stores the received DL GTP Tunnel and corresponding NG-RAN Node ID for the MBS session.

-	For the NG-RAN node to be removed, the MB-SMF removes the received NG-RAN Node ID and possibly DL GTP tunnel endpoint from the stored NG-RAN Node ID(s) for the DL GTP tunnel endpoint for the MBS session, and checks whether the DL GTP tunnel is in use by other NG-RAN nodes based on the stored NG-RAN Node ID(s) for DL GTP tunnel endpoint for the MBS session. If the related DL GTP tunnel is not in use by other NG-RAN nodes, the N3mb unicast transport tunnel shall be removed.

-	In a deployment where NG-RAN nodes do not share a common user plane entity, the MB-SMF always establishes or releases the shared tunnel towards the DL GTP tunnel endpoint if NG-RAN nodes are added or removed from handling the MBS session.

If the N3mb unicast transport tunnel need to be added or removed, the SMF sends an N4mb Session Modification Request to the MB-UPF to establish or release the N3mb unicast transport tunnel for establishing, or releasing the MBS stream for the MBS Session.

7.	The NG-RAN updates the MBS Session. It takes place in parallel with step 5 to step 6.

8	Another NG-RAN may report successful update of the MBS Session resources (which may include multiple MBS QoS Flows) by sending MBS Session Resource Update Response (TMGI, N2 SM information ([N3mb DL Tunnel Info])) message after the AMF transferred the Namf_MBSBroadcast_ContextUpdate Response () to the MB-SMF. N3mb DL Tunnel Info is only available when unicast transport applies between MB-UPF and NG-RAN and the NG-RAN wants the transport to be changed. The NG-RAN should be ready to receive using the N3mb DL tunnel. For more details, refer to TS 38.413 [15].

9.	The AMF transfers the Namf_MBSBroadcast_ContextStatusNotify request () to the MB-SMF. When the AMF receives the response from all NG-RAN nodes, the AMF should include an indication of completion of the operation in all NG-RANs. If the AMF does not receive responses from all NG-RAN nodes before the maximum response time elapses since the reception of the Namf_MBSBroadcast_ContextUpdate Request, then the AMF should transfer the Namf_MBSBroadcast_ContextStatusNotify request () which indicates partial success or failure. If received from NG-RAN node, the N3mb DL Tunnel Info and corresponding NG-RAN ID are included in the request.

10.	[Conditional] If MBS service area is changed in such a manner that NG-RAN nodes are added or removed from handling the MBS session, and N3mb unicast transport is used, the MB-SMF establishes or releases the shared delivery tunnel via unicast N3mb transport for the MBS Session in the same manner as described in step 6a.

Next CHANGE

### 7.3.6	Broadcast MBS Session Release Require

When NG-RAN is not able continue to provide the Broadcast MBS Session, e.g. due to lack of radiou resources, NG-RAN may intitiate Broadcast Session Release Require procedure.

Figure 7.3.6-1: Broadcast MBS Session Release Require

1.	If NG-RAN cannot continue to provide the Broadcast MBS session, e.g. due to lack of radio resources, the NG-RAN sends Broadcast Session Release Require (MBS Session ID) to the AMF.

2.	The AMF initiates Broadcast Session Release towards the NG-RAN as defined in steps 4 - 7 in clause 7.3.2.

3.	If unicast transport applies in N3mb, the AMF receives the DL tunnel Info for the Broadcast MBS Session from the NG-RAN in step 2. The AMF notifies the MB-SMF about the DL tunnel release via Namf_MBSBroadcast_ContextStatusNotify with N3mb DL Tunnel Info and corresponding NG-RAN ID.

4.	The MB-SMF performs N4mb Session Update to let MB-UPF stop the broadcast data forwarding towards the indicated DL tunnel and release the DL tunnel as specified in the step 6a of clause 7.3.3.

END OF CHANGES