3GPP TSG-WG SA2 Meeting #155 	S2-2302748

February 20 – 24, 2023, Athens, Greece	(revision of S2-220xxxx)

Source:	vivo

Title:	TS 23.586: Procedures for Ranging service exposure to UE.

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper proposes add the Procedures for Ranging service exposure to UE.

1. Introduction/Discussion

This paper implements the procedure of Ranging and sidelink positioning service exposure to a UE through PC5 based on the following conclusions in KI#6 (Ranging and sidelink positioning service exposure to a UE) of TR23.700-86:

For Ranging and sidelink positioning service exposure to a UE through PC5, following principles are taken as baseline for normative work:

-	SL Positioning Client UE discovers one of Reference UE and Target UE.

-	SL Positioning Client UE invokes the Ranging/Sidelink Positioning service to Reference UE/Target UE that is discovered by sending a anging and sidelink positioning service request for obtaining the Ranging and SL positioning result between Reference UE and Target UE. This request includes the user info of SL Positioning Client UE, Reference UE and Target UE.

-	Reference UE/Target UE that is contacted by the SL Positioning Client UE triggers the authorization of SL Positioning Client UE for the ranging and sidelink positioning service invocation.

NOTE 1:	The procedure for Privacy handling on Ranging and sidelink positioning service exposure needs to be determined by SA3 Group i.e. the authorization of the SL positioning client UE for Ranging and sidelink positioning service invocation of two other UEs.

-	The Ranging/Sidelink positioning procedure between Reference UE and Target UE is performed based on the conclusion of KI#4.

-	Reference UE/Target UE that is contacted by the SL Positioning Client UE provides the Ranging/Sidelink positioning result to SL Positioning Client UE.

NOTE 2:	Other information to be included in the interaction messages between SL Positioning Client UE and Reference UE/Target UE contacted by the SL Positioning Client UE is determined in normative work.

-	For the enhanced MO-LR case:

-	SL positioning client UE sends an enhanced MO-LR request to AMF;, the AMF sends a the Ranging/SL positioning request to the GMLC which also includes the SL positioning client UE ID.

-	GMLC reuses the procedure described in the conclusion for clause 8.7 (like the request from Application server), and sends the Ranging/SL positioning result to AMF.

-	The AMF sends the Ranging/SL positioning result to the SL positioning client UE when it receives the result from the GMLC.

NOTE 4:	The procedure for Privacy handling on Ranging and sidelink positioning service exposure needs to be determined by SA3 Group i.e. the authorization of the SL positioning client UE for Ranging and sidelink positioning service invocation of two other UEs.

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

## 6.X	Procedures for Ranging/SL positioning service exposure to UE

### 6.X.1	General

The Procedures for Ranging/SL positioning service exposure to UE can be initiated over the PC5 interface in clause 6.X.2 or through the network in clause 6.X.3.

### 6.X.2	Ranging/SL positioning service exposure to UE over PC5

Figure 6.X.2-1 illustrates the general Ranging/SL positioning service exposure to UE over PC5. To perform the Ranging/SL positioning service invocation, the SL positioning client UE is configured with the related information as described in clause 5.1.

Figure 6.X.2-1: Ranging/SL positioning service exposure to UE over PC5 Procedure

1)	SL positioning client UE performs the Target UE discovery procedure over the PC5 interface. The Target can be discovered directly via PC5 or indirectly via a Reference UE.

2)	If the Target UE is discovered directly via the PC5, the SL positioning client UE invokes the Ranging/SL positioning service from the Target UE by sending a Ranging/SL positioning service request to the Target UE via the PC5 message. The Source L2 ID of the Ranging/SL positioning service request is self-assigned and the Destination L2 ID is set to the Source L2 ID of the Target UE used in the discovery procedure. The Ranging/SL positioning service request includes the Ranging/SL positioning service requirement, e.g., the QoS, the Target UE ID and the SL positioning client UE. The Ranging/SL positioning service request also includes the Ranging/SL positioning result type indication that indicates which Ranging/SL positioning result type the SL positioning client UE want to obtain, e.g., the relative distance, the absolute position, relative position of the Target UE. If it is for the relative distance, relative position of the Target UE, the Ranging/SL positioning service request may also includes the Reference UE ID.

If the Target UE is discovered is discovered indirectly via a Reference UE, the SL positioning client UE invokes the Ranging/SL positioning service from the Reference UE by sending a Ranging/SL positioning service request to the Reference UE via the PC5 message. The Source L2 ID of the Ranging/SL positioning service request is self-assigned and the Destination L2 ID is set to the Source L2 ID of the Reference UE used in the discovery procedure. The Ranging/SL positioning service request includes the Ranging/SL positioning service requirement, e.g., the QoS, the Target UE ID, the SL positioning client UE and the Reference UE ID. The Ranging/SL positioning service request also includes the Ranging/SL positioning result type indication to indicate which Ranging/SL positioning result type the SL positioning client UE want to obtain, e.g., the relative distance, the absolute position, relative position of the Target UE.

3)	Target UE and Reference UE performs the Privacy check which determines whether the SL positioning client UE is authorized to invoke the Ranging/SL positioning service from the Target UE and Reference UE.

4)	Target UE and Reference UE perform the Ranging/SL positioning control operations.

5)	Target UE or Reference UE responds to the SL positioning client UE the Ranging/SL positioning results by sending a PC5 message Ranging/SL positioning service response.

### 6.X.3	Ranging/SL positioning service exposure to UE through 5GC

Figure 6.X.2-1 illustrates the general Ranging/SL positioning service exposure to SL positioning client UE though 5GC.

1)	SL positioning client UE determines to invoke the Ranging/SL positioning service through 5GC by sending an MO-LR request which additionally includes the Ranging/SL positioning invocation info including the Target UE(s) ID, Ranging service requirement.

2)	SL positioning client UE’s serving AMF determines the request from the SL positioning client UE is for Ranging/SL positioning service invocation based on the Ranging/SL positioning invocation info, then sends a Ranging service request to the GMLC in the Ngmlc_Location_ProvideLocation request, the Ranging service request includes the Target UE(s) ID and the Ranging service requirement.

3)	GMLC invoke the Nudm_SDM_Get and Nudm_UECM_Get service to determine that the SL positioning client UE is authorized to invoke the Ranging service from the Target UE.

4)	After privacy check, the GMLC determines which Target UE to act as the initiator UE for Ranging service exposure, and then sends a Ngmlc_Location_ProvideLocation request including the Target UE(s) ID and the Ranging service requirement to the selected Target UE’s serving AMF.

5)	The selected Target UE’s serving AMF triggers Target UE’s LMF to perform the Network initiated Ranging/SL positioning procedure for invoking the Ranging service from the selected Target UE.

6)	The LMF initiates the network initiated Ranging/SL positioning procedure, to triggers that the Target UE performs the Ranging/SL positioning control operations and then gets Ranging result from the selected Target UE.

7)	The LMF provides the Ranging result to the Target UE’s serving AMF in the Nlmf_Location_DetermineLocation response.

8)	The Target UE’s serving AMF forwards the Ranging result to the GMLC using the Ngmlc_Location_ProvideLocation response message.

9)	The GMLC returns the Ranging result to the SL positioning client UE’s serving AMF.

10)	The SL positioning client UE’s serving AMF sends the Ranging result to the SL positioning client UE in MO-LR response.

# * * * * End of changes * * * *