3GPP TSG-SA2 Meeting #155	S2-2302800

Athens, Greece, February 20 - 24, 2023	(revision of S2-2300998)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3825 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Alternative S-NSSAI replacement determined by NSSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As part of the FS_eNS_Ph3 study, the TR 23.700-41 V2.0.0 has concluded in clause 8.1 for KI#1 that an Alternative S-NSSAI can be determined in the AMF or in the NSSF. This CR attempts to implement the changes needed for the case when the NSSF determines the Alternative S-NSSAI. The detailed analysis of the NSSF functionality for the Alternative S-NSSAI creation is described in the discussion paper S2-2302426. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to specify the enhancements to the NSSF functionality resulting from TR 23.700-41 clause 8.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusion from the FS_eNS_Ph3 will not be implemented in the normative specificaitons. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.16.2.1, 5.2.16.3.1, 5.2.16.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

#### 5.2.16.2	Nnssf_NSSelection service

##### 5.2.16.2.1	Nnssf_NSSelection_Get service operation

Service operation name: Nnssf_NSSelection_Get

Description: This service operation enables Network Slice selection in both the Serving PLMN and HPLMN. It also enables the NSSF to provide to the AMF the Allowed NSSAI and the Configured NSSAI for the Serving PLMN. It allows also to provide the NSAG information which is applicable (clause 5.15.14 of TS 23.501 [2]). It allows also to provide to the AMF the Alternative S-NSSAI information (clause 5.15.X of TS 23.501 [2]).

It may be invoked during Registration procedure, during inter-PLMN mobility procedure, during PDU Session Establishment procedure or during UE Configuration Update procedure. When invoked during Registration procedure it may possibly trigger AMF re-allocation. When invoked during PDU Session Establishment procedure it may be invoked in the VPLMN or in the HPLMN; if invoked in the VPLMN it returns the hNRF selected by the hNSSF and, if applicable, the value of the HPLMN NSI ID. When invoked during UE Configuration Update procedure or inter-PLMN mobility procedure it may be invoked in the Serving PLMN.

NOTE 1:	The list of events, which trigger invoking of the Nnssf_NSSelection_Get service operation, is not exhaustive.

NOTE 2:	The NSSF can determine the serving network and Access Type from the TAI, as described in TS 29.571 [70].

Inputs, Required: None.

Inputs, Conditional Required:

If this service operation is invoked during Registration procedure for Network Slice selection or UE Configuration Update procedure, then the following inputs are required:

-	Subscribed S-NSSAI(s) with the indication if marked as default S-NSSAI, PLMN ID of the SUPI, TAI, NF type of the NF service consumer, Requester ID.

If this service operation is invoked to derive the S-NSSAI for the serving PLMN (as described in clause 4.11.1.3.3), the following inputs are required:

-	S-NSSAIs for the HPLMN associated with established PDN connection, PLMN ID of the SUPI, NF type of the NF service consumer, Requester ID.

If this service operation is invoked by target AMF during inter-PLMN mobility procedure, the following inputs are required:

-	S-NSSAIs for the HPLMN, PLMN ID of the SUPI, TAI.

If this service operation is invoked during PDN Connection Establishment in the Serving PLMN in EPS by a SMF+PGW-C, the following inputs are required:

-	Subscribed S-NSSAIs for the UE, PLMN ID of the SUPI, NF type of the NF service consumer, Requester ID.

If this service operation is invoked during PDU Session Establishment procedure in the Serving PLMN then the following inputs are required:

-	S-NSSAI, non-roaming/LBO roaming/HR roaming indication, PLMN ID of the SUPI, TAI, NF type of the NF service consumer, Requester ID.

If this service operation is invoked to determine an Alternative S-NSSAI for an S-NSSAI to be replaced, the following inputs are required:

-	S-NSSAI of the serving PLMN to be replaced, TAI, NF type of the NF service consumer, Requester ID.

Inputs, Optional:

If this service operation is invoked during Registration procedure for Network Slice selection or UE Configuration Update procedure, then the following inputs are provided if available:

-	Requested NSSAI, Mapping Of Requested NSSAI, Default Configured NSSAI Indication, NSSRG Information, UE support of subscription-based restrictions to simultaneous registration of network slice feature Indication, UDM indication to provide all subscribed S-NSSAIs for UEs not indicating support of subscription-based restrictions to simultaneous registration of network slices, Allowed NSSAI for current Access Type, Allowed NSSAI for the other Access Type and the corresponding Mapping Of Allowed NSSAIs for current Access Type and other Access Type, Rejected S-NSSAI(s) for RA, UE support of NSAG Information.

If this service operation is invoked during PDU Session Establishment procedure, then the following input is optional:

-	HPLMN S-NSSAI that maps to the S-NSSAI from the Allowed NSSAI of the Serving PLMN.

Outputs, Conditional Required:

If this service operation is invoked during Registration procedure for Network Slice selection or UE Configuration Update procedure, then one or more of the following outputs are required:

-	Allowed NSSAI, Configured NSSAI; Target AMF Set or, based on configuration, the list of candidate AMF(s).

If this service operation is invoked during inter-PLMN mobility procedure, then one or more of the following outputs are required:

-	Allowed NSSAI.

If this service operation is invoked to derive the S-NSSAI for the serving PLMN (as described in clause 4.11.1.3.3), the following output is required:

-	S-NSSAIs for the HPLMN associated with established PDN connection, Mapping of S-NSSAIs associated with established PDN connection in the Serving PLMN.

If this service operation is invoked during PDN Connection Establishment in the Serving PLMN in EPS by a SMF+PGW-C, the following outputs are required:

-	Subscribed S-NSSAIs for the UE, Mapping of S-NSSAIs associated with the subscribed S-NSSAIs for the UE in the Serving PLMN.

If this service operation is invoked during PDU Session Establishment procedure, then the following outputs are required:

-	The NRF to be used to select NFs/services within the selected Network Slice instance.

If this service operation is invoked to determine an Alternative S-NSSAI for an S-NSSAI to be replaced, the following outputs are required:

-	Alternative S-NSSAI mapping to the S-NSSAI to be replaced.

Outputs, conditional Optional:

If this service operation is invoked during UE Registration procedure or UE Configuration Update procedure, then one or more of the following outputs are optional:

-	Mapping Of Allowed NSSAI, Mapping Of Configured NSSAI, NSI ID(s) associated with the Network Slice instances of the Allowed NSSAI, NRF(s) to be used to select NFs/services within the selected Network Slice instance(s) and NRF to be used to determine the list of candidate AMF(s) from the AMF Set, rejected S-NSSAI with cause of rejection, Target NSSAI, the NSAG information (defined in clause 5.15.14 of TS 23.501 [2]), Alternative S-NSSAI mapping to an S-NSSAI to be replaced.

If this service operation is invoked during inter-PLMN mobility procedure, then the following output is optional:

-	Mapping Of Allowed NSSAI.

If this service operation is invoked during PDU Session Establishment procedure, then the following output is optional:

-	NSI ID associated with the S-NSSAI provided in the input.

## Next CHANGE

#### 5.2.16.3	Nnssf_NSSAIAvailability service

##### 5.2.16.3.1	General

Service description: This service enables to update the AMFs and the NSSF on the availability of S-NSSAIs and NSAGs on a per TA basis. It is also used to provide an Alternative S-NSSAI to NF Service Consumer in case when the NSSF determines that an S-NSSAI has to be replaced.

NOTE:	The NSSF can determine the serving network and Access Type from the TAI, as described in TS 29.571 [70].

## Next CHANGE

##### 5.2.16.3.4	Nnssf_NSSAIAvailability_Subscribe service operation

Service operation name: Nnssf_NSSAIAvailability_Subscribe

Description: This service operation enables a NF Service Consumer (e.g. AMF) to subscribe to a notification of any changes in status of the NSSAI availability information (e.g. S-NSSAIs and optionally NSI IDs available per TA and the restricted S-NSSAI(s) and optionally NSI IDs per PLMN in that TA in the serving PLMN of the UE) upon this is updated by another AMF. It also enables the NF Service Consumer (e.g. AMF, NSSF in the VPLMN) to subscribe to a notification when an Alternative S-NSSAI has to replace an S-NSSAI.

Inputs, Required: Callback URI of the NF Service Consumer.

Inputs, Conditional Required:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following inputs are required:

-	list of TAIs supported by the NF Service Consumer, event to be subscribed.

If this service operation is invoked to subscribe to a notification when any S-NSSAI has to replace by an Alternative S-NSSAI, then the following inputs are required:

-	for the S-NSSAI of the serving PLMN: set of S-NSSAIs that may be replaced and served by the NF Service Consumer, NF type of the NF Service Consumer (e.g. AMF or NSSF), Requester ID.

-	for HPLMN S-NSSAI: S-NSSAIs for the HPLMN, PLMN ID of the SUPI, NF type of the NF Service Consumer (e.g. AMF or NSSF), Requester ID.

Inputs, Optional: Expiry time.

Outputs, Required: SubscriptionID,

Outputs, Conditional Required:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following outputs are required:

-	Expiry time (if present in the request, may be included in the response based on operator's policy and taking into account the expiry time present in the request (i.e. should be less than or equal to that value); if not present in the request, may be included in the response based on operator's policy. Whatever the case, if not included in the response, this means that the subscription is valid without an expiry time).

If this service operation is invoked to subscribe to a notification when an Alternative S-NSSAI has to replace an S-NSSAI, then the following outputs are required:

-	for the S-NSSAI of the serving PLMN: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAI to be replaced.

-	for HPLMN S-NSSAI: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAIs of the HPLMN, PLMN ID.

Outputs, Optional: A list of TAIs and, for each TAI, the S-NSSAIs and optionally NSI IDs supported by the AMF and 5G-AN and authorized by the NSSF for the TAI and a list of S-NSSAIs and optionally NSI IDs restricted per PLMN for the TAI.

* * * END of CHANGES * * *