3GPP TSG-SA2 Meeting #155	S2-2302834

# Athens, Greece, February 20 - 24,  2023                       				(revision of S2-2300446)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3923 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for direct member selection assistance to AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2022-12-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion of AIMLsys KI#7, if this new member selection assistance functionality is not used, AF can select the members based on existing network exposure information as described in clause 4.15 of TS 23.502 [4]. The CR proposes to introduce a procedure that AF selects the members based on network exposure information collected directly from 5GC in case that member selection assistance functionality via NEF is not used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the detailed procedure that AF selects the members based on network exposure information collected directly from 5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for AF to select the members based on 5GC information when member selection assistance functionality is not used, will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.x(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** Start of Changes (All the text is new)***

4.15.X  AI/ML Member selection at the AF

Network exposure information as described in clause 4.15 of TS 23.502 [4], e.g., UE location reporting from the AMF, user plane information from the UPF and data analytics from NWDAF may be collected and used to assist the AF in application layer member selection e.g., assist in the selection of UE members participating in a federating learning operation.

This clause describes the procedure that how the AF leverages the 5GC network exposure for its member selection functionality to perform FL member selection.

Figure 4.15.X-1: Procedure for direct member selection assistance to AF

1.	[Optional] The AF requests the location reporting of the UEs from the AMF by invoking existing Namf_EventExposure_Subscribe (Location Reporting).

2.	[Optional] The AF initiates direct notification of QoS Monitoring procedure for delay information for the UEs in the candidate list, as defined in steps 1a-5 of clause 6.4.2.1 of TS 23.548 [10].

3.	[Optional] The AF requests user plane information, e.g. Throughput UL/DL, Packet transmission, Packet retransmission, for the UEs in the candidate list from UPF.

4.  [Optional] The AF requests analytics from NWDAF by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation, such as UE Communication, User Data Congestion Analytics, WLAN performance analytics per UE, etc, as defined in TS 23.288 [50].

NOTE X1:	In Steps 1-4, the AF maintains the candidate list itself and collects the member selection information directly from the 5GC, and includes the Target UE Identifier(s) for the UEs in the candidate list in the request.

5.	The AF selects FL members based on the information collected in Steps 1-4.

NOTE X2:	What information needs to be collected by the AF to perform member selection is decided by the AF itself.

NOTE X3:	AF may keep monitoring the information from AMF/UPF/SMF/NWDAF to update the members.

# *** End of Changes ***