3GPP TSG-WG SA2 Meeting #155	S2-2302867

Athens, Greece, 20 - 24 February, 2023	(revision of S2-23001987)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0625 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 Clarification on PFD Determination Analytics in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Mobile, KDDI, Samsung, SK Telecom,] Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This proposal is to remove the EN since the UPEAS specifies UPF event exposure for NWDAF data collection, specifically in clause 4.15 of TS 23.502. In general part, area of Interest is removed since it does not contain Input data table. Also, this proposal is to make clarification on PFD Determination Analytics in TS 23.288 or to make editorial changes to make the logic clear. E.g., only In case that PFD information for the existing Application ID is new or to be updated, the NWDAF then needs to notify analytics to the consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This proposal is to make clarification on PFD Determination Analytics in clauses 6.16.1, 6.16.3, 6.16.4 and adding the PFD Determination Analytics in clause 7.1 in TS 23.288. Revision of S2-23001987: - Revoke the change of the optional application ID list in the Analytics Filter Information, keeping it as mandatory in the request/subscription for the known application identifier(s). - Update NEF(PFDF) in PFD Information description in Table 6.16.2-1. - Update descriptions for Flow descriptor to be aligned with the flow description in PfdContent data type in TS 29.551. - Update procedure figure and descriptions for NWDAF-assisted PFD Determination. - Update Response Description for PFD Determination in Table 7.1-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear for implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.16.1, 6.16.3, 6.16.4, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

## 6.16	PFD Determination Analytics

### 6.16.1	General

This clause specifies the procedure on how NWDAF can provide NWDAF-assisted PFD Determination analytics, in the form of statistics.

To assist determination of PFDs for known application identifiers, if the related Service Level Agreement does not preclude, an NWDAF may perform data analytics on existing PFD information and user plane traffic and provide analytics results in the form of new or updated PFDs , e.g., IP 3-tuple list in PFD is new or updated, to an analytics consumer in the 5GC. The NEF(PFDF) as the consumer may forward new or updated PFD information provided by the NWDAF to the UPF via the SMF to detect a known application as defined in TS 23.502 [3].

The service consumer may be a NEF(PFDF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = " PFD Determination";

-	Target of Analytics Reporting: "any UE";

-	The list of know Application Identifier(s);

and may include Analytics Filter Information optionally containing:

-	S-NSSAI;

-	DNN;

-	An Analytics target period indicates the time period over which the statistics are requested;

-	Optionally, maximum number of objects. This refers to the analytic output (i.e. maximum number of new or updated PFDs);

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

### 6.16.2	Input Data

The NWDAF collects information on user data traffic from NF(s) for a specific S-NSSAI, DNN, Application ID(s), and retrieves the existing PFDs from the NEF(PFDF). The detailed input data are described in Table 6.16.2-1.

Table 6.16.2-1: Input data to detect known application from NFs

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF, UPF | UE ID for the UE that uses the application |
| S-NSSAI | SMF, UPF | Identifies the Network Slice for which analytics information is provided. |
| DNN | SMF, UPF | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Application ID | UPF | Identifies the application |
| IP Flow Information (1..max) | UPF | Per IP flow related information for the application |
| > IP 5-tuple | UPF | Identifies a service flow of the UE that uses the application. |
| > Start time | UPF | Start time of traffic detection for the flow. |
| > End time | UPF | End time of traffic detection for the flow. |
| > UL Data volume | UPF | Measured UL data traffic volume for the flow. |
| > DL Data volume | UPF | Measured DL data traffic volume for the flow. |
| > UL Data Rate | UPF | Measured UL data rate for the flow. |
| > DL Data Rate | UPF | Measured DL data rate for the flow. |
| > URL list | UPF | List of URLs extracted from the inspected user plane packets in the flow. |
| > Domain Name list | UPF | List of domain names extracted from the inspected user plane packets in the flow. |
| PFD Information | NEF(PFDF) | PFD Information stored in the UDR (as Application Data) and retrieved by NEF(PFDF), as defined in clause 6.1.2.3.2 of TS 23.503 [4]. |
| > Application ID | NEF(PFDF) | Identification of the application that refers to one or more application detection filters. |
| > IP 3-tuple list | NEF(PFDF) | Including protocol, server side IP address and port number. |
| > URL list | NEF(PFDF) | The significant parts of the URL to be matched, e.g. host name. |
| > Domain Name list | NEF(PFDF) | A Domain Name matching criteria and information about applicable protocol(s). |

NOTE:	Extensive reporting of all traffic flows may conflict with the requirement to avoid extra UPF load. An NWDAF may subscribe only for reporting for some UEs to limit the load.

### 6.16.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.16.3-1. The output analysis can be used to provision new or updated PFDs information for known applications.

Table 6.16.3-1: PFD Determination statistics

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) stored in the UDR. |
| List of application traffic description |  |
| > S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| > DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| > Flow descriptor | Flow descriptor containing 3-tuple (server IP address, server port, and protocol of uplink/downlink application traffic). |
| > URL list | the significant parts of the URL to be matched, e.g. host name. |
| > Domain name information | A Domain name matching criteria and information about applicable protocol(s). |

### 6.16.4	Procedures

The Figure 6.16.4-1 shows the procedure that a consumer can use to request PFD Determination analytics.

Figure 6.16.4-1: A procedure for NWDAF-assisted PFD Determination

1.	The Consumer NF (NEF(PFDF)) requests or subscribes to the NWDAF to request PFD Determination analytics for a known application identifier or a list of known application identifier(s). The Target of Analytics Reporting is set to Any UE.

The Analytics Filter Information may optionally include the S-NSSAI and/or DNN.

2.	The NWDAF fetches currently stored PFD information in use from UDR via NEF(PFDF) for the Application ID(s) provided in step 1, as listed in Table 6.16.2-1.

3.	The NWDAF collects information from the UPF (event “UserDataUsageMeasures”) as listed in Table 6.16.2-1 either via SMF (Option 1) or directly from UPF (Option 2) as defined in TS 23.501 [2] clause 5.8.2.17. NWDAF discovers the SMF (Option 1) or UPF (Option 2) through NRF as defined in TS 23.502 [3] clause 4.15.4.x.3. The event “UserDataUsageMeasures” is defined in TS 23.502 [3] clause 5.2.26.2.

4. 	UPF reports the data directly to NWDAF.

5.	The NWDAF derives PFD Determination analytics, e.g., new or updated PFD information for the existing Application ID.

6.	In case that PFD information for the existing Application ID is new or to be updated, the NWDAF notifies PFD Determination analytics to the consumer NF (i. e., NEF(PFDF)).

* * * Next Change * * *

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |

* * *End of Changes * * *