3GPP TSG-WG SA2 Meeting #155 	S2-2302953

Athens, Greece, 20-24 February 2023

# Title:	[Draft] LS on MBS Session status synchronisation

# Response to:	-

# Release:	Release XX

# Work Item:	5MBS_ph2

Source:	SA2

To:	RAN2, CT1

Cc:	-

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	harisz AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR 0196 to TS 23.247

1. Overall Description:

SA2 discussed the possibility for the UE that supports receiving MBS while in RRC_INACTIVE state to locally leave in order to avoid frequent transitions from RRC_INACTIVE to RRC_CONNECTED in order to send NAS signalling to SMF. In order to achieve synchronisation between the UE and the CN the UE is expected to include an "MBS Session Status IE" in every initial NAS message in similar fashion like the UE currently sends the "PDU Session Status IE" to AMF. Details are described in attached agreed CR. Considering this decision there will be some period of time that the number of UEs that will be considered "joined" for a specific MBS Session in SMF and NG-RAN will be incorrect. If this is considered an issue for RAN2 or CT, this period of time can either be controlled and deterministic e.g. by a mechanisms to be defined by CT1 where AMF is assigning a timer for the UE to send Registration Request or an Access Stratum signalling mechanism to keep the counting of UEs up to date could be defined by RAN2.

2. Actions:

To XXX:

ACTION: 	SA2 kindly ask RAN2 and CT1 to take above information into account and provide feedback if needed.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #156	17-21 April 2023		e-meeting

TSG-SA2 Meeting #157	22-26 May 2023		EU