3GPP TSG-WG SA2 Meeting #155	S2-2302979

Athens, Greece, February 20 – 24, 2023

Source:	Interdigital

Title:	Proposed texts for high level description on service exposure

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL/Rel-18

Abstract of the contribution: This contribution proposes texts for high level on service exposure of TS 23.586v0.1.0.

# 1.	Discussion

Based on TR conclusion, there shall be a high level description on service exposure.

# 2.	Text proposal

It is proposed to agree the following changes to TS 23.586:

# * * * * First change * * * *

## 5.X	Service exposure

## 5.X.Y	Service exposure to 5GC NF and Application Server

For service exposure of Ranging and SL Positioning to 5GC and to Application server, service exposure in subclause 5.5 in TS23. 273 is reused with following updates on relative positioning:

-	Type of Event is extended to include relative positioning between two UEs.

-	For relative positioning, two UE’s identities are included as attribute in the location service requests.

-	For relative positioning, accuracy and latency for distance and/or direction measurements are included in Requested LCS Quality of Service information.

When absolute positioning is requested, LMF and target UE may decide whether SL positioning or Uu positioning needs to be performed.

# * * * * End of change * * * *