3GPP TSG-SA WG2 Meeting # 155	S2-2303062

Athens, Greece,  Feb 20 – 24, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.167 | CR | 0370 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Emergency call domain selection after 2G/3G radio switch off, and/or UE in limited service state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA 2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023 02 10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In S2-2302198 (=NRG016_207) GSMA meeting NRG#16 has requested SA2 to update TS 23.167 to include a description of domain selection for emergency calls when the UE is in limited service state. They also request an update related to the UE being PS attached but VoLTE roaming being unavalable in the VPLMN. However, the author believes that this case is already covered by the existing standards. While there is no specific request from GSMA, their LS is related to the removal of both 2G and 3G coverage in the VPLMN. However, removal of the 2G/3G radio coverage does not automatically mean that “SMS over NAS” will be disabled in the VPLMN’s MME. From a UE perspective support of “SMS over SGs” or “SMS in MME” will result in the UE being “CS attached”, and hence the domain selection tables in annex H might lead to undesirable UE behaviour when there is no 2G/3G coverage. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarified that 4G combined attach with “SMS only” result does not mean that CS voice access is available. - Described actions in limited service state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary (and possibly large) delays in emergency call establishment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.5, H.6 (revision zero of this CR includes unmodified clauses of annex H in order to assist readers in detecting possible errors in this CR) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | UEs of earlier releases may implement this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.228: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS); Stage 2".

[2]	3GPP TS 23.060: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; General Packet Radio Service (GPRS); Service description; Stage 2".

/….. text not modified …./

[41]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[42]	3GPP TS 26.267: "Digital cellular telecommunication systems (Phase 2+); Universal Mobile Telecommunication System (UMTS); eCall data transfer; In-band modem solution General description".

[43]	3GPP TS 29.328: "IP Multimedia (IM) Subsystem Sh interface; Signalling flows and message contents".

[44]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[45]	ATIS-0700028: "Location Accuracy Improvements for Emergency Calls".

[46]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services".

[47]	IEEE Std 802.11-2012: "IEEE Standard for Information technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[48]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[49]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[50]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[51]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[52]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[xx]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[yy]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

**********  second change ************

Annex H (normative):
IMS emergency services using UTRAN, E-UTRAN and NG-RAN radio access network

# H.1	General

This annex includes additional requirements and clarifications when the IP-CAN is a PS Domain supporting UTRAN, E-UTRAN or NG-RAN radio access network.

NOTE:	For SNPN, as defined in TS 23.501 [48] the only supported RAT is NR and there is no support for Emergency Services Fallback.

If a PLMN supports other emergency numbers than those listed in TS 22.101 [8], the UE is connected to the PLMN using UTRAN, E-UTRAN or NG-RAN radio access network and the UE needs to know these other emergency numbers, then such emergency numbers shall be provided to the UE via the mobility management procedures as described in TS 24.301 [33], TS 24.008 [13] and TS 24.501 [52].

If an SNPN supports other emergency numbers than those listed in TS 22.101 [8], the UE is connected to the SNPN using NR access and the UE needs to know these other emergency numbers, then such emergency numbers shall be provided to the UE via the mobility management procedures as described in TS 24.501 [52].

For registration requests received from an emergency PDN connection or emergency PDU session, the P-CSCF shall reject any IMS registration which is not for the emergency purpose.

eCall is supported with E-UTRAN and NG-RAN for PLMNs only, i.e. eCall is not supported for SNPNs. UE configured for eCall Only Mode can receive a PSAP call back for a limited duration following the termination of the eCall due to its specific mobility management procedures requirement as defined in TS 22.101 [8].

# H.2	UE specific behaviour

For the specific case where the UE has selected to make an emergency call over EPS, GPRS or 5GS the UE shall use the following procedures:

-	A UE shall establish an emergency PDN connection or emergency PDU session and perform an IMS emergency registration before initiating a UE detectable emergency session via UTRAN, E-UTRAN or NG-RAN.

-	A UE shall not establish an emergency PDN connection or emergency PDU session if the UE initiated a non UE detectable emergency session and is subsequently informed by the network that the ongoing session is an emergency session.

NOTE 1:	If SRVCC is required in the network, an operator could download the local emergency numbers to avoid non UE detectable emergency sessions since UTRAN and E-UTRAN will not be able to identify emergency SRVCC handling without an emergency PDN connection.

-	If the UE initiates a non UE detectable emergency session, and the session initiation request is rejected by the P-CSCF with an indication that this is an emergency session, the UE shall select a domain according to the requirements for domain priority and selection in clause H.5 when applying the requirements in clause 7.1.2.

-	A UE in 5GS shall not initiate establishment of an IMS emergency session over Non-3GPP access unless the emergency session could not be established via 3GPP access.

-	If the UE has not been authenticated in the PS domain, the UE shall initiate an IMS emergency session establishment without registration according to clause 7.4.

-	The UE shall include the latest available Cell Global Identification (CGI) in the IMS emergency request establishing the emergency call.

-	When the UE is using NR satellite access to 5GC, the UE shall include an indication of NR satellite access in the IMS request.

NOTE 2:	When using UTRAN, the UE is not always able to read the current cell identity and in some cases the UE can be connected to several cells simultaneously.

NOTE 3:	For NR satellite access to 5GC the NR CGI (NCGI) available to the UE and the NCGI provided from NG-RAN to 5GC might be different. The NCGI provided by the UE might not relate to the UE's geographic location. A P-CSCF relying only on the UE provided NCGI could then lead to errors.

-	If the UE is required to include an equipment identifier (according to clauses 4.1 and 6.1) the equipment identifier shall be the IMEI.

-	For the media supported during IMS emergency sessions in clause 10.4 of TS 22.101 [8], media codec and format support is specified in TS 26.114 [34].

-	UE shall only perform eCall procedure over IMS as specified in clause 7.7.1 when it detects the "eCall supported" indication as defined in TS 23.401 [28] and TS 23.501 [48].

-	When PS access is available, but the UE does not detect the "eCall supported" indication as defined in TS 23.401 [28] and TS 23.501 [48], and there is no CS access available, the UE shall establish a regular IMS emergency call.

-	If the broadcast indicators in an E-UTRA cell connected to 5G and EPC as defined in TS 23.501 [48] indicate only one core network (EPC or 5GC) supports emergency services then the UE shall register to the core network which supports emergency services when the UE initiates emergency services. If broadcast indicators indicate that both 5GC and EPC support emergency services then the UE initiates emergency services to either EPC or 5GC according to the UE implementation.

-	If the broadcast indicator in an NR cell indicates that the cell provides access to SNPNs and that 5GC supports emergency services, the UE shall only register to the core network of the SNPN that indicates support for emergency services. If broadcast indicators indicate that one or more SNPNs and/or PLMNs support emergency services, then the UE initiates emergency services to either a SNPN or a PLMN according to UE implementation.

# H.3	High Level Procedures for IMS emergency calls

For the high level procedures (as described in clause 7.1.1) the following statements apply for UE detected emergency calls when PS domain with UTRAN, E-UTRAN or NG-RAN access is used:

-	the bearer registration to the IP-CAN is the PS attach or PS Registration procedure;

-	the IP-CAN bearer resource is the PDP context for GPRS, EPS Bearer for EPS and QoS Flow for 5GS, and TS 23.060 [2], TS 23.401 [28] and TS 23.501 [48] respectively describe how to indicate that the request is for emergency;

-	the release of reserved bearer resources is the release of a PDP context, an EPS Bearer or a QoS Flow.

For the high level procedures (as described in clause 7.1.2) the following statements apply for Non UE detectable emergency calls when PS domain with UTRAN, E-UTRAN or NG-RAN access is used:

-	The P-CSCF rejects the session initiation request with an indication that this is an emergency session; or

-	the P-CSCF allows the session initiation request to continue and the E-CSCF informs the UE that it is an emergency session. Such a session will not use an emergency PDN connection or emergency PDU session and will not have SRVCC or SRVCC for IMS emergency session support.

For the IMS Emergency Session Establishment without Registration (as described in clause 7.4), P-CSCF retrieves the additional subscriber related identifier(s) from IP-CAN via PCC in TS 23.203 [20] when PS domain with UTRAN and E-UTRAN access is used.

# H.4	Location handling

The applicable location solutions are specified in TS 23.271 [5] for control plane with UTRAN and E-UTRAN access, in TS 23.501 [48] and TS 23.502 [49] for control plane with NG-RAN access and in SUPL 2.0 (OMA AD SUPL[15], OMA TS ULP [16]) for user plane. For SUPL, SUPL Initiation Function using UDP/IP (OMA AD SUPL [15]) shall be supported by the SET.

If the operator policy requires network provided location using PCC-based solutions for the UE location (e.g. serving cell), the P-CSCF retrieves location information from the access network and includes it in the emergency service request, as described in TS 23.228 [1]. The UE location is marked as being network provided to distinguish it from location information that the UE provided.

If the operator policy requires network provided location using the LRF-based solution for the UE location (e.g. serving cell), the P-CSCF forwards the emergency request to E-CSCF. The E-CSCF retrieves location information from LRF as described in clause 6.2.2.

Operator policies in the P-CSCF and E-CSCF should be aligned to ensure that the E-CSCF doesn't attempt to retrieve location information from the LRF if network-provided location information is present in the initial emergency request. However, as described in clause 6.2.2, the E-CSCF may request the LRF to retrieve additional location information (e.g. the UE's geographical location information).

# H.5	Domain Priority and Selection Rules for Emergency Session Attempts

This clause details the domain priority and selection (see clause 7.3) for UE that attempts to make an emergency session for UTRAN, E-UTRAN or NG-RAN radio access networks based on the UE attach status to CS or PS domains and the network support for IMS emergency and IMS voice over PS.

The following table (Table H.1) defines these rules based on the UE (last 2 columns) for different initial conditions (first 4 columns) when an emergency session is initiated and when the UE is not in limited service state.

If the UE is in limited service state and camped on an E-UTRAN or NG-RAN cell, then the ims-EmergencySupport

broadcast bit (TS 36.331 [xx]; TS 38.331 [yy]) defines whether emergency access is allowed on that cell. If the ims-EmergencySupport indicates that emergency access is not allowed on that E-UTRAN/NG-RAN cell, and the UE needs to initiate an emergency call, then UE-implementation specific mechanisms should be used to attempt to find an acceptable cell.

NOTE: 	Clause 3.5 of TS 23.122 [41] indicates that when no suitable cell can be found, the UE searches for an acceptable cell. An acceptable cell is defined in TS 23.122 [41] as a cell that the UE can camp on to make emergency calls, so, a cell that has the ims-EmergencySupport bit set to ‘not allowed’ would not be an acceptable cell. In limited service state, the objective of the UE is to find an acceptable cell or a suitable cell. Hence if a UE is camped on a cell with the ims-EmergencySupport bit indicating that emergency access is not allowed, then it implies that the UE is not in the coverage of any acceptable cell.

For NG-eCall (eCall over IMS) domain selection in clause H.6 applies. This clause is not applicable for NG-eCall.

Table H.1: Domain Selection Rules for emergency session attempts for UTRAN, E-UTRAN or NG-RAN radio access networks

|  | CS Attached (NOTE 9) | PS Attached | VoIMS | EMS | First EMC Attempt | Second EMC Attempt |
| --- | --- | --- | --- | --- | --- | --- |
| A | N | Y | Y | Y | PS | CS if available and supported (NOTE 7, NOTE 8) |
| B | N | Y | N | Y | PS or CS if the emergency session includes at least voice. PS if the emergency session contains only media other than voice. | PS if first attempt in CS CS if first attempt in PS (NOTE 7, NOTE 8) |
| C | N | Y | Y or N | N | PS if ESFB is "Y" (NOTE 5). Else CS or PS for another 3GPP RAT with EMS or ESFB set to "Y" if available and supported and if the emergency session includes at least voice. Else PS for another 3GPP RAT with EMS or ESFB set to "Y" if available and supported if the emergency session contains only media other than voice. | PS if first attempt in CS CS if first attempt in PS (NOTE 7, NOTE 8) |
| D | Y | N | Y or N | Y or N | CS if the emergency session includes at least voice. PS if available and EMS or ESFB is "Y" and emergency session contains only media other than voice. | PS if available and EMS or ESFB is "Y" |
| E | Y | Y | Y | Y | If the emergency session includes at least voice, follow rules in TS 22.101 [8] which say to use the same domain as for a non-EMC (NOTE 2) PS if the emergency session contains only media other than voice. | PS if first attempt in CS CS if first attempt in PS |
| F | Y | Y | Y or N | N | PS if ESFB is "Y" (NOTE 5). Else PS for another 3GPP RAT with EMS if available and supported or CS, if the emergency session includes at least voice. | CS if first attempt in PS PS for another 3GPP RAT if available and supported and EMS or ESFB is "Y" if first attempt in CS. |
| G | Y | Y | N | Y | CS if the emergency session includes at least voice. PS if the emergency session contains only media other than voice. | PS |
| EMC = Emergency Session. EMC includes also normal calls initiated in the CS domain that are treated by the CS CN as emergency calls. VoIMS = Voice over IMS over PS sessions support as indicated by IMS Voice over PS session supported indication as defined in TS 23.401 [28], TS 23.060 [2] and TS 23.502 [49]. EMS = IMS Emergency Services supported as indicated by Emergency Service Support indicator as defined in TS 23.401 [28], TS 23.060 [2], TS 23.501 [48] and TS 23.502 [49]. ESFB = Emergency Services Fallback for 5GS as defined in TS 23.501 [48] and TS 23.502 [49]. NOTE 1: If the UE selects the CS domain and initiates a normal call using the dialled local emergency number (see clause 7.1.2), and the UE enters limited service state (e.g. due to a Location Registration failing), then the UE camps on an acceptable cell (see TS 23.122 [41]) and may proceed with the EMC by initiating an emergency call in limited service state. NOTE 2: Use of the same domain as for a non-EMC is restricted to UTRAN, E-UTRAN and NG-RAN access (e.g. excludes WLAN). NOTE 3: This NOTE applies to a UE in dual registration mode as defined in TS 23.501 [48]. A dual registration mode UE that is registered to both EPC and 5GC assumes attachment, for the purpose of the "PS Attached" column, to whichever of EPC or 5GC indicates EMS as "Y". When both EPC and 5GC indicate EMS as "Y", the UE shall assume attachment to either EPC or 5GC based on implementation. A UE that is registered to both EPC and 5GC does not use emergency services fallback and ignores the ESFB condition when performing domain selection. NOTE 4: The other 3GPP RAT for row C and row F can be any of UTRA, E-UTRAN connected to EPC, E-UTRA connected to 5GC or NR connected to 5GC that is supported by the UE and differs from the RAT to which the UE is currently attached in the PS domain (or is assumed to be attached based on NOTE 3). NOTE 5: The condition 'ESFB is "Y"' only applies for a UE that is camped on or connected to 5GS via NR or via E-UTRA and that supports Emergency Services Fallback. In that case the emergency call will be provided over E-UTRAN or E-UTRA connected to 5GC as defined in procedures in TS 23.502 [49]. The condition 'ESFB is "Y"' is taken into consideration by the UE only when the network has indicated EMS = "N" for the RAT on which the UE is camping or connected. NOTE 6: For 5GS, the value of the column "EMS" is for the RAT that UE is camped on or is connected to. NOTE 7: As an implementation option, when the first attempt uses PS and fails for reasons other than related to IMS, the second attempt may use PS with a different 3GPP RAT. In this case the UE can make a third attempt using CS. NOTE 8: As an implementation option, when the first attempt uses PS and fails because the first discovered P-CSCF is not available, additional attempts may use PS with alternate P-CSCFs (see clause E.1.1.1 of TS 23.228 [1] and clause 4.3.2.2.1 of TS 23.502 [49]). In this case the UE can make an attempt using CS after making failed PS attempts on all discovered alternate P-CSCFs. NOTE 9: With E-UTRAN access, “CS attached” means that the UE has performed a combined Attach/TAU and has been accepted by the network for EPS and non-EPS services. If the UE has been accepted for EPS and “SMS-only” then, and only for the purposes of this table, the UE should assume that it is not CS attached. |  |  |  |  |  |  |

# H.6	eCall over IMS

This clause details the domain priority and selection (see clause 7.3) for a UE that attempts to make an eCall over IMS session using E-UTRAN or NG-RAN radio access networks based on the availability of the CS or PS domains and the network support for IMS emergency, eCall over IMS and IMS voice over PS.

The following table (Table H.2) defines these rules based on the UE (last 2 columns) for different initial conditions (first 4 columns) when an eCall over IMS session is initiated.

Table H.2: Domain Selection Rules for eCall over IMS session attempts for E-UTRAN or NG-RAN radio access networks

|  | PS Available | VoIMS | EMS | ECL | First eCall Attempt | Second eCall Attempt |
| --- | --- | --- | --- | --- | --- | --- |
| A | Y | Y | Y | Y | PS | PS on another PS RAT if available with EMS=Y and ECL=Y or CS if available |
| B | Y | Y | Y | N | CS if available | PS (UE establishes IMS emergency session) |
| C | Y | Y or N | N | N | CS if available | PS on another PS RAT if available with EMS=Y or EMS unknown |
| D | Y | N | Y | Y | PS or CS if available | CS if first attempt in PS PS if first attempt in CS |
| E | Y | N | Y | N | CS if available | PS (UE establishes IMS emergency session) |
| F | N |  | - | - | CS if available |  |
| VoIMS = Voice over IMS over PS sessions support as indicated by IMS Voice over PS session supported indication as defined in TS 23.401 [28] and TS 23.502 [49]. A UE in limited service state shall assume that Voice over IMS over PS sessions is not supported. EMS = IMS Emergency Services supported as indicated by Emergency Service Support indicator as defined in TS 23.401 [28] and TS 23.501 [48] and TS 23.502 [49]. A UE in limited service state shall use the the ims-EmergencySupport broadcast bit (TS 36.331 [xx]; TS 38.331 [yy]) instead of the Emergency Service Support indicator. ECL = eCall Over IMS support as indicated by the eCall support indicator defined in TS 23.401 [28] and TS 23.501 [48]. NOTE 1: As an implementation option, when the first attempt uses PS and fails for reasons other than related to IMS, the second attempt may use PS with a different 3GPP RAT. In this case the UE, can make a third attempt using CS. |  |  |  |  |  |  |

NOTE:	If the E-UTRAN and NG-RAN cells available to the UE have different settings, the UE assumes "PS Available" and "ECL" apply to whichever cell is indicated (as defined in TS 23.401 [28] and TS 23.501 [48]) as providing eCall over IMS support. When support by more than one cell is indicated, a UE may select any cell to attempt eCall over IMS according to the UE implementation.

********** end of changes ************