SA WG2 Meeting #155	S2- 2303073

Athens, Greece, February 20th-24th, 2023

Source:	Meta USA

Title:	Discussion of media codec information from the AF for KI#9.

Document for:	Agreement

Agenda Item:	9.12.2

Work Item / Release:	XRM / Rel-18

Abstract of the contribution: This paper proposes the for xRM, no additional media codec information is required from AF (for KI#9).

1.	 Discussion

In the name of KI#9 –“Trade-off of QoE and Power Saving Requirements”, the following principle was concluded to guide the normative work.

- The PCF get the media codec information from the AF, and generate or update the PCC rules considering the application information (i.e. media codec information).

It means that during the normative phase, how this media codec info (if provided from AF and in what form or shape), will need to be understood before it can be formalised into normative specifications.

When looking into the history (TS 23.203 and 503) on how media codec info could be used by the PCC, the following is found:

Ref: 23.203: The AF, if involved, may provide the following application session related information, e.g. based on SIP and SDP: …. - Media Format, e.g. media format sub-field of the media announcement and all other parameter information (a= lines) associated with the media format; … At reception of the IMS service information from the P-CSCF, if configured through policy, the PCRF determines the Maximum Packet Loss Rate for UL and DL based on the IMS service information and taking into account information defined in TS 26.114 [45] and sends it to PCEF along with the PCC rule for the voice media. NOTE 12: Based on local configuration, the PCRF sets the Maximum Packet Loss Rate (UL, DL) corresponding to either the most robust codec configuration (e.g., codec, mode, redundancy) or the least robust codec configuration of the negotiated set in each direction.

Existing texts indicated that the codec information (a=lines) is meant for PCC to set the Maximum Packet Loss Rate (UL, DL) corresponding to either the most robust codec configuration or…from the negotiated codec between two endpoints (e.g, P-CSCF and UE). In other words, it is not meant for power saving trade-off.

For xRM, PCF is aware of the AF’s requirement for bandwidth/packet delay/error (e.g, PSDB, PSER) as these can be signalled directly via NEF API. Therefore, adding media codec information (in any shape or form) from the AF requires more details during the normative phase before this can be proceeded to stage 3.

2.	Proposal

With the absence of detailed information on how power saving is being traded-off with media codec information (in whatever shape or form) from the AF, it is proposed not to enhance 5GS in R18 for this purpose. The existing “media format” description, as currently defined in TS 23.203/503, can be left as it is.