3GPP TSG-WG SA2 Meeting #155 	S2-2303094

Athens, Greece, Feburary 20th – 24th, 2023

Source:	Intel

Title:	TS 23.586: UE Discovery & Selection

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper proposes UE Discovery & Selection for Ranging based services and sidelink positioning.

# 1. Introduction/Discussion

This paper proposes UE Discovery & Selection for Ranging based services and sidelink positioning.

# 2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

## 5.X	Ranging/Sidelink Positioning UE Discovery & Selection

### 5.X.1	General

Ranging/Sidelink Positioning UE Discovery is defined as the process that detects and identifies another UE for Ranging/Sidelink Positioning service via PC5 reference point.

Both Model A and Model B discovery as defined in TS 23.304 [4] are supported for the 5G ProSe capable UEs discovery (including commercial and public safety use cases).

Procedures for V2X communication over PC5 reference point as defined in TS 23.287 [3] are supported for the V2X capable UEs discovery.

Editor's note: Whether and how to support Restricted ProSe Discovery defined in TS 23.304 [4] for the 5G ProSe capable UE is FFS.

### 5.X.2	Located UE Discovery & Selection

When the network assisted SL positioning is applied, the Target UE (or triggered by LMF) may perform Ranging/Sidelink Positioning UE Discovery as specified in 5.X.1 to discover Located UE and report discovered Located UE identity to the LMF. To assist the Target UE the LMF may provide candidate list of Located UE to participate in the network assisted SL positioning for the target UE.

### 5.X.3	SL Positioning Server UE Discovery & Selection

Ranging/Sidelink Positioning UE Discovery as specified in 5.X.1 may be used for SL Positioning Server UE Discovery by including SL Positioning Server UE Discovery indication in discovery message.

# * * * * End of changes * * * *