3GPP TSG-WG SA2 Meeting #155 	S2-2303151

Athens, Greece, February 20 – 24, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Procedure for Mobile Terminated (MT) Ranging and Sidelink Positioning Service

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to add the Procedure for Mobile Terminated (MT) Ranging and Sidelink Positioning Service, based on the KI#5 conclusions in TR 23.700-86.

# 1. Introduction/Discussion

This paper proposes to add the procedure for the Mobile Terminated (MT) Ranging and Sidelink Positioning Service requested by an Application Funciton (5GC-MT scenario), as well as the the required functionality and features of the LMF and the AMF.

The conclusions of "Key Issue #5: Network assisted SL Positioning" (Clause 8.5) in TR 23.700-86 are taken into consideration:

- Legacy functionality specified in TS 23.273 [11] for location services shall be reused including MO-LR, MT-LR for both regulatory and commercial use, when the Target UE has NAS connection with AMF (directly or indirectly via a ProSe UE-to-Network Relay).

  - Either the Target UE or LMF determines if network assisted SL positioning will be applied. When LMF determines that network assisted SL positioning is used, LMF may trigger the Target UE to perform the discovery of Located UE(s).

  - The Location Service is triggered via the AMF that serves the Target UE, the request comes either from an application server via the GMLC, a 5G NF, or the Target UE.

  - Target UE discovers Located UE(s) for network assisted SL positioning. To assist the Target UE the LMF may provide candidate list of Located UE to participate in the network assisted SL positioning for the target UE. The provisioning of candidate list of Located UE assumes that the capability and location information of the Located UE in the list are maintained by 5GC.

Also, relevant conclusions for the above-mentioned procedure of “Key Issue #7: Ranging/Sidelink Positioning service exposure to Application server and for network assisted sidelink positioning” (clause 8.7) in TR 23.700-86 are taken into consideration:

- The 5GS Ranging/Sidelink Positioning service may be exposed to the Application server either via interaction with 5GC NFs e.g. GMLC or via the NEF.

- For Ranging/Sidelink Positioning service exposure to Application server, the LCS architecture includeing and MT-LR and MO-LR procedures are taken as baseline:

  - For MT-LR procedure, the GMLC selects the AMF.

  - LMF or UE performs result calculation.

  - If LMF determines to use Network Assisted SL Positioning to position a Target UE, Ranging/Sidelink Positioning results and Uu positioning results may be combined by LMF before exposing the results to the AF.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586 including 5.5 and 6.5.

# * * * * First change (new)* * * *

## 5.5	Network assisted Ranging/Sidelink Positioning

The features and functionalities of the LMF to enable Ranging/SL positioning service are as follows:

- determines whether to use sidelink positioning to position the target UE and/or provide ranging information.

- determines whether to send the request for the sidelink positioning and ranging to the target UE or to the located UE. The located UE may be a Relay UE e.g., in case the target UE is out of coverage.

- may trigger the Target UE to perform the discovery of Located UE(s).

- may provide candidate list of Located UE to participate in the SL positioning for the target UE.

- may use additional methods (e.g., Uu-based location services) together with the results received by the target UE or located UE to improve the quality of the positioning and/or ranging outputs. LMF processes received measurement data by the target UE or the located UE, if available, to determine position and ranging.

- receives by the target UE or the located UE results of sidelink positioning and/or ranging, while relative measurement data can be also received.

The AMF can select the appropriate LMF to serve a sidelink positioning and/or ranging based on the LMF Ranging and/or sidelink positioning capabilities.

# * * * * Second change (new)* * * *

### 6.5	Procedure for network assisted Ranging/Sidelink Positioning

Figure 6.5-1 illustrates the general procedure for ranging and sidelink positioning requested by the AF. In this scenario, it is assumed that at least one of the target UE and the Located UE(s) are in the network coverage and may be identified using a SUPI or GPSI. This procedure is applicable to a request from an AF (or LCS client) for ranging and current location of the target UE.

Figure 6.5-1 5GC-MT Ranging and Sidelink Positioning Request

- The AF (via NEF) sends a request to the GMLC for location information and/or ranging information of a target UE and also one or more other UEs (i.e. for which ranging is needed, named as Located UEs) that may be identified by an GPSI or an SUPI. The NEF authorizes the AF for the usage of the Ranging and LCS service. If the authorization fails, steps 2-19 are skipped, and NEF responds to the AF the failure of the service authorization in step 20.

2-3. The GMLC invokes a Nudm_SDM_Get service operation towards the UDM of the target UE to get the privacy settings of the UE identified by its GPSI or SUPI. The UDM returns the target UE Privacy setting of the UE. The GMLC checks the UE LCS privacy profile. If the target UE is not allowed to be located, steps 4-19 are skipped.

4.	GMLC invokes the Namf_Ranging_ProvidePositioningInfo service operation towards the AMF to request the ranging and location of the UE. The service operation includes the SUPI, the client type, scheduled location time, service type and other attributes as received or determined in step 1.

5.	If the target UE is in CM IDLE state, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.3 of TS 23.502 to establish a signalling connection with the UE and the same applies for the located UEs, in case the LMF should interact with them (in case Option B is enabled below).

6.	In case the indicator of privacy check related action indicates that the target and or located UE must either be notified or notified with privacy verification then the procedure in steps 7-8 of clause 6.1.2 of TS 23.273 could be used.

7.  The AMF selects an LMF based on the LMF Ranging and/or sidelink positioning capabilities, other available information as defined in clause 5.1 of TS 23.273 or based on AMF local configuration. The LMF selection takes into consideration the 5G-AN that currently serves the UE. The selection may use a NRF query.

8.	The AMF invokes the Nlmf_Location_DetermineRanging service operation towards the LMF to request location information and/or ranging information of a target UE. The service operation includes an LCS Correlation identifier, the serving cell identity, the client type and may include an indication if target and located UE supports LPP, the required positioning QoS, Located UEs if available.

9.	The LMF determines whether to use sidelink positioning to position the target UE and/or provide ranging information e.g., in case the Uu-based positioning is not feasible or the expected QoS cannot be provided. If sidelink based positioning is triggered, then the LMF selects either option A or option B. For instance, Option B is selected if the tahrget UE is not directly reachable by the LMF or if it is associated via a relay UE (located UE).

Option A (steps 10-12): The LMF sends the request to the target UE for the sidelink positioning and/or ranging.

Option B: (steps 13-15): The LMF sends the request to a located UE receives request for the sidelink positioning and/or ranging.

(Option A)

10.	The LMF provides the request to the target UE to initiate the sidelink positioning and/or ranging calculation. The LMF provides candidate list of Located UEs (as received in step 8 above) to assist SL positioning of the target UE.

11.	Sidelink positioning and/or ranging calculation is taking place

12.	The target UE provides to the LMF the results of Sidelink positioning and/or ranging and if requested relative measurement data.

(Option B)

13.	The LMF provides the request to the located UE to initiate the sidelink positioning and/or ranging calculation, providing the target UE identifier. The located UE can also be a Relay UE e.g., in case the target UE is out of coverage.

14.	 Sidelink positioning and/or ranging calculation is taking place.

15.	 The located UE provides to the LMF the results of Sidelink positioning and/or ranging and if requested relative measurement data.

16.	Optionally, the LMF can use additional methods (e.g., Uu-based location services) together with the results received by the target UE or located UE to improve the quality of the positioning and/or ranging outputs. LMF can also further process received measurement data, if available, to determine position and ranging.

17.	The LMF provides the Nlmf_Location_DetermineRanging Response towards the AMF to return ranging information and target UE or located UE position and UE Positioning Capability. The response may also include additional information such as the LCS Correlation identifier, age of estimated position/ranging, accuracy and may include information about the positioning method and the timestamp of the location estimate.

18. The AMF returns the Namf_Ranging_ProvidePositioningInfo Response towards the GMLC with an indication of the result of notification.

19. GMLC may perform privacy check to decide whether the GMLC can forward the location information to AF. The steps 16-22 of clause 6.1.2 of TS 23.273 could be used.

20.	The GMLC sends the ranging and location service response to the AF via the NEF, if the target UE is allowed to be located by the LCS Client or AF. Accordingly, NEF invokes Nnef_EventExposure_Notify or sends Nnef_EventExposure_Subscribe Response to the AF.

# * * * * End of changes * * * *