SA WG2 Meeting #155	S2-2303241

Athens, Greece,  2023-02-20 – 2023-02-24

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3855 | rev | 4 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of support for L4S |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Deutsche Telekom, Nokia, Nokia Shanghai Bell, Meta USA, AT&T, Charter, T-Mobile USA, Apple, Verizon, BT, Telstra, CableLabs, Dish Network, MediaTek, ZTE, China Mobile, OPPO, Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA2 concluded in the scope of the WID on XR and media service to introduce support for ECN marking for L4S in NG-RAN and PSA UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR intrduces a stage 2 description and the requirements to support ECN marking for L4S by NG-RAN and PSA UPF. This CR enables support for low and bounded latency using two variants: - NG-RAN performs ECN marking for L4S in the ECN bits of the IP header according to the implementation specific criteria on a per QoS Flow basis in UL and DL. - PSA UPF performs ECN marking for L4S in the ECN bits of the IP header based on information received from NG-RAN in GTP-U header extension on a per QoS Flow basis in UL and DL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing support for ECN marking for L4S. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 5.37.Y (new), 5.37.Y.1 (new), 5.37.Y.2 (new), 5.37.Y.3 (new), 5.8.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | This CR has a dependency to 4056 to TS 23.501 Rev 2 is based on S2-2301470: - 5.37.Y.1: - Introduction of NOTE 1 addressing the EN, clarifying that selection of the method (NG-RAN or PSA UPF) is based on operator’s configuration and policies. - Correction to NOTE 2, where ‘may’ is replaced by ‘can’ and wording alignment with existing specification text. - Aadition of text to NOTE 3 that was removed by mistake - NOTE4 addressing UE impacts - 5.37.Y.2: - Clarifying the conditions for enabling L4S marking for a QoS Flow in NG-RAN - 5.37.Y.3: - Clarifying that the congestion ifnormation provided by NG-RAN for the method of L4S marking by UPF carries ‘percentage of packets to be marked’. - Some editorial clean-up |  |  |  |  |  |  |  |  |

********** Start Of Changes*************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TS 22.261: “Service requirements for next generation new services and markets; Stage 1”.

[3]	3GPP TS 23.502: “Procedures for the 5G System; Stage 2”.

[4]	3GPP TS 23.203: “Policies and Charging control architecture; Stage 2”.

[5]	3GPP TS 23.040: “Technical realization of the Short Message Service (SMS); Stage 2”.

[6]	3GPP TS 24.011: “Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3”.

[7]	IETF RFC 7157: “Ipv6 Multihoming without Network Address Translation”.

[8]	IETF RFC 4191: “Default Router Preferences and More-Specific Routes”.

[9]	IETF RFC 2131: “Dynamic Host Configuration Protocol”.

[10]	IETF RFC 4862: “Ipv6 Stateless Address Autoconfiguration”.

[11]	ITU-T Recommendation I.130: “Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN”.

[12]	ITU-T Recommendation Q.65: “The unified functional methodology for the characterization of services and network capabilities”.

[13]	3GPP TS 24.301: “Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3”.

[14]	IETF RFC 3736: “Stateless DHCP Service for Ipv6”.

[15]	3GPP TS 23.228: “IP Multimedia Subsystem (IMS); Stage 2”.

[16]	3GPP TS 22.173: “IMS Multimedia Telephony Service and supplementary services; Stage 1”.

[17]	3GPP TS 23.122: “Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode”.

[18]	3GPP TS 23.167: “3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions”.

[19]	3GPP TS 23.003: “Numbering, Addressing and Identification”.

[20]	IETF RFC 7542: “The Network Access Identifier”.

[21]	3GPP TS 23.002: “Network Architecture”.

[22]	3GPP TS 23.335: “User Data Convergence (UDC); Technical realization and information flows; Stage 2”.

[23]	3GPP TS 23.221: “Architectural requirements”.

[24]	3GPP TS 22.153: “Multimedia priority service”.

[25]	3GPP TS 22.011: “Service Accessibility”.

[26]	3GPP TS 23.401: “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

[27]	3GPP TS 38.300: “NR; NR and NG-RAN Overall Description”.

[28]	3GPP TS 38.331: “NR; Radio Resource Control (RRC); Protocol Specification”.

[29]	3GPP TS 33.501: “Security architecture and procedures for 5G system”.

[30]	3GPP TS 36.300: “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”.

[31]	3GPP TS 37.340: “Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2”.

[32]	3GPP TS 23.214: “Architecture enhancements for control and user plane separation of EPC nodes; Stage 2”.

[33]	3GPP TS 22.101: “3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles”.

[34]	3GPP TS 38.413: “NG-RAN; NG Application Protocol (NGAP)”.

[35]	3GPP TS 33.126: “Lawful Interception Requirements”.

[36]	3GPP TS 23.682: “Architecture enhancements to facilitate communications with packet data networks and applications”.

[37]	3GPP TS 22.280: “Mission Critical Services Common Requirements (MCCoRe); Stage 1”.

[38]	3GPP TS 23.379: “Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2”.

[39]	3GPP TS 23.281: “Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2”.

[40]	3GPP TS 23.282: “Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2”.

[41]	3GPP TS 32.240: “Charging management; Charging architecture and principles”.

[42]	3GPP TS 38.401: “NG-RAN Architecture description”.

[43]	3GPP TS 23.402: “Architecture enhancements for non-3GPP accesses”.

[44]	IETF RFC 4960: “Stream Control Transmission Protocol”.

[45]	3GPP TS 23.503: “Policy and Charging Control Framework for the 5G System”.

[46]	3GPP TS 23.041: “Public Warning System”.

[47]	3GPP TS 24.501: “Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3”.

[48]	3GPP TS 24.502: “Access to the 5G System (5GS) via non-3GPP access networks; Stage 3”.

[49]	3GPP TS 29.500: “5G System; Technical Realization of Service Based Architecture; Stage 3”.

[50]	3GPP TS 38.304: “NR; User Equipment (UE) procedures in idle mode”.

[51]	3GPP TS 36.331: “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification”.

[52]	3GPP TS 36.304: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode”.

[53]	Void.

[54]	IETF RFC 4861: “Neighbor Discovery for IP version 6 (Ipv6)”.

[55]	3GPP TS 23.271: “Functional stage 2 description of Location Services (LCS)”.

[56]	3GPP TS 23.060: “General Packet Radio Service (GPRS); Service description; Stage 2”.

[57]	IETF RFC 4555: “IKEv2 Mobility and Multihoming Protocol (MOBIKE)”.

[58]	3GPP TS 29.510: “5G System: Network function repository services; Stage 3”.

[59]	3GPP TS 29.502: “5G System: Session Management Services: Stage 3”.

[60]	IETF RFC 7296: “Internet Key Exchange Protocol Version 2 (IKEv2) “.

[61]	3GPP TS 23.380: “IMS Restoration Procedures”.

[62]	3GPP TS 24.229: “IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3”.

[63]	3GPP TS 23.292: “IP Multimedia Subsystem (IMS) centralized services; Stage 2”.

[64]	3GPP TS 23.222: “Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs”.

[65]	3GPP TS 29.244: “Interface between the Control Plane and the User Plane Nodes; Stage 3”.

[66]	3GPP TS 32.421: “Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements”.

[67]	3GPP TS 32.290: “5G system; Services, operations and procedures of charging using Service Based Interface (SBI)”.

[68]	3GPP TS 32.255: “5G Data connectivity domain charging; Stage 2”.

[69]	3GPP TS 38.306: “NR; User Equipment -UE) radio access capabilities”.

[70]	3GPP TS 36.306: “Evolved Universal Terrestrial Radio Access -E-UTRA); User Equipment -UE) radio access capabilities”.

[71]	3GPP TS 29.518: “5G System; Access and Mobility Management Services; Stage 3”.

[72]	3GPP TS 23.285: “Architecture enhancements for V2X services”.

[73]	IETF RFC 2865: “Remote Authentication Dial In User Service (RADIUS)”.

[74]	IETF RFC 3162: “RADIUS and Ipv6”.

[75]	3GPP TS 29.281: “General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)”.

[76]	3GPP TS 26.238: “Uplink streaming”.

[77]	3GPP TR 26.939: “Guidelines on the Framework for Live Uplink Streaming (FLUS)”.

[78]	International Telecommunication Union (ITU), Standardization Bureau (TSB): “Operational Bulletin No. 1156”; http://handle.itu.int/11.1002/pub/810cad63-en (retrieved October 5, 2018).

[79]	3GPP TS 28.533: “Management and orchestration; Architecture framework”.

[80]	3GPP TS 24.250: “Protocol for Reliable Data Service; Stage 3”.

[81]	IETF RFC 8684: “TCP Extensions for Multipath Operation with Multiple Addresses”.

[82]	IETF RFC 8803: “0-RTT TCP Convert Protocol”.

[83]	IEEE Std 802.1CB-2017: “IEEE Standard for Local and metropolitan area networks-Frame Replication and Elimination for Reliability”.

[84]	3GPP TS 23.316: “Wireless and wireline convergence access support for the 5G System (5GS)”.

[85]	WiFi Alliance Technical Committee, Hotspot 2.0 Technical Task Group: “Hotspot 2.0 (Release 2) Technical Specification”.

[86]	3GPP TS 23.288: “Architecture enhancements for 5G System (5GS) to support network data analytics services”.

[87]	3GPP TS 23.273: “5G System (5GS) Location Services (LCS); Stage 2”.

[88]	3GPP TS 23.216: “Single Radio Voice Call Continuity (SRVCC); Stage 2”.

[89]	CableLabs DOCSIS MULPI: “Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification”.

[90]	BBF TR-124 issue 5: “Functional Requirements for Broadband Residential Gateway Devices”.

[91]	BBF TR-101 issue 2: “Migration to Ethernet-Based Broadband Aggregation”.

[92]	BBF TR-178 issue 1: “Multi-service Broadband Network Architecture and Nodal Requirements”.

[93]	BBF TR-456 issue 2: “AGF Functional Requirements”.

[94]	BBF WT-457: “FMIF Functional Requirements”.

Editor’s note:	The reference to BBF WT-457 will be revised when finalized by BBF.

[95]	IEEE Std 802.1Qcc-2018: “IEEE Standard for Local and metropolitan area networks – Bridges and Bridged Networks – Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements”.

[96]	Void.

[97]	IEEE Std 802.1AB-2016: “IEEE Standard for Local and metropolitan area networks – Station and Media Access Control Connectivity Discovery”.

[98]	IEEE Std 802.1Q-2018: “IEEE Standard for Local and metropolitan area networks–Bridges and Bridged Networks”.

[99]	3GPP TS 38.423: “NG-RAN; Xn Application Protocol (XnAP)”.

[100]	3GPP TS 36.413: “Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)”.

[101]	3GPP TS 29.274: “Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3”.

[102]	3GPP TS 23.632: “User Data Interworking, Coexistence and Migration; stage 2”.

[103]	3GPP TS 29.563: “5G System (5GS); HSS services for interworking with UDM; Stage 3”.

[104]	IEEE Std 802.1AS-2020: “IEEE Standard for Local and metropolitan area networks–Timing and Synchronization for Time-Sensitive Applications”.

[105]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains”.

[106]	IEEE Std 802.11-2012: “IEEE Standard for Information technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements – Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications”.

[107]	IEEE Std 1588-2008: “IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems”.

[108]	3GPP TS 28.552: “Management and orchestration; 5G performance measurements”.

[109]	3GPP TS 24.193: “Access Traffic Steering, Switching and Splitting; Stage 3”.

[110]	3GPP TS 24.526: “User Equipment (UE) policies for 5G System (5GS); Stage 3”.

[111]	3GPP TS 22.186: “Enhancement of 3GPP support for V2X scenarios; Stage 1”.

[112]	3GPP TR 38.824: “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”.

[113]	IEEE: “Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)”, https://standards.ieee.org/content/dam/ieee-standards/standards/web/documents/tutorials/eui.pdf.

[114]	3GPP TS 32.256: “Charging Management; 5G connection and mobility domain charging; Stage 2”.

[115]	3GPP TS 33.210: “Network Domain Security (NDS); IP network layer security”.

[116]	3GPP TS 38.415: “PDU Session User Plane Protocol”.

[117]	3GPP TS 24.535: “Device-side Time-Sensitive Networking (TSN) Translator (DS-TT) to network-side TSN Translator (NW-TT) protocol aspects; Stage 3”.

[118]	3GPP TS 32.274: “Charging Management; Short Message Service (SMS) charging”.

[119]	3GPP TS 23.008: “Organization of subscriber data”.

[120]	3GPP TS 38.314: “NR; Layer 2 measurements”.

[121]	3GPP TS 23.287: “Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services”.

[122]	3GPP TS 29.503: “5G System; Unified Data Management Services; Stage 3”.

[123]	3GPP TS 32.254: “Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging”.

[124]	3GPP TS 33.535: “Authentication and Key Management for Applications based on 3GPP credentials in the 5G System (5GS)”.

[125]	3GPP TS 38.410: “NG-RAN; NG general aspects and principles”.

[126]	IEEE Std 1588: “IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems”, Edition 2019.

[127]	ST 2059-2:2015: “SMPTE Standard – SMPTE Profile for Use of IEEE-1588 Precision Time Protocol in Professional Broadcast Applications”.

[128]	3GPP TS 23.304: “Proximity based Services (ProSe) in the 5G System (5GS)”.

[129]	3GPP TS 23.247: “Architectural enhancements for 5G multicast-broadcast services”.

[130]	3GPP TS 23.548: “5G System Enhancements for Edge Computing; Stage 2”.

[131]	IEEE Std 802.3: “Ethernet”.

[132]	3GPP TS 29.561: “5G System; Interworking between 5G Network and external Data Networks; Stage 3”.

[133]	3GPP TS 29.513: “Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3”.

[134]	3GPP TS 23.558: “Architecture for enabling Edge Applications (EA)”.

[135]	3GPP TS 26.501: “5G Media Streaming (5GMS); General description and architecture”.

[136]	3GPP TS 23.256: “Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2”.

[137]	GSMA NG.116: “Generic Network Slice Template”.

[138]	IETF RFC 3948: “UDP Encapsulation of Ipsec ESP Packets”.

[139]	3GPP TS 24.539: “5G System (5GS); Network to TSN translator (TT) protocol aspects; Stage 3”.

[140]	3GPP TS 33.220: “Generic Authentication Architecture (GAA); Generic bootstrapping architecture”.

[141]	3GPP TS 33.223: “Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function”.

[142]	3GPP TS 23.540: “Technical realization of Service Based Short Message Service; Stage 2”.

[143]	3GPP TS 38.321: “NR; Medium Access Control (MAC) protocol specification”.

[144]	3GPP TS 29.525: “5G System; UE Policy Control Service; Stage 3”.

[145]	3GPP TS 29.505: “5G System; Usage of the Unified Data Repository Services for Subscription Data; Stage 3”.

[146]	IEEE Std P802.1Qdj-d0.3: “IEEE Draft Standard for Local and metropolitan area networks – Bridges and Bridged Networks – Amendment XX: Configuration Enhancements for Time-Sensitive Networking”.

[147]	3GPP TS 23.700-05: “Study on architecture enhancements for vehicle-mounted relays”.

[Ref1]	IETF RFC 9330: “Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture”.

[Ref2]	IETF RFC 9331: “Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S) “.

[Ref3] 		IETF RFC 9332: “Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)”

********** Next Changes*************

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5G DDNMF	5G Direct Discovery Name Management Function

5G LAN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-AN PDB	5G Access Network Packet Delay Budget

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G GM	5G Grand Master

5G NSWO	5G Non-Seamless WLAN offload

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G VN	5G Virtual Network

5QI	5G QoS Identifier

ADRF	Analytics Data Repository Function

AF	Application Function

AKMA	Authentication and Key Management for Applications

AnLF	Analytics Logical Function

AMF	Access and Mobility Management Function

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CH	Credentials Holder

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DCCF	Data Collection Coordination Function

DCS	Default Credentials Server

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

EAC	Early Admission Control

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GBA	Generic Bootstrapping Architecture

GEO	Geostationary Orbit

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network Selection

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HMTC	High-Performance Machine-Type Communications

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IMEI/TAC	IMEI Type Allocation Code

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

I-UPF	Intermediate UPF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LEO	Low Earth Orbit

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

L4S	Low Latency, Low Loss and Scalable Throughput

MBS	Multicast/Broadcast Service

MBSF	Multicast/Broadcast Service Function

MBSR	Mobile Base Station Relay

MBSTF	Multicast/Broadcast Service Transport Function

MB-SMF	Multicast/Broadcast Session Management Function

MB-UPF	Multicast/Broadcast User Plane Function

MEO	Medium Earth Orbit

MFAF	Messaging Framework Adaptor Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MINT	Minimization of Service Interruption

ML	Machine Learning

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

MTLF	Model Training Logical Function

N3IWF	Non-3GPP InterWorking Function

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

NSAG	Network Slice AS Group

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-specific and SNPN Authentication and Authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NSSRG	Network Slice Simultaneous Registration Group

NSWO	Non-Seamless WLAN offload

NSWOF	Non-Seamless WLAN offload Function

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

ONN	Onboarding Network

ON-SNPN	Onboarding Standalone Non-Public Network

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PEI	Permanent Equipment Identifier

PER	Packet Error Rate

PFD	Packet Flow Description

PNI-NPN	Public Network Integrated Non-Public Network

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PTP	Precision Time Protocol

PVS	Provisioning Server

QFI	QoS Flow Identifier

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

®AN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SO-SNPN	Subscription Owner Standalone Non-Public Network

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TA	Tracking Area

TAI	Tracking Area Identity

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAI	TSC Assistance Information

TSCTSF	Time Sensitive Communication and Time Synchronization Function

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TT	TSN Translator

TWIF	Trusted WLAN Interworking Function

UAS NF	Uncrewed Aerial System Network Function

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

********** Next Changes (all new text)*************

### 5.37.Y Support of ECN marking for L4S to expose the congestion information

#### 5.37.Y.1 General

L4S (Low Latency, Low Loss and Scalable Throughput) is described in IETFRFC 9330 [Ref1], IETF RFC 9331 [Ref2] and IETF RFC 9332 [Ref3]. It exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the application server to trigger application layer rate adaptation.

In 5G System, ECN marking for L4S may be supported. ECN marking for L4S is enabled on a per QoS Flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS Flows. ECN marking for the L4S in the IP header is supported in either the NG-RAN (see clause 5.37.Y.2 and TS 38.300 [27]), or in the PSA UPF (see clause 5.37.Y.3).

NOTE 1: Whether NG-RAN or PSA UPF based ECN marking for L4S is used is decided by SMF based on operator’s network configuration and policies.

In the case of ECN marking for L4S by UPF, the NG-RAN is instructed to perform congestion information monitoring..

NOTE 2: As for any QoS flow, QoS rules in the UE and PDRs in the PSA UPF control which packets are bound to the L4S enabled QoS flow. The Packet Filter Set in the QoS rule or PDR can use packet filter(s) in clause 5.7.6.2 (e.g., ECT(1) and/or IP 5 tuple) to steer traffic to an L4S enabled QoS Flow.

NOTE 3: A QoS flow may be enabled with ECN marking for L4S requirement e.g., statically when a PDU session is established based on configuration in SMF or PCF, or dynamically based on detection of  the L4S traffic e.g., via ECT(1) and/or IP 5 tuple in the IP header whereby SMF or PCF triggers a setup of a QoS Flow enabled for L4S, or by requests by an AF.

NOTE: 4: To support this functionality, the UE needs to support transport for L4S feedback in the transport protocol.

Editor’s Note: During UE mobility, e.g., NG-RAN handover or local PSA UPF relocation, whether there are other impacts for ECN marking for L4S is FFS.

#### 5.37.Y.2 Support of ECN marking for L4S in NG-RAN

ECN marking for L4S may be supported in NG-RAN as specified in TS 38.300 [27].

To enable support of ECN marking for L4S in NG-RAN, dedicated QoS Flow(s) are used for carrying L4S enabled IP traffic. The SMF may be configured to, based on PCC Rule provide an indication for ECN marking for L4S to NG-RAN for a corresponding QoS Flow(s), but in the absence of such configuration the use of L4S on a QoS flow is controlled by a coordinated configuration in NG-RAN and 5GC.

The criteria based on which NG-RAN decides to mark ECN bits for L4S is NG-RAN implementation specific.

#### 5.37.Y.3 Support of ECN marking for L4S in PSA UPF

To enable ECN marking for L4S by a PSA UPF, a QoS Flow level ECN marking for L4S indicator may be sent by SMF to PSA UPF over N4. SMF also indicates to NG-RAN to report the congestion information (i.e., a percentage of packets that UPF uses for ECN marking for L4S) of the QoS Flow on UL and/or DL directions via GTP-U header extension to PSA UPF. If there is no UL packet when report for DL and/or UL needs to be provided, NG-RAN may generate an UL Dummy GTP-U Packet for such a reporting.

Upon successful activation of congestion information reporting for UL and/or DL, PSA UPF uses information sent by NG-RAN in GTP-U header extension (see TS 38.415 [116] and TS 38.300 [27]) to perform ECN bits marking for L4S for the corresponding direction.

********** Next Changes*************

#### 5.8.2.7	PDU Session and QoS Flow Policing

ARP is used for admission control (i.e. retention and pre-emption of the new QoS Flow). The value of ARP is not required to be provided to the UPF.

For every QoS Flow, the SMF shall determine the transport level packet marking value (e.g. the DSCP in the outer IP header) based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level and provide the transport level packet marking value to the UPF.

The SMF shall provide the Session-AMBR values of the PDU Session to the UPF so that the UPF can enforce the Session-AMBR of the PDU Session across all Non-GBR QoS Flows of the PDU Session.

SMF shall provide the GFBR and MFBR value for each GBR QoS Flow of the PDU Session to the UPF. SMF may also provide the Averaging window to the UPF, if Averaging window is not configured at the UPF or if it is different from the default value configured at the UPF.

SMF may decide to activate ECN marking for L4S by PSA UPF for the QoS Flow (see clause 5.37).In this case, the SMF shall send an ECN marking for L4S indicator to UPF.

********** End Of Changes*************