3GPP TSG-SA WG2 Meeting #155	S2-2303366

# Athens, GR, Februrary 20 - 24, 2023                                            revision of S2-2302354

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0218 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Link sharing for UE-to-UE Relay with Integrated Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Editor's notes in clause 6.7.3.2 about PC5 link modification is for the existing PC5 link sharing towards different End UEs. However, the abnormal case for this should be addressed in stage 3. Those ENs should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the ENs of link sharing for integarted discovery procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ENs are not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.7.3.2	Procedure

Figure 6.7.3.2-1: 5G ProSe Discovery integrated into PC5 unicast link establishment procedure

0.	UEs are authorized and provisioned with parameters to use the service provided by the UE-to-UE relays. UE-to-UE relays are authorized and provisioned with parameters to provide service of relaying traffic among UEs.

1.	UE-1 wants to establish a unicast communication with UE-2 and broadcasts a Direct Communication Request. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.

The relay_indication in the Direct Communication Request is used to indicate whether UE-to-UE relay can forward the Direct Communication Request message or not. It is also used to limit the number of hops of UE-to-UE relay by removing relay_indication in the Direct Communication Request message from the UE-to-UE Relay.

NOTE 1:	The data type of relay_indication can be determined in Stage 3.

2.	When receiving Direct Communication Request with relay_indication from UE-1, Relay (here, relay-1 and relay-2) may decide to participate in the procedure and broadcast a Direct Communication Request message in its proximity without relay_indication. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.

The Direct Communication Request message uses relay's Layer-2 ID as the source Layer-2 ID.

3.	When UE-2 receives a Direct Communication Request from one or multiple UE-to-UE relays, UE-2 select a UE-to-UE Relay which UE-2 will respond. UE-2 may select UE-to-UE relay according to the signal strength, local policy, operator policy per Relay Service Code if any.

4.	security establishment happens between UE-2 and the selected 5G ProSe UE-to-UE Relay (here relay-1), if needed.

In case the existing PC5 link can be reused, Link Modification Request and Link Modification Accept messages is used.

5.	UE-2 replies Direct Communication Accept message to relay-1. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.7.

6. For IP traffic, IPv6 prefix or IPv4 address is allocated for the target 5G ProSe Layer-3 End UE as defined in clause 5.5.1.3.

7.	security establishment happens between UE-1 and relay-1, if needed.

8.	Relay-1 responds with Direct Communication Accept to the UE-1. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.7.

9. For IP traffic, IPv6 prefix or IPv4 address is allocated for the source 5G ProSe Layer-3 End UE as defined in clause 5.5.1.3.

Editor's note:	It is FFS how to support non-IP case.

* * * * End of Changes * * * *