3GPP TSG-SA WG2 Meeting #155	S2-2303425

Athens, Greece, February 20 – 24, 2023	(revision of S2-2303420)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3956 | rev | 10 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transfer of Satellite Coverage Data to a UE and AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Intel, CATT, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The conclusions for FS_5GSAT_Ph2 in TR 23.700-28, include supporting the transfer of satellite coverage availability to a UE and AMF. This CR introduces the support as proposed in a comparison of alternative solutions in S2-2300668. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for transfer of coverage availability to a UE and to an AMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transfer of enhanced satellite coverage availability to a UE compared to support for EPS in Release 17 and to an AMF would not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X.Y (new), 5.4.X.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Guidance to MCC, please insert the CRs in this order: 5.4.X Support of discontinuous network coverage for satellite access (from CR4033) 5.4.X.1 Mobility Management and Power Saving Optimization (from CR4033) 5.4.X.x UE coverage information provisioning to the UE (from CR3956) 5.4.X.Z UE Coverage information provisioning to the AMF (from CR3956) 5.4.X.y Paging (from CR3966) 5.4.X.y Overload control (from CR4020) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | SA2#155: Alignment of the term satellite coverage availability information, clarifications about satellite coverage availability information comes from an external server and the satellite coverage availability information is not per UE. |  |  |  |  |  |  |  |  |

**** First Change (all new) ****

5.4.X Support of discontinuous network coverage

#### 5.4.X.Y Coverage availability information provisioning to the UE

A UE may use satellite coverage availability information for satellite access to support discontinuous coverage operations. Satellite coverage availability information can be provided to a UE by an external server via a PDU Session or SMS.

#### 5.4.X.Z Coverage availability information provisioning to the AMF

The AMF may use satellite coverage availability information to support satellite access by UEs with discontinuous coverage operation. Satellite coverage availability information may be provisioned to the AMF by O&M or by AF.

The satellite coverage availability information provisioned to the AMF describes when and where satellite coverage is expected to be available in an area. The satellite coverage availability information is not UE specific and can be applied by the AMF for any UE in the affected area.

Editor’s Note: 	It needs to be determined whether both sources (i.e. O&M and AF) of satellite coverage availability information will be supported or just one of them.

Editor's Note:	Whether NEF exposure procedure can be re-used for provisioning satellite coverage availability information from the AF is FFS.

**** End of Changes ****