3GPP TSG-WG SA2 Meeting #155	S2-2303501

# Athens, Greece, Feb 20th – Feb 24th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0723 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Model accuracy guarantee during FL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the conclusions of KI#8 in TR 23.700-81, NWDAF containing MTLF as FL Server can monitor the accuracy level of the trained model and exclude a Client NWDAF from the FL group if the Accuracy feedback of the initial/common model calculated by the Client NWDAF is much different from the Accuracy calculated from the AnLF. How FL server receives the accuracy feedback from AnLF is following KI#1. At last meeting, the CR S2-2301980 is agreed which introduces AnLF-assisted MTLF ML Models Accuracy Monitoring, according to the conclusions of KI#1 in TR 23.700-81. Besides subscribe-notify model services, request-response model services can also be defined for Models Accuracy request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the procedures related to the ML Model Accuracy guarantee when MTLF acts as Federated Learning server. - Adding the procedures related to the ML Model Accuracy request for the purpose of AnLF-assisted MTLF ML Models Accuracy Monitoring. - Adding NWDAF services description of ML Model Accuracy request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R18 agreements on KI#8 and KI#1 will not be properly captured in R18 NWDAF description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.Z (new), 6.2E.3.X (new), 7.1, 7.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2C.2.Z Procedures for Model Accuracy guarantee during Federated Learning

This clause specifies how to guarantee the Model Accuracy in the Federated Learning execution phase.

During Federated Learning, the global ML model is generated by aggregating local model information from FL client NWDAF(s). The FL server NWDAF monitors the Model Accuracy of the global ML model by requesting the FL client NWDAF(s) to provide the Model Accuracy of the global ML model, which is calculated using the local dataset in the FL client NWDAF(s). The FL server NWDAF may calculate the Model Accuracy of the global ML model by itself when it has enough data. During the Model Accuracy monitoring for Federated Learning, the FL server NWDAF should remove a FL client NWDAF from the FL members if the Model Accuracy of the global ML model calculated by the FL client NWDAF is much different from that calculated by other FL client NWDAFs, to avoid the Model Accuracy of the aggregated global ML model is keeping affected by the different data distribution of the local data in the FL client NWDAF.

FL client NWDAF N

FL server NWDAF

2. Nnwdaf_MLModelAccuracy_Request

…

FL client NWDAF 1

1. Aggregated global ML Model provisioning from FL server NWDAF to FL client NWDAFs

3. Nnwdaf_MLModelAccuracy_Request Response

4. Reselect FL members for next iteration according to the ML Model check result if needed.

5. Training with local data, local model information reporting

and model aggregation

Repeat

Figure 6.2C.2.Z-1: Procedure of Model Accuracy guarantee in the FL execution phase

The procedure for FL server NWDAF monitoring Model Accuracy in the Federated Learning execution phase is as follows:

- The FL server NWDAF aggregates all the local ML model information retrieved from the FL client NWDAF(s) to update the global ML model, and sends the aggregated ML model information to each FL client NWDAF. Each FL client NWDAF updates its own ML model based on the aggregated ML model information distributed by the FL server NWDAF.

- The FL server NWDAF sends Nnwdaf_MLModelAccuracy_Request (Analytics ID(s), identifier of the global ML model, accuracy metrics to be checked) to each FL client NWDAF.

- The FL client NWDAF calculates the Model Accuracy of the global ML Model using the local training data as the testing dataset, and sends a Nnwdaf_MLModelAccuracy_Request Response (Analytics ID(s), identifier of the global ML model, accuracy metrics checked) to the FL server NWDAF.

- The FL server NWDAF reselects FL members for the next iteration according to the global ML Model check results if needed. If the accuracy metrics calculated by a FL client MTLF are much different from that by other FL client MTLFs or that by the AnLF, the Sever NWDAF should remove the FL client MTLF from the FL members. If the FL server NWDAF reselects a new MTLF as the FL client NWDAF steps 1 to 3 should be applied before adding the new MTLF into the FL members.

- The FL server NWDAF and the FL client NWDAFs repeat FL training iterations. After certain iterations, the FL server NWDAF may provide the global ML model to the AnLF to check the Model Accuracy (see clause 6.2E.3.X), before stopping the Federated Learning.

# * * * * Next change * * * *

6.2E.3.X	Procedures for checking the analytics accuracy of a ML model

A NWDAF containing MTLF requests a NWDAF containing AnLF to check Model Accuracy of a ML Model previously provisioned. If the NWDAF containing AnLF has the accuracy check capability and gets collected data with the corresponding ground truth from ADRF or storage of itself, it calculates the Model Accuracy of the specific ML Model using the already collected data with the corresponding ground truth and returns the Model Accuracy in response; Otherwise, the NWDAF containing AnLF will return a failure with corresponding cause.

Figure 6.2E.3.X-1 illustrates the procedure of checking the analytics accuracy of an ML model for the purpose of AnLF-assisted MTLF ML Models Accuracy Monitoring. Using Nnwdaf_MLModelAccuracy_Request service operations, a service consumer, i.e. an NWDAF containing MTLF, requests a service producer, i.e. a NWDAF containing AnLF, to provide Model Accuracy of a previously provisioned ML model.

NWDAF containing

MTLF

NWDAF containing

AnLF

0. ML Model provisioning from MTLF to AnLF

2. Calculate the Model Accuracy of the specific ML Model, if possible; Otherwise determine the failure cause.

1. Nnwdaf_MLModelAccuracy_Request

3. Nnwdaf_MLModelAccuracy_Request Response

Figure 6.2E.3.X-1: Procedure for checking analytics accuracy of a ML model

0.	[Precondition] An NWDAF containing MTLF previously provides a ML model to a NWDAF containing AnLF.

1.	The NWDAF containing MTLF sends a Nnwdaf_MLModelAccuracy_Request (Analytics ID(s), unique identifier of the ML model to be checked, accuracy metrics to be checked) to the NWDAF containing AnLF.

NOTE 1:	The NWDAF containing MTLF acting as the service consumer can trigger Model Accuracy check for many purposes: e.g. to decide which ML model among multiple candidates is more suitable to the AnLF, to check the model performance of a ML model already provisioned to an AnLF periodically.

2.	If the NWDAF containing AnLF has the accuracy check capability and collected data with the corresponding ground truth from ADRF or storage of itself, it calculates the Model Accuracy of the specific ML Model by comparing the prediction generated by the ML mode using the already collected data against the corresponding ground truth. Otherwise, the NWDAF containing AnLF determines the failure cause, e.g. service not supported, Analytics ID not supported, data not ready.

3.	The NWDAF containing AnLF sends a Nnwdaf_MLModelAccuracy_Request Response (Analytics ID(s), unique identifier of the ML model, [accuracy metrics checked] or [failure cause]).

NOTE 2: This procedure is also used by FL server NWDAF to check model accuracy of the global ML model from FL client NWDAF.

# * * * * Next change * * * *

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_MLModelAccuracy | Request | Request / Response | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |

# * * * * Next change * * * *

7.X	Nnwdaf_MLModelAccuracy Service

7.X.1	General

Service Description: This service enables the consumer (e.g. a NWDAF containing MTLF, a FL server NWDAF in Federated Learning) to request the producer (e.g. a NWDAF containing AnLF, a FL client NWDAF in Federated Learning) to check Model Accuracy of a specific ML Model previously provisioned.

If the NWDAF containing AnLF acting as the producer has the accuracy check capability and already collected data with the corresponding ground truth from ADRF or storage of itself, it calculates the Model Accuracy of the specific ML Model using the already collected data with the corresponding ground truth and returns the Model Accuracy in response; Otherwise, the NWDAF containing AnLF returns a failure with corresponding cause.

When the FL client NWDAF in Federated Learning acts as the producer, it calculates the Model Accuracy of the global ML Model using the local training data as the testing dataset and sends a Nnwdaf_MLModelAccuracy_Request Response to the FL server NWDAF.

7.X.2	Nnwdaf_MLModelAccuracy_Request service operation

Service operation name: Nnwdaf_MLModelAccuracy_Request

Description: The consumer requests NWDAF to check Model Accuracy of the ML Model previously provisioned.

Inputs, Required: Analytics ID(s) defined in Table 7.1-2, identifier of the ML model to be checked.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: accuracy metrics checked, failure cause (e.g. service not supported, Analytics ID not supported, data not ready).

# * * * * End of changes * * * *