3GPP TSG-SA2 Meeting #155 	S2-2303551

# Athens, Greek,  February 20th – 24th, 2023	                                Revision of S2-2303484

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0628 | rev | 4 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for Federated Learning among Multiple NWDAFs in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, NTT DOCOMO, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To address the 3 Editor’s notes left in 6.2C Editor’s Note: Procedure of MTLF registration and discovery with respect to FL is FFS. - Propose the registration and discovery procedure for FL in 6.2C.1. MTLF (FL server or FL client) register s to NRF with its NF profile, so the FL server and FL clients can be selected when FL is needed. Editor’s Note: Definition of FL capability is FFS. - Clarify that the FL capability type can be FL server or FL client. Editor’ Note: Further details on providing the accuracy of FL model process is for FFS. - Propose the exchanging process of model metric among consumer, FL server and FL client. Revision of S2-2302430: To make clarification and remove the following 2 ENs: Editor's note: The ML model information exchanged between the Client NWDAF(s) and the Server NWDAF is FFS. Editor's note: It is FFS how the global model metric is derived during the procedure for Federated Learning among Multiple NWDAF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the Federated Learning procedure in 6.2C to remove 3 ENs |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | In revision 2, the following ENs are addressed: - Editor's note: It is FFS how the global model metric is derived during the procedure for Federated Learning among Multiple NWDAF. - Editor's note: The ML model information exchanged between the Client NWDAF(s) and the Server NWDAF is FFS. |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.2C	Federated Learning among Multiple NWDAFs

6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Federated Learning technique to train an ML model.

6.2C.2	Procedures

6.2C.2.1	Registration and Discovery procedure for Federated Learning

Figure 6.2X.2.1-1: Registration and Discovery procedure for Federated Learning

1-3.  NWDAF containing MTLF as Server NWDAF or Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability information including FL capability Type (i.e. FL server or FL client) and Time interval supporting FL as described in clause 5.2.

Editor's note:	Definition of FL capability and whether FL capability includes computation capability is FFS.

4-6  NWDAF containing MTLF determines ML model requires FL based on Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as Server NWDAF, the MTLF first discovers and selects Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, Model filter information as defined in TS 23.288 [5], FL capability Type (i.e. FL server), check if the selected Server NWDAF is currently executing an  Federated Learning procedure for the Analytics ID, Time Period of Interest, Service Area.

Once the Server NWDAF (the requested or the selected one) is determined, it discovers and selects other NWDAF(s) containing MTLF as Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, FL capability Type (i.e. FL client), Service Area, Data availability by the Client NWDAF, Time Period of Interest, Interoperability Indicator.

Editor’s Note: Additional criteria for FL server to discover FL clients from the NRF (e.g. data availability) is FFS.

7.  Server NWDAF sends Federated Learning preparation request to the Client NWDAF(s) with Interoperability information. In the preparation request, the following parameters may be added: Available data requirement, availability time requirement.

Editor’s Note: Other parameters in the preparation request and the definition of “Available data requirement” and “availability time requirement” are FFS.

8.  Client NWDAF(s) decides whether to join the Federated Learning process based on implementation. Example criteria used by a Client NWDAF(s) may be based on its availability, computation and communication capability, and Interoperability information.

9.  Client NWDAF(s) sends response to Server NWDAF indicating if it will join the FL procedure.

10.  Server NWDAF conducts selection of Client NWDAF(s).

6.2C.2.2	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.2-1: General procedure for Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF) sends a subscription request to NWDAF containing MTLF to   retrieve a ML model, using Nnwdaf_MLModelProvision service as defined in TS 23.288 [5], including Analytics ID, model metric (e.g., accuracy, precision, recall, etc), pre-determined status (e.g. requested accuracy level and/or total training time) and/or periodically reporting condition (e.g. training round/time).

NOTE 1: The requested accuracy level can be used to indicate the target accuracy level of the training process, and the FL server NWDAF may stop the training process when the requested accuracy level is achieved during the training process.

1.	Server NWDAF sends a request to the selected NWDAF containing MTLF(Client NWDAF) that participates in the Federated learning to perform the local model training for Federated Learning, including model metric.

2.	Each Client NWDAF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

3.	During Federated Learning training procedure, each Client NWDAF further trains the retrieved ML model from the Server NWDAF based on its own data, and reports interim local ML model information to the Server NWDAF. Each Client NWDAF also computes local model metric and includes it in the local training status report sent to the Server NWDAF, which optionally also contains other information about local training in the Client NWDAF, e.g., characteristics of local training dataset (e.g., area, sampling ratio, max/min of value of each dimension of data), etc.

NOTE 2: The parameters in characteristics of local training dataset are up to the implementation.

The ML model information, which is sent from the Client NWDAF(s) to the Server NWDAF during the FL training process, is the information needed by the Server NWDAF to build the aggregated model based on the locally trained models

Editor's note:	The services (i.e., Nnwdaf_MLModelTraining, Nnwdaf_MLModelTrainingInfo) in step 1, 3, 5a and 6 will be updated in the Figure 6.2C.2.1-1.

4.	The Server NWDAF aggregates all the local ML model information retrieved at step 3, to update the global ML model.  The Server NWDAF may also compute the global model metric, e.g. based on the local model metric(s) or by applying the global model on the validation dataset (if available).

5a.	[Optional] Based on the consumer request, the Server NWDAF updates the training status (e.g., global model metric) to the consumer periodically (e.g. a certain number of training rounds or every 10 min) or dynamically when some pre-determined status is achieved (e.g. the requested accuracy level is achieved or training time expires).

5b.	[Optional] Consumer decides whether the current model can fulfil the requirement, e.g. global model metric is satisfactory for the consumer. The consumer modifies subscription (stops or continues the training process) according to the consumer decision.

5c. [Optional]	According to the request from the consumer, Server NWDAF updates or terminates the current FL training process.

6.	If the FL procedure continues, Server NWDAF sends the aggregated ML model information to each Client NWDAF for next round model training.

7.	Each Client NWDAF updates its own ML model based on the aggregated ML model information distributed by the Server NWDAF at step 6.

NOTE 3:	The steps 2-7 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

After the training procedure is complete, the Server NWDAF may send the globally optimal ML model information to the consumer.

END of CHANGES