3GPP TSG-SA2 Meeting #155	S2-2303648

# Athens, Greece, February 20 – 24, 2023									       (revision of S2-2302757)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3915 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys: KI#7 – UE latency performace filtering criterion supported by the NEF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Nokia, Nokia Shanghai Bell, SK Telecom, Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to enable 5GC to assist the Application AI/ML AF to preform candidate member selection, a new NEF services for assistance to member selection is specified. The AF can request the NEF to provide a list of recommended UEs based on the member selection filtering criteria requested by the AF. UE lantecy performace is one of the filtering criteria that can let NEF provide a list of UE which fulfil the transimission latency threshold requested by the AF. Then the AF can further determine which UE(s) should be selected to participate in the AIML operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a UE latency performance criterion and the corresponding description. - Add the procedure on how the NEF can use such criterion to derive the candidate UE list and provide to the AF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide a 5GC mechanism to support the member selection based on the UE latency performance to assist the Application AI/ML operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.Y.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_AIMLsys. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

4.15.Y.2	Filtering Criteria for 5GS assistance for UE member selection

Table 4.15.Y.2-1 provides a summary of the of the UE member filtering criteria that the AIML AF may request.

Table 4.15.Y.2-1: Description of UE Member filtering criteria

| UE Member filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| E2E data volume transfer time | Indicate the E2E data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE to AF | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = E2E data volume transfer time | Clasue 6.x in TS 23.288. |

* * * End of Changes * * *