3GPP TSG-WG SA2 Meeting #155	S2-2303739

# Athens, Greece, Feb 20th – 24th, 2023      					(revision of S2-2303653 and S2-2302375)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0699 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhance WLAN performance analytics for Federated Learing member selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following conclusion has been added to TR 23.700-80: WLAN performance analytics are enhanced to allow analytics outputs on a per UE granularity. For UEs' list provided by the AF, the NWDAF can provide WLAN performance analytics per UE granularity to assist AF to determine the final list of member UEs. This CR is to enhance the WLAN performance analytics in TS 23.288 according to this conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhance the existing WLAN performance analytics to support per UE granularity explicitly; - Clarify the consumer of the WLAN performance analytics per UE could be NEF or AF, which could assit them to do federated learning member selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WLAN performance analytics can only support to monitor WLAN performance per SSID or per area but rather than per UE, which is useful for AF to do Federated Learning member selection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.11, 6.11.1, 6.11.2, 6.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.11	WLAN performance analytics

6.11.1	General

The NWDAF provides WLAN performance analytics to a service consumer NF. The analytics results are generated based on the data from other 5GC NFs and OAM. The analytics results, provided in the form of statistics or predictions, contain quality and performance of WLAN connection of UE according to UE location and SSID. The consumer can request either one-time or continuous reporting.

The service consumer may be an NF (e.g. PCF, NEF or AF).

If a service consumer is PCF, the WLAN performance analytics can be used to update WLANSP as defined in TS 23.503 [4].

If the service consumer is NEF or AF, the WLAN performance analytics per UE granularity can be used to select candidate members for an application layer operation.

The consumer of these analytics may indicate in the request or subscription:

-	Analytics ID = "WLAN performance";

-	Target of Analytics Reporting: a single UE (SUPI), a group of UEs (an Internal Group ID), or any UE;

-	Analytics Filter Information:

-	Area of Interest (list of TA or Cells);

-	SSID(s);

-	BSSID(s); and

-	optional list of analytics subsets that are requested (see clause 6.11.3);

-	An Analytics target period indicates the time period over which the statistics or prediction are requested, either in the past or in the future;

-	Optional maximum number of objects;

-	Preferred level of accuracy of the analytics;

-	Preferred level of accuracy per analytics subset (see clause 6.11.3);

-	Preferred order of results for the list of WLAN performance information:

-	ordering criterion: "time slot start", "number of UEs", "RSSI", "RTT" or "Traffic information"; and

-	order: ascending or descending; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

If the Target of Analytics Reporting is any UE, then the Analytics Filter should at least include Area of Interest or SSID(s) or BSSID(s).

6.11.2	Input Data

For the purpose of generating WLAN performance analytics results, the NWDAF collects the data as listed in Table 6.11.2-1.

Table 6.11.2-1: Data collected by NWDAF for WLAN performance analytics

| Information | Source | Description |
| --- | --- | --- |
| WLAN measurement results | OAM | The WLAN measurement results per wireless network served by the WLAN AP. |
| > SSID / BSSID / HESSID |  | SSID / BSSID / HESSID of the selected WLAN during the period of analysis. |
| > RSSI |  | Measured RSSI of the selected WLAN during the period of analysis. |
| > RTT |  | Measured RTT of the selected WLAN during the period of interest. |
| > UE Location |  | Location information tagged by UE when it reports WLAN MDT measurement (e.g. Cell ID and/or longitude/latitude if available). |
| Information on PDU Session for WLAN (1..max) | SMF | Information on PDU session for which Access Type is Non-3GPP and RAT Type is TRUSTED_WLAN. |
| > UE ID |  | SUPI |
| > SSID / BSSID |  | SSID / BSSID that the PDU session is related to. |
| > Start time of the PDU Session for WLAN |  | The time stamp that indicates when the existing PDU Session's access type changes to WLAN or when the new PDU Session for WLAN is established. |
| > End time of the PDU Session for WLAN |  | The time stamp that indicates when the existing WLAN based PDU Session's access type is not WLAN any more or when the PDU Session for WLAN is released. |
| UE communications (1..max) | UPF | List of communication time slots |
| > Communication start |  | The time stamp that PDU session(s) for WLAN starts. |
| > Communication stop |  | The time stamp that PDU session(s) for WLAN ends. |
| > UL data rate |  | UL data rate of PDU session(s) for WLAN. |
| > DL data rate |  | DL data rate of PDU session(s) for WLAN. |
| > Traffic volume |  | Traffic volume of PDU session(s) for WLAN. |

NOTE 1:	WLAN Data from OAM is collected via MDT and aligned with the WLAN measurement reporting list described in clause 5.1.1.3.3 of TS 37.320 [20]. It is assumed that not all UEs support MDT WLAN measurements.

NOTE 2:	UE Location from OAM can be used to deduce WLAN location.

6.11.3	Output Analytics

The NWDAF generates WLAN performance analytics. Depending on the Analytics Target Period, the output consists of statistics or predictions. The detailed information provided by the NWDAF is defined in Table 6.11.3-1 for statistics and Table 6.11.3-2 for predictions.

Table 6.11.3-1: WLAN performance statistics

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell Ids |
| List of Analytics per SSID | SSIDs of WLAN access points deployed in the Area of Interest |
| > Time slot entry (1..max) | List of time slots during the Analytics target period |
| >> Time slot start | Time slot start time within the Analytics target period |
| >> Duration | Duration of the time slot |
| >> RSSI (NOTE 1) | Measured RSSI |
| >> RTT (NOTE 1) | Measured RTT |
| >> Traffic Information (NOTE 1) | UL/DL data rate, Traffic volume |
| >> Number of UEs (NOTE 1) | Number of UEs observed for the SSID |
| List of Analytics per UE | UE ID(s) of WLAN performance analytics |
| > Time slot entry (1..max) | List of time slots during the Analytics target period |
| >> Time slot start | Time slot start time within the Analytics target period |
| >> Duration | Duration of the time slot |
| >> RSSI (NOTE 1) | Measured RSSI |
| >> RTT (NOTE 1) | Measured RTT |
| >> Traffic Information (NOTE 1) | UL/DL data rate, Traffic volume |
| NOTE 1: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset". |  |

Table 6.11.3-2: WLAN performance predictions

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell Ids |
| List of Analytics per SSID | SSIDs of WLAN access points deployed in the Area of Interest |
| > Time slot entry (1..max) | List of time slots during the Analytics target period |
| >> Time slot start | Time slot start time within the Analytics target period |
| >> Duration | Duration of the time slot |
| >> RSSI (NOTE 1) | Predicted RSSI |
| >> RTT (NOTE 1) | Predicted RTT |
| >> Traffic Information (NOTE 1) | Predicted UL/DL data rate, Traffic volume |
| >> Number of UEs (NOTE 1) | Number of UEs predicted for the SSID |
| List of Analytics per UE | UE ID(s) of WLAN performance analytics |
| > Time slot entry (1..max) | List of time slots during the Analytics target period |
| >> Time slot start | Time slot start time within the Analytics target period |
| >> Duration | Duration of the time slot |
| >> RSSI (NOTE 1) | Measured RSSI |
| >> RTT (NOTE 1) | Measured RTT |
| >> Traffic Information (NOTE 1) | UL/DL data rate, Traffic volume |
| >> Confidence | Confidence of the prediction |
| NOTE 1: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset". |  |

* * * End of Changes * * *