3GPP TSG-SA2 Meeting #155	S2-xxxxxxx

# Athens, Germany, Feb 20 - 24, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4113 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DCAF Discovery |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | IPLOOK |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  |  |  |  |
| Category: | B |  |  |  |  |  |  | Release: |  |  |  | Rel-18 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |
| Reason for change: | TR 23.700-91 has concluded following: UE data is transmitted via user-plane to ASP server and then the ASP Server sends the data to the DCAF. When this DCAF discovers the NF(s) from which data should be retrived, and finds NF(s) (e.g. SMF) associated to another DCAF it discovers the DCAF associated with the NF via NRF using this NF as input data. |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: | - Introduce how an NF can discovers a DCAF. |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | Not possible to collect data, and request statistics in a multiple DCAF deployment. |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: | 6.2.Y(New) |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | N |  |  |  |  |  |  |  |  |  |
| Other specs | X |  |  | Other core specifications |  |  |  |  | TS/TR TS/TR |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.2.Y	DCAF

The Data Collection Application Function (DCAF) supports the following functionality:

-   Enabling the NWDAF to perform data collection from the Application.

-   Sending analytics results to the UE .

-  collecting UE data from ASP server.

The DCAF functionality is specified in TS 26.531 [X].

* * * Next Changes * * *

6.3.Y      DCAF discovery and selection

Multiple instances of DCAF may be deployed in a network.

The NF consumers shall utilize the NRF and NWDAF to discover DCAF instance(s) unless DCAF information is available by other means, e.g. locally configured on NF consumers. The DCAF selection function in NF consumers selects an DCAF instance based on the available DCAF instances.

The following factors may be considered by the NF consumer for DCAF selection:

-	S-NSSAI.

-	Analytics ID(s).

-	DCAF Serving Area information, i.e. list of TAIs for which the DCAF can provide analytics and/or data.

-	 NF type of the data source.

-	 NF Set ID of the data source.

NOTE:	NF Set ID can be used when the DCAF determines that it needs to discover another DCAF which is responsible for co-ordinating the collection of required data. The DCAF does a new discovery for a target DCAF via NRF using NF Set ID of the data source.

* * * End of Changes * * *