SA WG2 Meeting #155	S2-2302265

Athens, Greece, Feb 20 – 24, 2023	(revision of S2-2301403)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0096 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ECS Address Configuration Information delivery in roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Nokia, Nokia Shanghai Bell, [Huawei, CMCC] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_ph2 |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per conclusion of KI#1 in TR 23.700-62, in the UDM and UDR, the subscription data of EACI is stored per PLMN ID. The V-SMF (for the LBO case) or the H-SMF via the V-SMF (for the HR case) provides the UE with the EACI as indicated in step 11 in figure 4.3.2.2.1-1 of TS 23.502 [9]. For the HR case, the V-SMF does not modify, but just delivers the EACI provided by the H-SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PLMN ID in ECS Address Configuration Information provided by AF, that the 5GC can identify which PLMN the ECAI associated to. Add a new clause 6.5.2.X to describe the ECS Address Configuration Information provisioning in roaming case. Reusing the Parameter Provision procedure that the V-AF provides the ECS Address Configuration Information to V-UDR after the authorised by the UDM located in VPLMN. And the V-SMF retrieves the ECS Address Configuration Information from V-UDR according to the HPLMN ID, and delivers the candidate ECS Address Configuration Information to H-SMF during the PDU session establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE in HR case cannot receive ECS Address Configuration Information, and H-SMF in HPLMN cannot decide in certain roaming PLMN, which ECS Address Configuration Information should be delivered to UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1, 6.5.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

6.5.2.1	ECS Address Configuration Information

The ECS Address Configuration Information (EACI) consists of one or more ECS Configuration Information as defined in clause 8.3.2.1 of TS 23.558 [5]. The ECS Configuration Information may contain Spatial Validity Conditions, which includes one of the following alternatives:

-	a Geographical Service Area (see TS 23.558 [5]);

-	a list of TA(s); or

-	a list of countries (list of MCC); or

-	a list of PLMN IDs (see Table 4.15.6.3d-1 of TS 23.502[x]).

A UE may receive multiple instances of ECS Address Configuration Information e.g., corresponding to different ECSPs (e.g., the MNO or a 3rd party service provider).

The ECS Address Configuration Information is sent to the UE on a per PDU Session basis. The same PDU session can be used by multiple ECS providers.

The SMF does not need to be aware of the internal structure of the ECS Address Configuration Information.

* * * Start of Change 2 * * *

6.5.2.X	ECS Address Provisioning in Roaming

For both the LBO and the HR cases, the subscription data of the ECS Address Configuration Information (EACI) in UDM or UDR is stored per PLMN ID.

For the LBO case, an AF in the visited PLMN (V-AF) may provide the EACI to the UDM via the H-NEF by using the External Parameter Provisioning procedure as described in clause 4.15.6.3d in TS 23.502 [3]. This ECS Address Configuration Information is further provided to the SMF as part of the subscription information.

For the HR-SBO :

-	For the case where the HPLMN knows the EACI in the VPLMN: For this scenario, the AF in the HPLMNmay provide the VPLMN EACI to the H-NEF via External Parameter Provisioning procedure as described in clause 4.15.6.3d in TS 23.502[3], and further to the UDM and to H-SMF as part of the subscription information. During the HR PDU session establishment procedure, the H-SMF sends the VPLMN EACI to the V-SMF and then to the UE as described in section 4.3.2.2 of TS 23.502. The V-SMF does not modify, but just delivers the EACI provided by the H-SMF.

-	For the case where the HPLMN does not know the EACI in VPLMN: For this scenario, the AF in the VPLMN provides the VPLMN EACI via External Parameter Provisioning procedure as described in clause 4.15.6.2 in TS 23.502[3] to the V-NEF that stores this information in V-UDR. The V-SMF retrieves the information from V-NEF, the retrieval is not related with an individual UE but with a DNN and S-NSSAI.. During the HR PDU Session establishment procedure, the V-SMF sends the VPLMN EACI obtained from V-NEF to the H-SMF. The H-SMF decides the VPLMN ECS Address Configuration Information to be sent to the V-SMF and then to UE according to the PLMN ID additionally.

* * * End of Change * * *