3GPP TSG-SA2 Meeting #155	S2-2302363

Feb 20 – 24, 2023, Athens, Greece			                                      (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4129 | Rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on accessing to SNPN via Untrusted non-3GPP access network using credentials owned by a Credential Holder |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | IN TR 23.700-08, the following conclusions are agreed for normative work: - Access to SNPN services via Untrusted non-3GPP access network includes support for Credential Holder, UE Onboarding and Emergency services. However, with the approved N3IWF mechanism in S2-2303070, the UE only tries to select an N3IWF of its subscribed SNPN, regardless the location of the UE. There is no chance for the UE to select an N3IWF of an SNPN other than its subscribed SNPN. As per clarifications in TS 23.501 clause 5.30.2.9: As a deployment option, the SNPN in Figure 5.30.2.9.2-1 can also be another SNPN than the subscribed SNPN for the UE (i.e. none of the SNPN IDs broadcast by NG-RAN matches the SNPN ID corresponding to the subscribed SNPN). With the current N3IWF selection mechanism, the UE cannot access to an SNPN other than the subscribed SNPN using credentials owned by a CH via untrusted non-3GPP access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that UE accessing to an SNPN using credentials owned by a Credentials Holder via Untrusted non-3GPP access network is not supported if the SNPN is not the subscribed SNPN for the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification may cause misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.30.2.12	Access to SNPN services via Untrusted non-3GPP access

Access to SNPN services via Untrusted non-3GPP access network follows the specification in the previous 5.30.2 clauses with the differences as specified in this clause.

N3IWF selection is supported as follows:

-	When UE registers to SNPN with credentials owned by the SNPN, UE uses the same N3IWF selection procedure as specified for access to stand-alone non-public network services via PLMN in clause 6.3.6.2a.

UE accessing to an SNPN using credentials owned by a Credentials Holder via Untrusted non-3GPP access network is supported as per clause 5.30.2.9 with the following difference: The SNPN in Figure 5.30.2.9.2-1 and Figure 5.30.2.9.3-1 is the subscribed SNPN for the UE.

Emergency services are supported as follows:

-	UE initiates N3IWF selection for emergency services when the UE detects a user request for emergency session and determines that Untrusted non-3GPP access is to be used for the emergency access. The UE with SNPN subscription performs the following:

-	If the UE determines that it is located in the same country as the subscribed SNPN, the UE uses the configured N3IWF FQDN for N3IWF selection.

-	Otherwise, the UE follows the N3IWF selection procedure for Emergency services for UE not equipped with valid SNPN credentials, in the same way as defined in clause 6.3.6.4.2 for a UE not equipped with a UICC.

UE onboarding is supported as follows:

-	When UE registers to SNPN over Untrusted non-3GPP access for UE Onboarding, UE may select an N3IWF in the SNPN which supports UE Onboarding by using a pre-configured N3IWF FQDN used for Onboarding.

-	If the PVS is reachable from the local Untrusted non-3GPP access network (e.g. via the Internet) using the local IP connectivity, UE may connect directly (i.e. without being connected to an N3WIF) with a PVS to obtain the SNPN credentials.

END CHANGE