3GPP TSG SA WG2 Meeting #155	S2-2302505

# February 20 - 24, 2023, Athens

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1265 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Describe option to support IMS-MRF registration and discovery using NRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA INC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TR 23.700-87 it has been concluded to include SBI based registration, discovery and selection of IMS-MRF using NRF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds note to SBI architecture in Annex AA to mention IMS-MRF may use NRFdiscovery Adds note to service interface support to indicate Nnrf may be used. Adds new generic clause to address NRF discovery for IMS media functions, and adds text specific for IMS- MRF. Includes clarification on deployments using separate MRFC and MRFP and combined MRF for media processing and control. Describes how MRF uses a IMS Media services list so that not all MRF to allow for deployments where specialized MRF are used, or MRF are used in specialed topology. Clarifies that Enhanced MRF introduced as part of IMS-DC service (also in TR 23.700-87 conclusions) is differentiated via the use of the IMS Media services list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to discover IMS-MRF using NRF method. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.1.1 , AA.1.3 , AA.XX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 3895 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of First Change ****

## AA.1.1	Architectural Support

Figure AA.1.1-1 shows the architecture to support SBA interactions between IMS entities.

Figure AA.1.1-1: System Architecture to support SBA in IMS

Figure AA.1.1-2 shows the architecture using the reference point representation.

Figure AA.1.1-2: System Architecture to support SBA in IMS in reference point representation

NOTE:	Figure AA.1.1.1 and Figure AA.1.1.2 do not show IMS-MRF which may utilize NRF based registration, discovery and selection.

**** Start of Second Change ****

## AA.1.3	Service based interface to support SBA in IMS

Npcf:	Service-based interface exhibited by PCF.

Nhss:	Service-based interface exhibited by an SBI capable HSS.

These SBI services provide equivalent functionality to the Diameter Rx and Cx/Sh reference points.

To support co-existence of IMS nodes supporting SBA services and IMS nodes not supporting SBA services SBI enabled IMS nodes may support both SBI and non-SBI interfaces.

NOTE:	Nnrf to assist in service registration, discovery and selection may be used by other IMS nodes as described in this annex.

**** Start of Third Change ****

## AA.X SBI Discovery and Selection for IMS Media Functions

### AA.X.1 NRF based IMS-MRF Discovery and Selection

#### AA.X.1.1	General

While no SBI procedures are defined for IMS-MRF (and hence IMS-MRFC and IMS-MRFP), it is possible to utilize NRF to discover and select the IMS-MRF to use for an application or service. In networks where IMS-MRFC and IMS-MRFC are co-located it is only necessary to indicate that the service function is IMS-MRF, in networks where discreet IMS-MRFC and IMS-MRFP are deployed it is necessary to indicate which service function is needed – in some cases IMS-MRF, IMS-MRFC, and IMS-MRFP indications may co-exist inside the same network.

When an SBI capable IMS entity selects a MRF it shall use the IMS Media service list to ensure that the selected IMS-MRF supports the required media capabilities for the application.  The SBI capable IMS entity may utilize other available information to assist in selecting the MRF based on other implementation criteria (e.g. proximity of the IMS UE to MRF to minimize transmission delays).

An SBI capable IMS entity always selects an IMS-MRF (or IMS-MRFC or IMS-MRFP) within its own PLMN.

#### AA.X.1.2	IMS-MRF Registration

If an IMS-MRF has been configured to support NRF based discovery or selection it shall register with the NRF using the Nnrf_NFManagement_NFRegister Request message as defined in TS 23.502 [94]. The NF profile of the IMS-MRF registered in NRF includes necessary information for an SBI capable IMS entity interact with the IMS-MRF using non-SBI procedures.  In addition, the NF profile for IMS-MRF includes a text-based list of IMS Media services performed by the IMS-MRF (e.g. video transcoding, voice conferencing, in-call announcements).

NOTE:	The list of Media services is implementation specific and individual entries in the list need to be configured to exactly correspond to the discovery requests made by IMS-AS or other SBA entity.

The MRF registration may include other information pertaining to the implementation (e.g.NF location to support routing media to nearest MRF, MSISDN prefix, or IP address for IP based “least-cost” routing).

#### AA.X.1.3	IMS-MRF Discovery and Selection

During the IMS procedure, an SBI capable IMS entity sends a Nnrf_NFDiscovery_Request to NRF as defined in TS 23.502 [94] to discover applicable MRF instances within a given PLMN.  The SBI capable entity may include a list of the service or services it is requesting to use, in which case the NRF will respond with the IMS-MRF (s) that support all the services listed. The SBI capable IMS entity may store all returned IMS-MRF instances and their NF profiles for subsequent use, including location and IP address (if supplied according to implementation).  If the IMS Media services list is blank the NRF shall return all applicable IMS-MRF according to the other selection parameters included in the request.

NOTE:	The SBI enabled IMS entity can also perform IMS-MRF discovery prior to receiving an IMS request without any including any applicable user, or IMS media service information in the request.

When separate IMS-MRFC and IMS-MRFP are used, and when there is no direct association between an IMS-MRFC and IMS-MRFP, then discovery and selection of the MRFP may be needed after the IMS-MRFC discovery; in such a case the IMS-MRFC may perform SBI discovery of the NRF IMS-MRFP, as if it were an IMS-AS.

#### AA.X.1.4	Enhanced IMS-MRF for Data Channel Services

To support IMS Data Channel services the Enhanced IMS-MRF is defined in clause XX, in regard to the registration, discovery, and selection described in this clause, the enhanced IMS-MRF is a type of service listed in the IMS Media services field.

**** End of Changes ****