3GPP TSG-WG SA2 Meeting #155	S2-2302537

# Feb 20th – Feb 24th, 2023; Athens                   		 	 				      (Revision of S2-2301982)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0637 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Analytics/ML Model Accuracy Monitoring Functional Description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, [Lenovo] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | High level structure of Functional Description of Analytics/ML Model Accuracy Monitoring. The main purpose is to first reach common understanding how to caperture KI#1 in 23.288 and then have a job split among interesting companies. On top of revision1, clarify what is new test data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the conclusion of KI#1: How to improve correctness of NWDAF 5GC" in the clause 8.1 of TR 23.700-81, this contribution mainly aims to add a general clause to describe high level Functional Description of Analytics/ML Model Accuracy Monitoring |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | High level decription of Analytics/ML Model Accuracy Monitoring is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5C(new clause), 5C.1(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5C	Analytics/ML Model Accuracy Monitoring Functional Description

5C.1	General

NWDAF has the accuracy monitoring capability of Analytics and/or ML Model, where NWDAF may provide the accuracy information to consumers when requested or use it for its internal processes.

Input data is collected from Data Producer NF when there is a request for inference/prediction per analytics ID in NWDAF for specific time period in future while ground truth data are collected from Data Producer NF corresponding to analytic ID requested at the time which the prediction refers to.

Analytics/ML Model Accuracy Monitoring is to be achieved by comparing the predictions using the current trained ML model and its corresponding ground truth data i.e. the corresponding true observed events.

The NWDAF (containing AnLF/MTLF) with accuracy monitoring capability decides to initiate analytics accuracy monitoring based on:

-	A request from an analytics accuracy consumer. The analytics accuracy consumer may be an NWDAF containing AnLF, NWDAF containing MTLF or an analytics consumer NF.

-	Analytics Feedback Information which may be provided by an Analytics Consumer NF.

-	Internal trigger

Accuracy Monitoring Functionality are composed by Analtyics Accuracy Monitoring Functionality and ML Model Accuracy Monitoring Functionality. NWDAF containing AnLF supports Analytics Accuracy Monitoring. NWDAF containing MTLF supports ML Model Accuracy Monitoring

AnLF Analytics Accuracy Monitoring Functionality as defined in clause 6.2D is able to provide or notify the accuracy information of Analytics IDs to the analytics consumers of such service and when the analytics accuracy does not meet the analytics consumer's requirements, the analytics consumer may stop using analytics for a period of time or obtain new analytics. In addition, updated analytics for the provided Analytics IDs may be provided to analytics consumers as requested, if the updated analytics is able to be generated within the correction time period.

MTLF ML Model Accuracy Monitoring Functionality as defined in clause 6.2E is able to determine ML Model degradation based on:

- newly collected data including input data, prediction (if available) and the ground truth data from Data Producer NF(s);

- or AnLF providing notifications of the accuracy degradation of the analytics generated by the ML model

NWDAF containing MTLF may reselect a new ML model or retrain the existing ML model and consequently notify the ML model accuracy degradation to the ML Model consumer(s). Optionally, NWDAF containing MTLF may retrain the existing ML model based on newly collected data from Data Producer NF(s).

* * * * End of Changes * * * *