3GPP TSG-WG SA2 Meeting #155	S2-2302548

Athens, Greece, February 20 – 24, 2023	(revision of S2-230xxxx)

Source:	LG Electronics

Title:	Discussion on slice removal indication from SMF to AMF

Document for:	Approval

Agenda Item:	9.11.2 Network Slicing Phase 3

Work Item / Release:	eNS_Ph3 / Rel-18

Abstract of the contribution: This paper discusses whether slice removal indication is needed.

Discussion

This contribution tries to resolve following Editor's note in S2-2301606 (TS 23.501 CR#3939).

Editor's note:	Whether SMF provides slice removal indication to AMF is FFS.

The main purpose of providing slice removal indication to the AMF is to avoid duplicated detection in the AMF since the SMF already detected inactivity. It is possible that the AMF deregisters slice when the AMF notified that the associated PDU Session is released. However, if the SMF triggered PDU Session release due to e.g. SSC mode 2, the UE will re-establish a PDU Session using the same slice. In this case, if the AMF deregisters the slice, the UE needs to perform Registration procedure to register the slice, which causes unnecessary signalling. Alternatively, the AMF can run a deregistration inactivity timer but this causes duplicated detection and delays deregistering the slice.

Above issue is caused mainly based on assumption that the AMF does not know why the SMF triggered PDU Session release. However, according to stage-3 specification (TS 29.502), the SMF provides a cause and it indicates why the PDU Session is released.

Figure 5.2.2.5.1-1: SM context status notification ##### 6.1.6.2.8 Type: SmContextStatusNotification Table 6.1.6.2.8-1: Definition of type SmContextStatusNotification Attribute name; Data type; P; Cardinality; Description; Applicability. statusInfo; StatusInfo; M; 1; This IE shall contain status information about the SM context.. smallDataRateStatus; SmallDataRateStatus; C; 0..1; This IE shall be present, if the NF Service Consumer has indicated support of CIoT and if the status is available. When present, it shall indicate the current small data rate control status for the PDU session.; CIOT. apnRateStatus; ApnRateStatus; C; 0..1; This IE shall be present, if the NF Service Consumer has indicated support of CIoT and if the status is available. When present, it shall indicate the current APN rate control status for the PDN connection (APN rates are shared by all PDN connections of the UE to this APN).; CIOT. ddnFailureStatus; boolean; C; 0..1; This IE shall be present if the DDN Failure shall be reported (see clause 5.2.8.2.8 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: DDN failure detected - false (default): DDN failure is not detected; CIOT. notifyCorrelationIdsForddnFailure; array(string); C; 1..N; This IE shall be present if the DDN Failure shall be reported. When present, it shall contain the notification correlation Id(s) of the DDN failure subscriptions for which a DDN failure has been detected. This parameter can be useful if the NF service consumer has multiple subscriptions for the same PDU session.; CIOT. newIntermediateSmfId; NfInstanceId; C; 0..1; This IE may be present for a PDU session with an I-SMF or V-SMF, if the resourceStatus attribute in statusInfo is set to "UPDATED" and the cause in statusInfo is set to "CHANGED_INTERMEDIATE_SMF". When present, it shall include the NF instance identifier of the new intermediate SMF when it is changed within an SMF set.; ES3XX. newSmfId; NfInstanceId; C; 0..1; This IE may be present if resourceStatus in statusInfo is set to "TRANSFERRED". When present, it shall include: - the new I-SMF instance identifier if the cause in statusInfo is "ISMF_CONTEXT_TRANSFER"; - the new SMF instance identifier if the cause in statusInfo is "SMF_CONTEXT_TRANSFER". This IE may also be present if the resourceStatus attribute in statusInfo is set to "UPDATED". When present, it shall include the NF instance identifier of the new H-SMF or SMF (for a PDU session with an I-SMF) handling the PDU session, when it is changed within an SMF set, if the cause in statusInfo is "CHANGED_ANCHOR_SMF".; CTXTR ES3XX. newSmfSetId; NfSetId; C; 0..1; This IE may be present if resourceStatus in statusInfo is: - TRANSFERRED When present, it shall include: - The new I-SMF set identifier if cause in statusInfo is "ISMF_SERVICE_CONTEXT_TRANSFER"; - The new SMF set identifier if cause in statusInfo is "SMF_SERVICE_CONTEXT_TRANSFER".; CTXTR. oldSmfId; NfInstanceId; C; 0..1; This IE shall be present if resourceStatus in statusInfo is: - TRANSFERRED When present, it shall include: - The old I-SMF instance identifier if cause in statusInfo is "ISMF_CONTEXT_TRANSFER"; - The old SMF instance identifier if cause in statusInfo is "SMF_CONTEXT_TRANSFER".; CTXTR. oldSmContextRef; Uri; C; 0..1; This IE may be present if resourceStatus in statusInfo is: - TRANSFERRED When present, this IE shall include the identifier of the SM Context resource in the old I-SMF or SMF. This IE may also be present if resourceStatus in statusInfo is "UNCHANGED", the SMF selection during PDU Session re-establishment for SSC mode 3 is needed and the runtime coordination between old SMF and AF is enabled. When present, this IE shall contain the URI of the SM Context resource in the old SMF with the structure: {apiRoot}/nsmf-pdusession/v1/sm-contexts/{smContextRef}.; CTXTR EnEDGE. altAnchorSmfUri; Uri; C; 0..1; This IE shall be present if resourceStatus in statusInfo is: - ALT_ANCHOR_SMF When present, it shall contain the API URI of the alternative (H-)SMF towards which the PDU session is established.; AASN. altAnchorSmfId; NfInstanceId; C; 0..1; This IE may be present if resourceStatus in statusInfo is: - ALT_ANCHOR_SMF When present, it shall contain the NF Instance Id of the alternative (H-)SMF towards which the PDU session is established.; AASN. targetDnaiInfo; TargetDnaiInfo; C; 0..1; This IE shall be present if the I-SMF selection or removal for the current PDU session, or SMF selection during PDU Session re-establishment for SSC mode 2/3 is needed, or if it is received from the SMF in Notify Status. When present, this IE shall include the target DNAI Information.; EnEDGE. oldPduSessionRef; Uri; C; 0..1; This IE shall be present if resourceStatus in statusInfo is "UNCHANGED", the SMF selection during PDU Session re-establishment for SSC mode 3 is needed and the runtime coordination between old SMF and AF is enabled. When present, this IE shall contain the URI of the PDU session resource in the old SMF. The URI shall be an absolute URI, including apiRoot (see clause 6.1.3.6.2).; EnEDGE. interPlmnApiRoot; Uri; C; 0..1; This IE should be present if the information has changed and, within the statusInfo IE, the resourceStatus IE is set to "UPDATED" and the cause IE set to "CHANGED_INTERMEDIATE_SMF". When present, it shall contain the apiRoot of the SM context to be used in inter-PLMN signalling request targeting the SM context.. NOTE: If resourceStatus in statusInfo is "TRANSFERRED", at least one of newSmfId and newSmfSetId shall be included.. 6.1.6.2.29 Type: StatusInfo Table 6.1.6.2.29-1: Definition of type StatusInfo Attribute name; Data type; P; Cardinality; Description; Applicability. resourceStatus; ResourceStatus; M; 1; This IE shall indicate the status of the SM context or PDU session resource.. cause; Cause; O; 0..1; When present, this IE shall indicate the cause for the resource status change.. cnAssistedRanPara; CnAssistedRanPara; C; 0..1; This attribute shall be present when the cause value is "CN_ASSISTED_RAN_PARAMETER_TUNING". When present, this IE shall include the SMF derived CN assisted RAN parameters tuning.; CARPT. anType; AccessType; C; 0..1; This IE shall indicate the access type of PDU session.; HOFAIL. …… ##### 6.1.6.3.8 Enumeration: Cause The enumeration Cause indicates a cause information. It shall comply with the provisions defined in table 6.1.6.3.8-1. Table 6.1.6.3.8-1: Enumeration Cause Enumeration value; Description. "REL_DUE_TO_HO"; Release due to Handover. "EPS_FALLBACK"; Mobility due to EPS fallback for IMS voice is on-going.. "REL_DUE_TO_UP_SEC"; Release due to user plane Security requirements that cannot be fulfilled.. "DNN_CONGESTION"; Release due to the DNN based congestion control.. "S_NSSAI_CONGESTION"; Release due to the S-NSSAI based congestion control.. "REL_DUE_TO_REACTIVATION"; Release due to PDU session reactivation.. "5G_AN_NOT_RESPONDING"; The 5G AN did not respond to the request initiated by the network.. "REL_DUE_TO_SLICE_NOT_AVAILABLE"; Release due to the associated S-NSSAI becomes no longer available.. "REL_DUE_TO_DUPLICATE_SESSION_ID"; Release due to a UE request to establish a new PDU session with an identical PDU session Id.. "PDU_SESSION_STATUS_MISMATCH"; Release due to mismatch of PDU Session status between UE and AMF.. "HO_FAILURE"; Handover preparation failure. "INSUFFICIENT_UP_RESOURCES"; Failure to activate the User Plane connection of a PDU session due to insufficient user plane resources.. "PDU_SESSION_HANDED_OVER"; The PDU session is handed over to another system or access.. "PDU_SESSION_RESUMED"; Resume the user plane connection of the PDU session.. "CN_ASSISTED_RAN_PARAMETER_TUNING"; SMF derived CN assisted RAN parameters tuning.. "ISMF_ CONTEXT_TRANSFER"; The PDU session shall be transferred from old I-SMF to new I-SMF.. "SMF_ CONTEXT_TRANSFER"; The PDU session shall be transferred from old SMF to new SMF.. "REL_DUE_TO_PS_TO_CS_HO"; Release due to 5G SRVCC from NG-RAN to 3GPP UTRAN, as specified in clause 6.5.4 of 3GPP TS 23.216 [35].. "REL_DUE_TO_SUBSCRIPTION_CHANGE"; PDU session release due to UE subscription changes, triggered by the SMF e.g. due to the removal of subscribed DNNs, or by the AMF e.g. due to ODB changes.. "HO_CANCEL"; Handover cancellation. "REL_DUE_TO_SLICE_NOT_AUTHORIZED"; Release due to Network Slice-Specific Authentication and Authorization failure or revocation.. "PDU_SESSION_HAND_OVER_FAILURE"; Failure to handover PDU session to another access. "DDN_FAILURE_STATUS"; DDN failure status reporting. "REL_DUE_TO_CP_ONLY_NOT_APPLICABLE"; Release due to Control Plane Only indication associated with PDU Session is not applicable any longer. "NOT_SUPPORTED_WITH_ISMF"; PDU session release due to a requested functionality that is not supported for a PDU session with an I-SMF/V-SMF.. "CHANGED_ANCHOR_SMF"; The anchor SMF of the PDU session is changed.. "CHANGED_INTERMEDIATE_SMF"; The intermediate SMF (e.g. I-SMF or V-SMF) is changed.. "TARGET_DNAI_NOTIFICATION"; Notify the target DNAI for I-SMF selection for the current PDU Session, or SMF selection during PDU Session re-establishment for SSC mode 2/3.. "REL_DUE_TO_VPLMN_QOS_FAILURE"; Release due to QoS not complying with VPLMN QoS constraints, i.e. VPLMN QoS constraints are required for the PDU session and the H-SMF provides QoS parameters not complying with VPLMN QoS required by the V-SMF.. "REL_DUE_TO_SMF_NOT_SUPPORT_PSETR"; Release the PDU session due to the (H-)SMF does not support the PSETR feature when the V/I-SMF has failed.. "REL_DUE_TO_SNPN_SNPN_MOBILITY"; Release due to the PDU session is rejected by the new AMF during registration procedure as the continuity of the PDU Session cannot be supported between networks with SNPN-SNPN mobility.. "REL_DUE_TO_NO_HR_AGREEMENT"; Release due to the PDU session is rejected by the new AMF during registration procedure as the continuity of the PDU Session cannot be supported between networks with inter-PLMN mobility where no HR agreement exists.. "REL_DUE_TO_UNSPECIFIED_REASON"; Release due to unspecified reasons..

Observation 1: According to stage-3 specification, the AMF knows why the SMF triggered PDU Session release.

Based on the SMF provided cause and local policy, the AMF can determine whether the slice needs to be deregistered immediately or needs to run a deregistration inactivity timer. Therefore, there is no need to provide separate indication to the AMF. We proposes to remove the slice removal indication.

Observation 2: The AMF can determine whether to trigger slice deregistration or to run a deregistration inactivity timer based on local policy and existing cause provided by the SMF.

Proposal: It is proposed to remove the slice removal indication.

Summary

Observation 1: According to stage-3 specification, the AMF knows why the SMF triggered PDU Session release.

Observation 2: The AMF can determine whether to trigger slice deregistration or to run a deregistration inactivity timer based on local policy and existing cause provided by the SMF.

Proposal: It is proposed to remove the slice removal indication.

Proposal

### 5.15.X	Support of network slice usage control

#### 5.15.x.1	General

Network slice usage control is achieved as follows:

1)	Configuring network-controlled Slice Usage Policy to supporting UEs (see caluse 5.15.x.2)

2)	Configuring PDU Sessions inactivity timers, and network slice deregistration inactivity timers (see clause 5.15.x.3)

5.15.x.2	UE Configuration of network-controlled Slice Usage Policy

The UE during the Registration procedure may indicate in UE MM Core Network Capability that it supports UE configuration of network-controlled Slice Usage Policy. If so, the AMF determines Slice Usage Policy for a network slice for the UE and may configure the UE with this information together with Configured NSSAI to control the usage of this network slice. The AMF may be locally configured with network Slice Usage Policy or receive the policy from the (AM-)PCF.

The network-controlled Slice Usage Policy is provided to the UE in the Registration Accept or the UE Configuration Update Command and may include:

-	An indication, for one or more of S-NSSAI(s) of the HPLMN in the Configured NSSAI, whether the UE needs to register the network slice with the network when applications in the UE require data transmission in the network slice (i.e. the UE needs to register the network slice only on demand).

NOTE:	All Other network slices in the Configured NSSAI are handled by the UE using UE specific policies (e.g. they may be registered irrespective of applications need)

-	For all on demand S-NSSAI(s) of the HPLMN in the Configured NSSAI, a deregistration inactivity timer that causes the UE to deregister the network slice after the last PDU Session associated with the S-NSSAI is released. This deregistration inactivity timer is started at the UE and AMF per access type when the last PDU Session associated with the S-NSSAI is released, or the network slice is included in the Allowed NSSAI and no PDU session is established. The deregistration inactivity timer is stopped and reset when the first PDU session is established or the S-NSSAI is removed from the Allowed NSSAI. The AMF and UE may locally remove the S-NSSAI from the Allowed NSSAI when the timer expires. The AMF may also send a UE Configuration Update Command to remove the slice from the Allowed NSSAI.

If the UE and network state became misaligned, the UE may, for example, request connectivity in a network slice which is no longer allowed. In this case, the AMF shall provide the updated Allowed NSSAI in a UE Configuration Update Command after rejecting the PDU Session establishment. The UE may then re-register with the network slice if needed.

5.15.x.3	Network-based per UE network slice usage behaviour control

The 5GC performs usage monitoring to be able to enforce the release of inactive PDU Sessions, and deregistering of UEs from network slices with no PDU Sessions on them according to its own policies. In order to support usage monitoring for a network slice:

-	the AMF runs a slice deregistration inactivity timer per S-NSSAI and access type to deregister the network slice which is started when the network slice is not used by any PDU Session over the corresponding access type. The deregistration inactivity timer is stopped and reset when at least a PDU Session associated with the network slice is successfully established or the network slice is removed form the Allowed NSSAI. When the deregistration inactivity timer for a network slice over an access type expires, the AMF removes the network slice from the Allowed NSSAI over the access type by sending the UE Configuration Update Command to impacted UE(s).

-	the SMFs provide to UPFs that handle the PDU sessions in the network slice a PDU Session inactivity timer. The PDU Session inactivity timer is started after no data packet is transmitted or received and runs until the next data packet is transmitted or received which restarts the timer again. If the PDU Session inactivity timer expires before any packet is received or transmitted, the UPF reports this PDU Session inactivity event to the SMF to cause the SMF to release the PDU Session. When the AMF receives the notification of PDU Session release and if the network slice of the released PDU Session is not used by other PDU Sessions (i.e. the last PDU Session using the network slice is released) over the corresponding access type, based on local configuraiton, the AMF may trigger the UE Configuration Update procedure to remove the network slice from the Allowed NSSAI over that corresponding access type.

Editor's note:	Roaming aspects are FFS.

The deregistration inactivity and PDU Session inactivity timers are either pre-configured in the AMF/SMF or received from the PCF/UDM. Only if a S-NSSAI is dedicated for a single AF, the AF use external parameter provisioning procedure to provide deregistration inactivity and PDU session inactivity timers as described in clause 4.15.6.2 of TS 23.502 [3].

Editor's note:	It is FFS whether the timer is stored at the UDM or PCF.

To enable a serving network to direct UEs to a preferred network slice, the AMF may request the UE to transfer a PDU Session from one S-NSSAI to another S-NSSAI as described in clause 5.15.Y.