3GPP TSG-WG SA2 Meeting #155 	S2-2302590

# Athen, 20-24 February 2023	(revision of S2-2300826)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3795 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Group QoS request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, SK Telecom, LG Uplus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduce the support of QoS request for AF sessions for a list of UEs and Group-MBR Monitoring as concluded in TR 23.700-80. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of QoS request for AF sessions for a list of UEs and Group-MBR Monitoring |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of support of this feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.6x (new), 4.15.6.6y (new), 5.2.13.2.4, 5.2.6.1, 5.2.6.X (new), 5.2.6.X.1 (new), 5.2.6.X.2 (new), 5.2.6.X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (ALL NEW TEXT)

4.15.6.6x	Setting up AF sessions with required QoS procedure

Figure 4.15.6.6x-1: Setting up AF sessions with required QoS procedure

1.	The AF sends a request to reserve resources for AF sessions for a list of UEs using Nnef_AFsessionWithGroupQoS_Create request message (a list of UE address, AF Identifier, Flow description information or External Application Identifier, QoS Reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), QoS parameter(s) to be measured, Reporting frequency, Target of reporting, optional an indication of local event notification, Group-MBR monitoring information, DNN, S-NSSAI) to the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as described in clause 6.1.3.22 of TS 23.503 [20]. The AF may provide QoS parameter(s) to be measured as defined in clause 5.X of TS 23.501 [2], Reporting frequency, Target of reporting, optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20]. The AF may provide the Group-MBR monitoring information which includes the Event ID for the Group-MBR monitoring event, flow direction, Reporting frequency, Reporting Threshold. The AF shall not include both the Group-MBR monitoring information and the UL bitrate or DL bitrate as defined in 5.X of TS 23.501 [2] in the QoS parameter(s) to be measured.

2.	The NEF authorizes the AF request and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all steps (except step 5) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed. If the AF request from step 1 contains the Group-MBR monitoring information, the NEF generates the Group-MBR Monitoring subscription based on the information received from the AF and generates the Notification Correlation ID for the Group-MBR Monitoring.

3.	The NEF uses the list of UE address to discover the serving PCFs from the BSF(s).

4.  If a single PCF is discovered, BSF returns the PCF address to the NEF. If more than one PCFs are discovered, the BSF returns the PCF addresses for the list of UE address to the NEF.

5.  Step 5-10 apply for each UE address in the list of UE address. The NEF provides the UE address and the received parameters in step 1 to the PCF in the Npcf_PolicyAuthorization_Create request. If the AF provided the Group-MBR monitoring information, the NEF sends the Npcf_PolicyAuthorization_Create request message with  the Target of reporting set to the NEF and the other information based on the Group-MBR monitoring information.

6.	Steps 4 in Figure 4.15.6.6-1 applies. The PCF generates authorized QoS Monitoring policy according to the information provided from the NEF in step 5 and provides PCC rules with the policy to the SMF as described in clause 6.1.3.21 of TS 23.503 [20]. The SMF configures the UPF to perform QoS Monitoring as described in clause 5.X of TS 23.501 [2].

7.	If the NEF receives the Npcf_PolicyAuthorization_Create response, the NEF sends a Nnef_AFsessionWithGroupQoS_Create response message (Transaction Reference ID, UE address, Result) to the AF. Result indicates whether the request is granted or not for the UE.

8.	Steps 6 in Figure 4.15.6.6-1 applies.

9.	(indirect notification from the UPF) Steps 7 in Figure 4.15.6.6-1 applies.

9a. (direct notification from the UPF) If the UPF receives Target of the reporting set to the NEF and Indication of direct event notification in step 6, the UPF sends the report to the NEF as described in clause 5.X of TS 23.501 [2].

10. If the NEF receives the Npcf_PolicyAuthorization_Notify, the NEF sends Nnef_AFsessionWithGroupQoS_Notify message with the events reported by the PCF for the UE address to the AF. If the NEF receives the Nupf_EventExposure_Notify, the NEF sends Nnef_AFsessionWithGroupQoS_Notify message with the events reported by the UPF for the UE address to the AF.

11. If the NEF has subscription for the Group-MBR Monitoring, the NEF aggregates the bitrate received from step 9a. If the aggregated bit rate exceeds the Reporting Threshold for the Group-MBR Monitoring, the NEF sends the Nnef_AFsessionWithGroupQoS_Notify to the AF.

The AF may send Nnef_AFsessionWithGroupQoS_Revoke request to the NEF in order to revoke the AF request. The NEF authorizes the revoke request and triggers the Npcf_PolicyAuthorization_Delete and the Npcf_PolicyAuthorization_Unsubscribe operations for the AF request.

Next CHANGE

4.15.6.6y	AF sessions with required QoS update procedure

Figure 4.15.6.6y-1: AF sessions with required QoS update procedure

1.	For established AF sessions with required QoS for the list of UE addresses, the AF may send a Nnef_AFsessionWithQoS_Update request message (AF Identifier, Transaction Reference ID, [a list of UE address], [Flow description information], [QoS Reference or individual QoS parameters], [Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20])], QoS parameter(s) to be measured, Reporting frequency, Target of reporting, optional an indication of local event notification, Group-MBR monitoring information) to NEF for updating the reserved resources. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as specified in clause 6.1.3.22 of TS 23.503 [20]. The AF may provide QoS parameter(s) to be measured as defined in clause 5.X of TS 23.501 [2], Reporting frequency, Target of reporting, optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20]. The AF may provide the Group-MBR monitoring information which includes the Event ID for the Group-MBR monitoring event, flow direction, Reporting frequency, Reporting Threshold. The AF shall not include both the Group-MBR monitoring information and the UL bitrate or DL bitrate as defined in 5.X of TS 23.501 [2] in the QoS parameter(s) to be measured.

2.	The NEF authorizes the AF request of updating AF session with required QoS and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all steps (except step 5) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed. If the NEF received the Group-MBR monitoring information in step 1, the NEF updates the subscription for the Group-MBR monitoring accordingly.

3.	Step 3-7 apply for each UE address in the list of UE address. The NEF uses the list of UE address associated with the Transaction Reference ID to discover the serving PCFs from the BSF(s), if needed. For each UE address in the list of UE addresses, the NEF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Update request and forwards received parameters to the PCF.

4.	Step 4 in Figure 4.15.6.6a-1 applies.

5.	If the NEF receives the Npcf_PolicyAuthorization_Update response, the NEF sends a Nnef_AFsessionWithGroupQoS_Update response message (Transaction Reference ID, UE address, Result) to the AF. Result indicates whether the request is granted or not for the UE address.

6.	Step 6 in Figure 4.15.6.6a-1 applies

7.	The NEF sends Nnef_AFsessionWithGroupQoS_Notify message with the events reported by the PCFs for the UE address to the AF.

8.	If the NEF has subscription for the Group-MBR Monitoring, the NEF aggregates the bitrate received from step 7. If the aggregated bit rate exceeds the Reporting Threshold for the Group-MBR Monitoring, the NEF sends the Nnef_AFsessionWithGroupQoS_Notify to the AF.

Next CHANGE

5.2.13.2.4	Nbsf_Management_Discovery service operation

Service Operation name: Nbsf_Management discovery

Description: Discovers the PCF and PCF set selected for a PDU Session identified by the tuple (UE address(es), SUPI, GPSI, DNN, S-NSSAI), or discovers the PCF and PCF set selected for the UE identified by the tuple (SUPI, GPSI). This operation may also be used to determine the SUPI from the tuple (UE address, DNN, S-NSSAI).

Inputs, Required: UE address (i.e. IP address or MAC address)[Required, for a PDU Session and for a UE], a list of UE address [Conditional, for multiple UEs], DNN [Conditional], S-NSSAI [Conditional], if the target PCF is for a PDU Session, MBS session ID as defined in TS 23.247 [78], [Required, for an MBS Session].

SUPI and/or GPSI, if the target PCF is for a UE.

NOTE:	For support of time sensitive communication and time synchronization (as described in clause 5.28.3.2 of TS 23.501 [2]) the UE address contains the DS-TT port MAC address for Ethernet type PDU Session.

Inputs, Optional: If the target PCF is for a PDU Session, SUPI, GPSI.

Outputs, Required: PCF address(es), PCF address(es) and the list of UE address for each PCF address [Conditional], PCF instance ID [Conditional, if available] and PCF Set ID [Conditional, if available], level of Binding [Conditional, if available] (see clause 6.3.1.0 of TS 23.501 [2]).

Outputs, Optional: SUPI, if available.

Next CHANGE

5.2.6	NEF Services

5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_EASDeployment | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_AFsessionWithGroupQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TimeSynchronization | ConfigUpdate | Request/Response | AF |
|  | ConfigCreate | Request/Response | AF |
|  | ConfigDelete | Request/Response | AF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF |
|  | CapsSubscribe | Subscribe/Notify | AF |
|  | CapsUnsubscribe | Subscribe/Notify | AF |
|  | CapsNotify | Subscribe/Notify | AF |
| Nnef_ASTI | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_AMPolicyAuthorization | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | Subscribe |  | AF |
|  | Unsubscribe |  | AF |
| Nnef_AMInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_UEId | Get | Request/Response | AF |
| Nnef_SMService | MoForwardSm | Request/Response | SMS-SC |
| Nnef_PDTQPolicyNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |

Next CHANGE

5.2.6.X	Nnef_AFsessionWithGroupQoS service

5.2.6.X.1	General

This service is used to request a specific QoS for AF sessions for a group of UEs (See clause 4.15.6.6x).

This service is also used to support subscription and notification of QoS Monitoring for URLLC, as described in clause 5.33.3.2 of TS 23.501 [2] for more than one UEs.

5.2.6.X.2	Nnef_AFsessionWithGroupQoS_Create service operation

Service operation name: Nnef_AFsessionWithGroupQoS Create

Description: The consumer requests the network to provide a specific QoS for AF sessions for a list of UEs.

Inputs, Required: AF Identifier, a list of UE address (i.e. IP address or MAC address), Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], Group-MBR Monitoring information.

Inputs, Optional: time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured as defined in clause 5.X of TS 23.501 [2], Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], DNN if available, S-NSSAI if available.

Outputs, Required: Transaction Reference ID, UE address, Result

Output (optional): None.

5.2.6.X.3	Nnef_AFsessionWithGroupQoS_Notify service operation

Service operation name: Nnef_AFsessionWithGroupQoS Notify

Description: NEF reports the QoS Flow level event(s) or the Group-MBR Monitoring event to the consumer.

Inputs, Required: Reports of the events as defined in clause 6.1.3.18 of TS 23.503 [20]

Inputs, Optional: When the event report is for QoS Monitoring, includes QoS Monitoring report for the QoS parameter(s) to be measured defined in clause 5.X of TS 23.501 [2], e.g. UL packet delay, DL packet delay, or round trip packet delay of the single UP path or two UP paths in the case of redundant transmission, as defined in clause 5.33.3.2 of TS 23.501 [2]. When the event report is for Group-MBR Monitoring, includes Group-MBR Monitoring report as defined in

Outputs, Required: None.

Output (optional): None.

5.2.6.X.4	Nnef_AFsessionWithGroupQoS_Revoke service operation

Service operation name: Nnef_AFsessionWithGroupQoS_Revoke

Description: The consumer requests the network to revoke the AF sessions requested by with a requested QoS or the AF sessions with requested QoS including Alternative Service Requirements.

Inputs, Required: Transaction Reference ID

Inputs, Optional: A list of UE address

Outputs, Required: Transaction Reference ID, Result

Output (optional): None.

5.2.6.X.3	Nnef_AFsessionWithGroupQoS_Update service operation

Service operation name: Nnef_AFsessionWithGroupQoS Update

Description: The consumer requests the network to update the Service Requirement(s) and/or additional Alternative Service Requirement(s) for AF sessions for a list of UEs.

Inputs, Required: Transaction Reference ID

Inputs, Optional: a list of UE address, Flow description information (as described in clause 6.1.3.6 of TS 23.503 [20]), QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], Group-MBR Monitoring information

Outputs, Required: Result

Output (optional): None.

END of CHANGES