3GPP TSG-WG SA2 Meeting #155	S2-2302662

# Athens, Greece, February 20 – 24, 2023		(revision of S2-22xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0302 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for MT-LR Ranging and sidelink positioning service exposure to Application server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure for Ranging/SL Positioning service exposure to AF is necessary and shall be specified in the normative phase, based on the conclusion of KI#7 in TR 23.700-86. From the perspective of “functional description and information flows”, it adds the MT-LR Procedure with Sidelink Positioning for relative positioning as one basic procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the MT-LR Procedure with Sidelink Positioning for relative positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MT-LR Procedure with Sidelink Positioning for relative positioning is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new) * * * *

6.1.X	MT-LR Procedure with Sidelink Positioning for relative positioning

3.Nudm_UECM_Get

NG-RAN

AMF

LMF

GMLC

UDM

NEF

AF

UE2

UE1

5. NW Triggered Service Request

1a. Nnef_EventExposure_Subscribe

4. Namf_ Ranging _ProvidePositioningInfo Request

6. LMF Selection

7. Nlmf_ Ranging _DetermineLocation Request

12. Nlmf_ Ranging _DetermineLocation Response

8. Ranging/SL Positioning request

10. Ranging/SL Positioning response

13. Namf_Ranging_ProvidePositioningInfo Response

15. Nnef_EventExposure_Notify

11. SL ranging calculation

1b. Ngmlc_Ranging_ProvideLocation Request

2.Nudm_SDM_Get

14. Ngmlc_ Ranging _ProvideLocation Response

9. Ranging with other UEs

Figure 6.1.X-1: MT-LR Procedure with Sidelink Positioning for relative positioning

1. 	The AF requests the Ranging/SL Positioning service between UE1 and UE2 toward the GMLC via the NEF by invoking Nnef_EventExposure_Subscribe request message. The parameters for Ranging/SL positioning service requirement include Reference UE ID (e.g. UE1), Target UE ID (e.g. UE2), Ranging/SL Positioning service type, Requested Quality, etc. The Ranging/SL Positioning service type indicates the distance between two UEs or more UEs and/or the direction and/or relative positioning of one UE (i.e.Target UE) from another UE (i.e. Reference UE) via PC5 interface. The Requested Quality may include delay, accuracy, etc.

2. 	The GMLC invokes a Nudm_SDM_Get service operation towards the UDM(s) of the UE1 and UE2 to get the privacy settings of the UE identified by its GPSI or SUPI and make the authorization. If the authorization fails, steps 3-14 are skipped.

3.	The GMLC determines the initiator UE (e.g. UE1) from the UE1 and UE2, which is used to perform signalling exchange with the LMF. Then the GMLC invokes a Nudm_UECM_Get service operation towards the UDM of the initiator UE with SUPI. The UDM returns the network addresses of the current serving AMF.

4.	The GMLC invokes the Namf_Ranging_ProvidePositioningInfo Rquest service operation towards the AMF to request the Ranging/SL Positioning information of the two UEs.

5.	If the Reference UE is in CM-IDLE state, the AMF initiates the NW triggered service request procedure as defined in TS 23.502 [3].

6.	The AMF selects an appropriated LMF considering the Ranging/SL positioning capability.

7.	The AMF invokes the Nlmf_ Ranging _DetermineLocation Request to the LMF.

8.	The LMF determines the LMF or UE to perform result calculation and sends the Ranging/SL Positioning request to the UE1.

9.	The UE1 performs the Ranging/SL Positioning operation with the UE2.

10.	The UE1 reports the Ranging/SL Positioning measurements or result to the LMF by the Ranging/SL Positioning response message.

11.	The LMF makes calculation based on the Ranging/SL Positioning measurements.

12.	The LMF returns the result via Nlmf_ Ranging _DetermineLocation Response towards the AMF.

13.	The AMF returns the result via Namf_ Ranging _Provide positioningInfo Reponse towards the GMLC.

14.	The GMLC returns the result via Ngmlc_ Ranging _ProvideLocation Response toward the NEF.

15.	The NEF returns the result via Nnef_EventExposure_Notify toward the AF.

# * * * * End of changes * * * *