3GPP TSG-WG SA2 Meeting #155	S2-2302682

# Athens, Greece, February 20 – 24, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0189 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update functional entity clauses for group message delivery and power saving functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2 #154AHE meeting, the affected functional entities for UE receiving MBS data in RRC_INACTIVE state, and for efficient radio resource utilization for multiple broadcast MBS Sessions via multiple CNs to deliver the same broadcast content in the case of network sharing (i.e., MOCN), are documented in clause 5.3.2 in S2-2301592 and S2-2301453. However, in case of group message delivery and UEs using power saving functions, such affected aspects are not reflected in clause 5.2.3 accordingly (Although their general descriptions and procedures were agreed in #154AHE as well). For the sake of not missing such functions, it is proposed to correct the description in clause 5.3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the clause of functional entities, adding the description for group message delivery in clause 5.3.2.9, 5.3.2.10, 5.3.2.11, 5.3.2.12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing the features of group message delivery and UE using powering saving functions in TS 23.247. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.7, 5.3.2.8, 5.3.2.9, 5.3.2.10, 5.3.2.11, 5.3.2.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | New clause 6.X referenced in this CR is introduced in CR0164 New clause 7.2.X and 7.3.X referenced in this CR is introduced in CR0162 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.3.2.7	NG-RAN

In addition to the functions defined in TS 23.501 [5], the NG-RAN performs the following functions to support MBS:

-	Management of MBS QoS flows via N2.

-	Delivery of MBS data packets for multiple UEs over radio using PTM or PTP.

-	Configuration of UE for MBS QoS flow reception at AS layer.

-	Control switching between PTM and PTP delivery per UE.

-	Support for multicast session service continuity during Xn Handover and N2 Handover.

-	Support group paging at multicast session activation over radio toward UEs in CM-IDLE state and CM-CONNECTED with RRC INACTIVE state.

-	Reception of MBS data packets from 5GC via shared MBS traffic delivery.

-	Support the functions and procedures to enable a UE using power saving functions to receive MBS service, as defined in clause 6.X, 7.2.X and 7.3.X.

Editor's Note: The details for for the NG-RAN impact will be confirmed with RAN WGs.

# * * * * Second change * * * *

5.3.2.8	UE

In addition to the functions defined in TS 23.501 [5], the UE may perform the following functions to support MBS:

-	Reception of multicast data using PTM/PTP.

-	Reception of broadcast data using PTM.

-	Handling of incoming MBS QoS flows.

-	Support of signalling for joining and leaving a Multicast MBS session.

-	MBS resource management support at AS layer.

-	Reception of notification in CM-IDLE state and CM-CONNECTED with RRC _INACTIVE state for multicast data transmission.

-	Support the functions and procedures to enable a UE using power saving functions to receive MBS service, as defined in clause 6.X, 7.2.X and 7.3.X.

NOTE:	UE functionality for MBS security is provided in TS 33.501 [20].

# * * * * Third change * * * *

5.3.2.9	AF

The AF performs the following functions to support MBS:

-	Requesting multicast or broadcast service from the 5GC by providing service information including QoS requirement to 5GC.

-	Instructing MBS session operation towards 5GC if needed.

-	Interacting with NEF for MBS related service exposure.

-	Support for group message delivery as defined in clause 6.15 and 7.5, e.g., interacting with NEF, sending group message to UE.

-	Support the functions and procedures to enable a UE using power saving functions to receive MBS service, as defined in clause 6.X, 7.2.X and 7.3.X.

# * * * * fourth change * * * *

5.3.2.10	NEF

In addition to the functions defined in TS 23.501 [5], the NEF performs the following functions to support MBS:

-	Providing an interface to AFs for MBS procedures including service provisioning, MBS session and QoS management.

-	Interacting with AF and NFs in 5GC, e.g. MB-SMF for MBS session operations, determination of transport parameters.

-	Selection of MB-SMF to serve an MBS Session.

-	Support for group message delivery as mentioned in clause 6.15 and 7.5, e.g., control plane interaction with AF/MBSF, transforming group message payload, handling user plane traffic for MBSTF.

# * * * * Fifth change * * * *

5.3.2.11	MBSF

The MBSF performs the following functions to support MBS:

-	Service level functionality to support MBS, and interworking with LTE MBMS

-	Interacting with AF and MB-SMF for MBS session operations, determination of transport parameters, and session transport.

-	Selection of MB-SMF to serve an MBS Session.

-	Controlling MBSTF if the MBSTF is used.

-	Determination of destination IP multicast address for the MBS session if IP multicast address is sourced by MBSTF.

-	Support for group message delivery as mentioned in clause 6.15 and 7.5, e.g., interacting with NEF, configuring MBSTF.

NOTE 1:	MBSF functionality related to service and MBS data handling (e.g. encoding) is to be determined with SA WG4.

NOTE 2:	MBSF functionality for MBS security is provided in TS 33.501 [20].

# * * * * Sixth change * * * *

5.3.2.12	MBSTF

The MBSTF performs the following functions to support MBS if deployed:

-	Media anchor for MBS data traffic if needed.

-	Sourcing of IP Multicast if needed.

-	Generic packet transport functionalities available to any IP multicast enabled application such as framing, multiple flows, packet FEC (encoding).

-	Multicast/broadcast delivery of input files as objects or object flows.

-	Support for group message delivery as mentioned in clause 6.15 and 7.5, e.g., interacting with MSBF, handling user plane traffic from NEF.

NOTE 1:	MBSTF functionality related to MBS data handling (e.g. encoding) is to be determined with SA WG4.

NOTE 2:	MBSTF functionality for MBS security is provided in TS 33.501 [20].

# * * * * End of changes * * * *