SA WG2 Meeting #S2-155	S2-2302723

20 - 24 February, 2023, Athens, Greece	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4152 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 clarification on the UE registration area and UDM check (option 4) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the SA2#154 meeting, the CR#3767 (S2-2211308) is approved for the KI#2. But the UDM check part is missing which is in the TR conclusion and approved 23.502 CR. In the SA2#154AHE meeting, the controversial issue is the EN “How to ensure that the Time Synchronization Coverage Area and the Registration Area consist of the same is FFS”. According to the proposal in the discussion paper S2-230xxxx, this CR reflect the optoin 4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the UDM check for the covrage area filter. there is no need to adjust RA based on the AF requested geographical area for time sync service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UDM check part for KI#2 is missing. The EN about RA and time sync coverage area is not resolved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.10, 5.27.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.27.1.10	Support for coverage area filters for time synchronization service

This feature enables the AF to request time synchronization service for a UE or a group of UEs in a specific geographical area (so called coverage area). The requested coverage area contains a list of Tracking Area (TA) or a geographical area (e.g. a civic address or shapes that the NEF transforms to list of TAs based on pre-configuration).

TSCTSF checks with the UDM if the UE is allowed to receive the time sync service requested by AF.

The spatial validity condition is resolved at the TSCTSF. In order to do that, the TSCTSF discovers the AMF(s) serving the list of TA(s) that comprise the spatial validity using the NRF and subscribes to the discovered AMF(s) to receive notifications about presence of the UE in an Area of Interest events (as described in clause 5.3.4.4 of TS 23.501 [2]). An Area of Interest (AoI) for each AMF is represented by a list of TA(s), wherein the Area of Interest is identical to the requested coverage area, or the Area of Interest is a TA.

Based on the outcome provided by the AMF about the UE's presence in the AoI, the TSCTSF determines if the time synchronization service is activated or deactivated.

For access stratum distribution activation/deactivation, the TSCTSF will enable/disable access stratum time distribution to the UE at the serving NG-RAN node reusing the procedures in clause 4.15.9.4 of TS 23.502 [3]. For (g)PTP distribution activation/deactivation, the TSCTSF will modify the PTP instance configuration by means of sending a PMIC to the impacted UE/DS-TTs and UMIC to the impacted UPF/NW-TT, as described in clause K.2.2 of TS 23.501 [2].

If the time synchronization service is modified based on the reports of UE presence in the Area of Interest, the TSCTSF informs the AF for the impacted UE(s) by indicating the PTP port state for the related DS-TT PTP port (in the case of (g)PTP based time distribution) or notifying the AF with the indication of 5G access status time distribution status (enabled or disabled, for ASTI based time distribution).

Time Synchronization coverage area shall include at least one TA, i.e., geographical area, its granularity that AF may request for a time synchronization service is restricted to a TA level.

Next CHANGE

#### 5.27.1.11	Controlling time synchronization service based on the Subscription

The distribution of timing information, 5G access stratum-based time distribution and (g)PTP-based time distribution, for a UE may be controlled based on subscription data stored in the UDM. The (g)PTP-based or 5G access stratum-based time synchronization service may be provided to a UE based on the UE's subscription which is specified in the TS 23.502 [3] clause 5.2.3.3.1.

The Access and Mobility Subscription data include for the control of 5G access stratum-based time distribution the following information:

-	the Access Stratum Time Synchronization Service Authorization, which indicates whether the UE should be provisioned with 5G system internal clock timing information over access stratum as specified in TS 38.331 [28].

-	optionally, the Uu time synchronization error budget.

-	optionally, one or more periods of start and stop times defining the times when the UE should be provisioned with 5G system internal clock timing information.

-	optionally, a Time Synchronization Coverage Area comprising a list of TAs where the UE shall be provisioned with 5G system internal clock timing information.

During the Registration procedure, the AMF retrieves the subscription from UDM. If the AMF receives 5G access stratum-based time synchronization service subscription for the given UE, the AMF controls the 5G access stratum-based time distribution:

-	If the 5G access stratum-based time synchronization service is allowed for the UE, the AMF provides the 5G access stratum time distribution indication to the NG-RAN so that it can provide 5G timing information to the UE.

-	The AMF may provide a Uu time synchronization error budget to the NG-RAN (as described in clause 5.27.1.9). If the UE's subscription contains a Uu time synchronization error budget, then AMF sends it to NG-RAN. Otherwise, the AMF uses the pre-configured Uu time synchronization error budget and sends it to NG-RAN.

-	If the UE's subscription contains Coverage Area (defined as a list of TAs), the AMF configures the NG-RAN to provide the 5G timing information to UE only when the UE is in the Coverage Area.

-	If the AMF receives the start and stop times, then the AMF enables and disables the 5G access stratum time distribution indication to the NG-RAN according to the expiry of start and stop times if the UE is in CM_Connected state. If the UE is in CM_Idle state when a Start time condition is met, the AMF pages the UE and provides the 5G access stratum time distribution indication to NG-RAN as part of the subsequent service request procedure initiated by the UE in the response to the paging.

-	The TSCTSF retrieves the Time Synchronization Subscription data from the UDM when the TSCTSF receives an AF request for 5G access stratum-based time synchronization service. According to the "AF request Authorization" in the Time Synchronization Subscription data, the TSCTSF determines whether the UE is authorized for an AF-requested time synchronization service. If the UE is authorized, the TSCTSF proceeds as specified in clause 5.27.1.8 and TS 23.502 [3].

The Time Synchronization Subscription data is the subscription data for the control of (g)PTP-based time distribution and 5G access stratum-based time distribution and includes the following information:

-	the "AF request Authorization", indicating whether the UE is authorized for an AF-requested 5G access stratum-based time distribution and (g)PTP-based time distribution services. The indication is provided separately for each service:

-	"allowed" or "not allowed" for (g)PTP based time synchronization service (per DNN/S-NSSAI and UE identity),

-	"allowed" or "not allowed" for ASTI based time synchronization services (per UE identity).

-	optionally, a list of TA(s) which specifies an area (a so called Time Synchronization Coverage Area) in which an AF may request time synchronization services.

-	one or more Subscribed time synchronization service ID(s), each containing the DNN/S-NSSAI and a reference to a PTP instance configuration pre-configured at the TSCTSF (e.g. PTP profile, PTP domain, etc.):

-	optionally, for each PTP instance configuration, one or more periods of start and stop times defining active times of time synchronization service for the PTP instance.

-	optionally, for each PTP instance configuration, a Time Synchronization Coverage Area defining a list of TAs where the (g)PTP-based time synchronization is available for the UEs in the PTP instance.

-	optionally, Uu time synchronization error budget.

The TSCTSF retrieves the Time Synchronization Subscription data from UDM. The TSCTSF controls the Time Synchronization Service. If the UE subscription includes (g)PTP-based time synchronization service subscription:

-	The TSCTSF retrieves the Time Synchronization Subscription data from the UDM when the TSCTSF receives an AF request for the time synchronization service (either ASTI or (g)PTP).

-	According to the "AF request Authorization" in the UE's subscription data, the TSCTSF determines whether the UE is authorized for an AF-requested time synchronization service and, if the UE's subscription data contains a Time Synchronization Coverage Area (i.e., a list of TA(s) defining the restricted area for AF request), whether the UE is in the allowed area. If the AF request is authorized and the corresponding UE is in the allowed area, the TSCTSF proceeds as specified in clause 5.27.1.8 and TS 23.502 [3].

-	The TSCTSF retrieves the Time Synchronization Subscription data from the UDM when it receives notification from the PCF that a UE has established a PDU Session that is potentially impacted by (g)PTP-based time synchronization service.

-	The TSCTSF retrieves the PTP instance configurations referenced from the "Subscribed time synchronization service ID(s)". The PTP instance configurations are stored locally in the TSCTSF. The TSCTSF determines if one or more of the PTP instance configurations match with the DNN/S-NSSAI of the given PDU Session. If no PTP instance exists for the given PTP instance configuration, the TSCTSF initializes the PTP instance in 5GS as described in clause K.2.2 of TS 23.501 [2].

-	The TSCTSF configures a PTP port in DS-TT and adds it to the corresponding PTP instance in NW-TT as described in clause K.2.2 of TS 23.501 [2].

-	If the PTP instance configuration referenced by the Time Synchronization Subscription data for the UE contains start and stop times, the TSCTSF, upon expiry of start time, creates the PTP instance and adds the PTP port in DS-TT to the PTP instance. Upon expiry of stop time, if this is the last period of start and stop times in the PTP instance configuration, the TSCTSF deletes the PTP instance, otherwise the TSCTSF temporarily disables the PTP instance.

-	If the PTP instance configuration referenced by the Time Synchronization Subscription data for the UE contains a Time Synchronization Coverage Area, the TSCTSF subscribes to UE's Presence in Area(s) of Interest corresponding to the Time Synchronization Coverage Area at the discovered AMF(s). When the TSCTSF determines that the UE has moved inside or outside of the Time Synchronization Coverage Area, the TSCTSF adds or temporarily removes the PTP port in DS-TT from the corresponding PTP instance.

END OF CHANGES