3GPP TSG-SA2 Meeting #155	S2-2302757

# Athens, Greece, February 20 – 24, 2023									(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3915 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys: KI#7 – UE latency performace filtering criterion supported by the NEF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Nokia, Nokia Shanghai Bell, SK Telecom, Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to enable 5GC to assist the Application AI/ML AF to preform candidate member selection, a new NEF services for assistance to member selection is specified. The AF can request the NEF to provide a list of recommended UEs based on the member selection filtering criteria requested by the AF. UE lantecy performace is one of the filtering criteria that can let NEF provide a list of UE which fulfil the transimission latency threshold requested by the AF. Then the AF can further determine which UE(s) should be selected to participate in the AIML operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a UE latency performance criterion and the corresponding description. - Add the procedure on how the NEF can use such criterion to derive the candidate UE list and provide to the AF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide a 5GC mechanism to support the member selection based on the UE latency performance to assist the Application AI/ML operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.Y.2, 4.15.Y.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_AIMLsys. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 4.15.Y.2	Filtering Criteria for 5GS assistance for UE member selection

Table 4.15.Y.2-1 provides a summary of the of the UE member filtering criteria that the AIML AF may request.

Table 4.15.Y.2-1: Description of UE Member filtering criteria

| UE Member filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| UE latency performance | Indicate the UE latency performance that refers to a time delay for completing the transmission of a specific data volume between UE to AF | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = UE Latency Performance | 4.15.Y.3.X |

* * * Next Changes * * *

#### 4.15.Y.3.X	Specific procedure for the UE latency performance criterion

1.	AF subscribes the member selection assistance functionality by sending Nnef_UEMemberSelectionAssistance_subscribe request including a list of target UE(s), the UE latency performace filtering criteria and the time window(s) for selecting the candidate UEs.

2.	NEF verifies the authorization of the AF Request and determine to invoke the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request based on the member selection criteria (i.e. UE latency performace) provided by the AF.

3.	NEF requests or subscribes to analytics for UE latency performance analytics from NWDAF. The Analytics ID is set to “UE lantency performance” and provides the input information as specified in 23.288[50] cause 6.7.x.1.

4.	NWDAF may collect data from AMF, SMF, UPF and OAM as specified in 23.288[50] clause 6.7.x.4.

5.	NWDAF derives requested analytics, in the form of latency performance statistics or predictions or both.

6.	NWDAF provides requested latency performance analytics to the NEF, using either Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response depending on the service used in step 2.

7.	NEF consolidates the information collected from the NWDAF and may also based on the collected information from other 5GC NF to derive the list(s) of candidate UE(s) which fulfill the UE latency performance filtering criterion in the AF request.

8.	NEF sends a Nnef_ UEMemberSelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s).

* * * End of Changes * * *