3GPP TSG-SA WG2 Meeting #155	S2-2302791

# Athens, Greece, Feb 20 – 24, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0194 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resource efficiency in MOCN network sharing for location dependent MBS service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When multiple broadcast MBS Sessions are established via different CNs and a shared NG-RAN to deliver the same location-dependent broadcast MBS service, how to achieve resource efficiency is to be decided. The following options are considered: Option 1: Pure NG-RAN based solution. For the same location dependent broadcast MBS service, it is assumed that the same content is delivered in the MBS service areas which are mapped to the same geographical area and belong to the different CNs. After identifying the MBS service areas from different CNs for the same geographical area, the NG-RAN node can decide to apply resource efficient mechanism for delivering the same content in those MBS service areas. Option 2: AF decides the MBS Area Session ID which is common for all PLMNs When the AF creates location dependent broadcast service towards MB-SMF, it also allocates the corresponding MBS Area Session ID for different MBS service areas and transfers to all the involved MB-SMFs. For this option, extra functionality i.e. MBS Area Session ID allocation function needs to be introduced in AF. Option 3: One MB-SMF decides the MBS area session ID and then transfers it to other MB-SMFs The AF requests broadcast service creation towards one MB-SMF and the MB-SMF provides the MBS Area session ID for each MBS area in the response message. Then in the following broadcast session creation procedures towards other MB-SMFs, AF would include the MBS Area session ID provided by the first MB-SMF. For option 1, there is no extra signaling or configuration requirement; while for option 2 and 3, extra functionality and complex interactions need to be introduced in the 5GC. So it is proposed to adopt Option 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support of resource efficiency in MOCN network sharing for location dependent MBS service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resource efficiency in MOCN network sharing for location dependent MBS service is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

7.3.4	Support for Location dependent Broadcast Service

The clause describes procedures to support the Location dependent broadcast service as described in clause 6.2.

The MBS session creation procedure is performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same Broadcast MBS session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	For Location dependent Broadcast Service, MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the TMGI and Area Session ID.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The MBS service area(s) are indicated to the UE in the Service Announcement as defined in clause 6.11.

The MBS session establishment procedure is performed as defined in clause 7.3.1 with the following additions:

-	MB-SMF requests the AMF to transfer an N2 message (i.e. MBS Session Resource Setup Request) to the NG-RAN nodes of the MBS service area with Broadcast MBS session information which additionally includes the Area Session IDs and MBS service areas.

-	The NG-RAN uses the received MBS Session ID and Area Session IDs to determine the local Broadcast MBS session contexts.

-	The NG-RAN responds for service areas it handles with the Area Session ID(s) and DL tunnel endpoint(s) for the DL tunnel(s) from MB-SMF if unicast transport applies over N3mb between MB-UPF and NG-RAN.

-	According to the Area Session ID(s) and DL tunnel endpoint(s) provided by NG-RAN, the MB-SMF instructs the MB-UPF to send location dependent content.

When multiple location dependent broadcast MBS sessions are established in different CNs for the same broadcast MBS service, it is assumed that the same content is delivered in the MBS service areas which are mapped to the same geographical area and belong to the different CNs. If a shared NG-RAN is used by the different CNs for the location dependent broadcast MBS sessions delivery, to support resource efficiency:

- The NG-RAN determines the broadcast MBS sessions delivering the same location dependent broadcast MBS services as specified in clause 6.X and additionally based on the Area Session IDs and MBS service areas received from the MB-SMFs for the broadcast MBS sessions.

- The NG-RAN responds for service areas it handles with the Area Session ID(s) and DL tunnel endpoint(s) to either multiple MB-SMFs of the different PLMN or only one MB-SMF, depending on whether the NG-RAN decides to receive multiple copies of MBS data from the CNs of different PLMNs or only a single copy of MBS data, if unicast transport applies over N3mb between the MB-UPF(s) and NG-RAN. If multiple DL tunnels are established towards the MB-UPFs in different CNs for multiple MBS service areas which are mapped to the same geographical area, the NG-RAN realizes that the multiple copies of MBS data received from these DL tunnels carry the same content of the location dependent broadcast MBS service.

- The NG-RAN allocates radio resource for only one of the established broadcast MBS Sessions in the MBS service areas it handles, according to the mapping of Area Session IDs, MBS service areas and associated MBS session information received from one of the MB-SMFs.

* * * End of Changes * * * *