# 3GPP TSG-WG SA2 Meeting #155	S2-2302817

# Athens, Greece, February 20 - 24,  2023 		(revision of S2-230xxxx)

| # CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | # 23.502 | # CR | # 3919 | # rev | # - | # Current version: | # 18.0.0 |  |
| # For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| # Proposed change affects: | # UICC apps |  | # ME |  | # Radio Access Network |  | # Core Network | # X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| # Title: | # Event exposure for bulk subscription from the PCF to the AF in TS 23.502 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Source to WG: | # China Mobile, Nokia, Nokia Shanghai Bell, ZTE |  |  |  |  |  |  |  |  |  |
| # Source to TSG: | # SA2 |  |  |  |  |  |  |  |  |  |
| # Work item code: | # TEI18_ADEE |  |  |  |  |  | # Date: |  |  | # 2023-01-31 |
| # Category: | # B |  |  |  |  |  | # Release: |  |  | # Rel-18 |
|  | # Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) # Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | # Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| # Reason for change: |  | # The event exposure for bulk subscription from the PCF to the AF directly or via NEF may be useful for the AF to enforce application layer logic and the related description is also specified in TS 23.503 as following: # +++ # If the AF requests the PCF to report on the outcome of the service area coverage change, the PCF reports the outcome of the service area coverage change to the AF and notifies the current service area coverage to the AF. The outcome is the result of the execution of the request of service coverage change at the PCF; the outcome is successful if the request was executed, and includes the current service area coverage that may be the same or different from the service area coverage provided by the AF. The subscription may also be implicit. In this case there may be bulk subscription, either for an Internal-Group-Id or for any UE. In order to prevent massive notifications to the AF, the request for any UE is associated to a specific Application Identifier or DNN, S-NSSAI. For bulk subscription, when the AF request includes an expiration time, the PCF stops reporting to the AF when the expiration time is reached. # +++ # But how the procedure works is still not clear in TS 23.502, therefore, it is proposed to adding the procedure about the event exposure for bulk subscription from the PCF to the AF directly or via NEF. |  |  |  |  |  |  |  |  |
| # Summary of change: |  | - Add the procedure that the AF subscribes to notifications of events from the PCF with the bulk subscription directly or via NEF. - Fix the consumer of the corresponding service operation. |  |  |  |  |  |  |  |  |
| # Consequences if not approved: |  | # How the AF subscribes to notifications of events from the PCF with the bulk subscription is missing. |  |  |  |  |  |  |  |  |
| # Clauses affected: |  | # 4.15.3.2.4, 5.2.5.1 |  |  |  |  |  |  |  |  |
|  |  | # Y | # N |  |  |  |  |  |  |  |
| # Other specs |  |  | # X | # Other core specifications |  |  |  | # TS/TR ... CR ... |  |  |
| # affected: |  |  | # X | # Test specifications |  |  |  | # TS/TR ... CR ... |  |  |
| # (show related CRs) |  |  | # X | # O&M Specifications |  |  |  | # TS/TR ... CR ... |  |  |
| # Other comments: |  |  |  |  |  |  |  |  |  |  |
| # This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 4.15.3.2.4	Exposure with bulk subscription

Based on operator configuration NEF may perform bulk subscription with the NFs that provides necessary services. This feature is controlled by local policies of the NEF that control which events (set of Event ID(s)) and UE(s) are target of a bulk subscription.

When the NEF performs bulk subscription (subscribes for any UE (i.e. all UEs), group of UE(s) (e.g. identifying a certain type of UEs such as IoT UEs)), it subscribes to all the NFs that provide the necessary services (e.g. In a given PLMN, NEF may subscribe to all AMFs that support reachability notification for IoT UEs). Upon receiving bulk subscription from the NEF, the NFs store this information. Whenever the corresponding event(s) occur for the requested UE(s) as in bulk subscription request, NFs notify the NEF with the requested information.

The following call flow shows how network exposure can happen for one UE, groups of UE(s) (e.g. identifying a certain type of UEs such as IoT UEs) or any UE.

Figure 4.15.3.2.4-1: NF registration/status notification and Exposure with bulk subscription

1.	NEF subscribes to the NRF for any newly registered NF along with its NF services.

2.	When an NF instantiates, it registers itself along with the supported NF services with the NRF.

3.	NRF acknowledges the registration

4.	NRF notifies the NEF with the newly registered NF along with the supported NF services.

5.	NEF evaluates the NF and NF services supported against the pre-configured events within NEF. Based on that, NEF subscribes with the corresponding NF either for a single UE, group of UE(s) (e.g. identifying a certain type of UEs such as IoT UEs), any UE. NF acknowledges the subscription with the NEF.

6 - 7.	When the event trigger happens, NF notifies the requested information towards the subscribing NEF along with the time stamp. NEF may store the information in the UDR along with the time stamp using either Nudr_DM_Create or Nudr_DM_Update service operation as appropriate.

8.	Application registers with the NEF for a certain event identified by event filters. If the registration for the event is authorized by the NEF, the NEF records the association of the event and the requester identity.

9 - 10.	When the event trigger happens, NF notifies the requested information towards the subscribing NEF. NEF may store the information in the UDR using either Nudr_DM_Create or Nudr_DM_Update service operation as appropriate.

11a-b.	NEF reads from UDR with Nudr_DM_Query and notifies the application along with the time stamp for the corresponding subscribed events.

The application detection event exposure from PCF may be used by an AF for the detection of application start or stop event. In this case, the AF may subscribe to notifications of events from the PCF with the bulk subscription as described in clause 6.1.3.18 of TS 23.503 [20], either by sending Npcf_EventExposure_Subscribe directly to the PCF or via NEF by sending Nnef_EventExposure_Subscribe request. Subscription can be cancelled by sending Npcf_EventExposure_Unsubscribe directly to PCF or Nnef_EventExposure_Unsubscribe via NEF. The corresponding call flow is as follows:

Figure 4.15.3.2.4-2: bulk subscription to notifications of events from the PCF by the AF

1.	AF sends request to NEF to subscribe to notifications of events via Nnef_EventExposure_Subscribe.

2.	NEF subscribes to the PCF serving S-NSSAI and/or DNN via Npcf_EventExposure_Subscribe on the notifications of events.

3.	If the NEF is not deployed, a trusted AF may directly send the request to PCF serving S-NSSAI and/or DNN via Npcf_EventExposure_Subscribe to subscribe to notifications of events.

4.	When a PDU session is established, the serving SMF may trigger the SM Policy Association Establishment procedure as specified in clause 4.16.4, and the PCF may provide to the SMF the related PCRT on the notifications of events.

Additionally, the PCF may also update the policy information of those PDU Sessions that are already established for users of the group to subscribe to the SMF on the notifications of events.

5.	When the event trigger happens, for example, the SMF receives the report from UPF about detecting the start or the stop of application traffic, SMF triggers the SM Policy Association Modification procedure as specified in clause 4.16.5.1, to report the requested information towards the PCF.

6~7.	PCF notifies AF of the requested information via NEF.

PCF may remove the PCRT from the SMF when the PCF receives the notification of the event for the first time.

8.	PCF notifies AF of the requested information.

# * * * * Second change * * * *

5.2.5.1	General

The following table illustrates the PCF Services.

Table 5.2.5.1-1: NF services provided by PCF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Npcf_AMPolicyControl | Create | Request/Response | AMF |
|  | Update | Request/Response | AMF |
|  | UpdateNotify | Subscribe/Notify | AMF |
|  | Delete | Request/Response | AMF |
| Npcf_Policy Authorization | Create | Request/Response | AF, NEF, TSCTSF |
| (NOTE) | Update | Request/Response | AF, NEF, TSCTSF |
|  | Delete | Request/Response | AF, NEF, TSCTSF |
|  | Notify | Subscribe/Notify | AF, NEF, NWDAF, PCF, TSCTSF |
|  | Subscribe |  | AF, NEF, NWDAF, PCF, TSCTSF |
|  | Unsubscribe |  | AF, NEF, NWDAF, PCF, TSCTSF |
| Npcf_SMPolicyControl | Create | Request/Response | SMF |
|  | UpdateNotify | Subscribe/Notify | SMF |
|  | Update | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
| Npcf_BDTPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
| Npcf_UEPolicyControl | Create | Request/Response | AMF, V-PCF |
|  | Update | Request/Response | AMF, V-PCF |
|  | UpdateNotify | Subscribe/Notify | AMF, V-PCF |
|  | Delete | Request/Response | AMF, V-PCF |
| Npcf_EventExposure | Subscribe | Subscribe/Notify | AF, NEF, NWDAF |
|  | Unsubscribe |  |  |
|  | Notify |  |  |
| Npcf_AMPolicyAuthorization | Create | Request/Response | AF, NEF, TSCTSF |
|  | Update | Request/Response | AF, NEF, TSCTSF |
|  | Delete | Request/Response | AF, NEF, TSCTSF |
|  | Notify | Subscribe/Notify | AF, NEF, 5G DDNMF |
|  | Subscribe |  | AF, NEF, 5G DDNMF |
|  | Unsubscribe |  | AF, NEF, 5G DDNMF |
| Npcf_PDTQPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
| NOTE: In the Npcf_PolicyAuthorization operations, PCF is a consumer when the PCF for the UE and the PCF for the PDU session are different. |  |  |  |

# * * * * End of changes * * * *