3GPP TSG SA WG2 Meeting #155	S2-2302835

# February 20 - 24, 2023, Athens, Greece	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1273 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Symmetric application data channel call flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Sol#20 in TR 23.700-87 is used as baseline for the DC solution description in TS 23.228. TR 23.700-87 contains three procedures: one to establish a bootstrap DC, one to establish a P2P application DC and one to establish a P2A/A2P application DC. The last procedure allows only the originating UE to establish an application DC to the DCMF to communciate with a DC AS. For applications like language translation it is necessary that both UEs establish an application DC and communicate via the application DC with the same DC AS, e.g., UE#1 sends text to the DC AS which translates the text into a different language and sends it to the peer UE#2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new call flow under the Procedure clause is proposed enabling originating and terminating UE to establish symmetrically application data channels for the same application to communicate with the corresponding Application Data Channel server via the application data channels. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solution does not cover all relevant DC use cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX.x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** BEGIN of Changes (all new text) ****

Annex XX (normative):
Support of Data Channel services in IMS

# XX.x	Procedures

### XX.x.y	Application data channel establishment for originating and terminating UE

This procedure enables originating and terminating UE to establish application data channels for the same application to communicate with the same Data Channel Application Server.

Figure XX.x.y-1: Symmetric Application Data Channel Establishment

0.	IMS session and bootstrap data channels are established. Data channel applications are downloaded to UE#1 and UE#2.

1.	UE#1 sends SIP re-INVITE request with an updated SDP to IMS AS. The updated SDP contains the bootstrap data channel and the application data channel information.

2.	IMS AS validates the data channel media description information and/or user subscription data to determine whether the DCSF needs to be notified.

3.	IMS AS selects and notifies the DCSF about the call event and data channel establishment request.

4.	The DCSF determines how to process the application data channel establishment request based on the parameters in the notification from DCSF and/or operator policies.

5.	DCSF determines that the added application data channel media in the SDP offer requires the DC Application Server is the endpoint for both originating and terminating UE and that the application DC must be anchored at the DCMF or enhanced MRF. DCSF communicates with the DC Application Server for DC resource control. Once the application data channel is established, tha DC Application Server will send/receive traffic to/from UE#1 through the MDC2 interface.

NOTE X:	Details on how the DCSF communicates with the DC Application Server are out of scope of Release 18.

6.	DCSF invokes Nimsas MediaControl service to send data channel control request to IMS AS, including information how to relay data channel media via the MDC2 interface.

7.	IMS AS reserves data channel media resources at the DCMF via DC2 or at the enhanced MRF via Mr’/Cr based on the DC media information received from DCSF.

8.	IMS AS notifies the DCSF about MediaControl instruction control response.

9.	The DCSF stores the media resource information and establishes MDC1 connections with DCMF or enhanced MRF for application DC traffic to UE#1 and replies to the Nimsas notification request.

10-11.	IMS AS sends re-INVITE which include the SDP offer from DCMF or enhanced MRF for the application data channel to the originating S-CSCF and then to the remote network and UE#2.

12-15.	UE#2 and terminating network return 200 OK with SDP answer for audio/video and for the application data channel. The terminating network P-CSCF executes QoS procedure for application data channel media based on the SDP in the 200 OK.

16. IMS AS notifies the DCSF about the successful result of the MediaChangeRequest event.

17.	DCSF replies to IMS AS.

18-19.	The IMS AS includes SDP answer for application data channel to UE#1 in 200 OK and sends 200 OK to S-CSCF and P-CSCF.

20.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP in the 200 OK.

21.	P-CSCF returns the 200 OK to UE#1.

22.	UE#1 sends ACK to the terminating network.

23.	The application data channel between UE#1 and DC Application Server is established via DCMF or enhanced MRF. DCMF or enhanced MRF forwards data channel traffic between UE#1 and DC Application Server via MDC2.

24.	The application data channel between UE#2 and DC Application Server is established via DCMF or enhanced MRF. DCMF or enhanced MRF forwards data channel traffic between UE#2 and DC Application Server via MDC2.

**** End of Changes ****