3GPP TSG-WG SA2 Meeting #155	S2-2302872

February 20-24, 2023  Athens															(revision of S2-23xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0715 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNA_Ph3 ML Model storage in ADRF procedure in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is concluded in TR 23.700-81 for KI#4 as follow: 7. For storing ML models in ADRF, MTLF can store ML model in ADRF based on MTLF policy. 8. MTLF sends the ML model or ML model address to ADRF by using ADRF's ML model storage service. NOTE 2: How MTLF sends the ML model by using 3GPP defined service-based interface is determined in normative phase. Therefore, the procedure description of ML Model storage in ADRF needs to be updated. In addition, new new ADRF services used for ML model storage and delete has been defined in #154-AH-e. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause 6.2A.X - Defining procedure of ML Model storage and delete in ADRF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Storing ML models in ADRF is not updated to reflect conclusions in TR23.700-81 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change (all new text) * * *

### 6.2A.X	ML Model storage and delete

The procedure in Figure 6.2A.X-1 is used by an ADRF service consumer, i.e. an NWDAF containing MTLF tostorage and delete ML Model Information from ADRF, using Nadrf_MLModelManagement services as defined in clause 10.X.

Figure 6.2A.X-1: ML model storage and delete

1.	The ADRF service consumer (i.e. an NWDAF containing MTLF) request to store ML model(s) by invoking Nadrf_MLModelManagement_StorageRequest service operation. ML model(s) or ML model address(es) stored in NWDAF containing MTLF are provided to the ADRF in the request message.

The parameters that can be provided by the ADRF Service Consumer are as follow:

-	the tuple (NF instance ID of the NWDAF containing MTLF, address (e.g. URL or FQDN) of Model file stored in NWDAF containing MTLF), when multiple ML models is not supported; or

-	the tuple (NF instance ID of the NWDAF containing MTLF, one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file stored in NWDAF containing MTLF).

NOTE:	The ADRF can download ML model files based on the ML model addresses provided by the NWDAF containing MTLF. How the ADRF downloads the ML model and locally stores the ML model is left for implementation.

2.	The ADRF service consumer (i.e. an NWDAF containing MTLF) request to delete ML model file(s) or ML model address(es) by invoking Nadrf_MLModelManagement_Delete service operation.

* * *End of Changes * * *