3GPP TSG-WG SA2 Meeting #155 	S2-2302899

February 20 – 24, 2023, Athens, GR	(revision of S2-230xxxx)

Source:	Sony

Title:	Introduction of Ranging and sidelink positioning service exposure to a UE

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper introduces the feature Ranging and sidelink positioning service exposure to a UE in TS 23.586.

# 1. Introduction/Discussion

This paper proposes to add the feature Ranging and sidelink positioning service exposure to a UE based on the KI#6 conclusion in TR 23.700-86.

# 2. Text Proposal

It is proposed to capture the following changes in TS 23.586.

# * * * * First change* * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[6]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[7]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[x]	3GPP TS 33.122: " Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

# * * * * Second change* * * *

## 5.x	Ranging and sidelink positioning service exposure to a UE

### 5.x.1 General

A SL Positioning Client UE may use Ranging/Sidelink Positioning service exposure if the SL Positioning Client UE is authorized to use the exposure service and the involved UEs privacy criteria are met.

The SL Positioning Client UE can invoke the Ranging and sidelink positioning service exposure through PC5 or through the 5GC network.

### 5.x.2 Ranging/Sidelink Positioning service exposure through PC5

Ranging and sidelink positioning service exposure to a UE through PC5 is provided in the following way:

-	SL Positioning Client UE discovers one of Reference UE and Target UE, that supports the exposure service.

-	SL Positioning Client UE invokes the Ranging/Sidelink Positioning service exposure on either the Reference UE or Target UE. The service request includes the user info of SL Positioning Client UE, Reference UE and Target UE.

-	The UE providing the exposure service triggers the authorization of SL Positioning Client UE and check that the Reference UE’s and Target UE’s privacy criteria are met.

Editor’s Note: The procedure for Privacy handling on Ranging and sidelink positioning service exposure needs to be determined by SA3 Group i.e. the authorization of the SL positioning client UE for Ranging and sidelink positioning service invocation of two other UEs.

-	The Ranging/Sidelink positioning procedure between Reference UE and Target UE is performed and the UE providing the service exposure provides the Ranging/Sidelink positioning result to SL Positioning Client UE.

### 5.x.3 Ranging/Sidelink Positioning service exposure through 5GC network

Ranging and sidelink positioning service exposure to a UE through network, is provided either over user plane or over control plane.

The Ranging and sidelink positioning service exposure over user plane is triggered by the SL positioning client UE invoking the Application exposure API over the application layer as specified clause 5.y with the following additions:

-	The SL Positioning Client UE is provisioned with the Onboarding Enrolment Information during the registration procedure. The Onboarding Enrolment Information is specified in TS 33.122 [x].

-	The authorization of the SL positioning client UE for Ranging and sidelink positioning service invocation performed and checking that the Reference UE’s and Target UE’s privacy criteria are met, follows the same the procedures as for the AF in clause 5.y.

Editor’s Note: SA WG3 needs to confirm that the authorization of the SL Positioning Client UE can reuse the same procedures as for authorizing AF to use the Application exposure API.

The Ranging and sidelink positioning service exposure over control plane is triggered by:

-	SL positioning client UE sends an enhanced MO-LR to AMF which includes user info of the Reference UE and Target UE. The AMF sends a Ranging/SL positioning request of the Reference UE ID and Target UE ID to the GMLC, the request also includes the SL positioning client UE ID.

-	GMLC reuses the procedure described in the clause 5.y to check that the SL positioning client UE is authorized, check that the Reference UE’s and Target UE’s privacy criteria are met, and sends the Ranging/SL positioning request to the AMF serving either the Reference UE or Target UE.

-	The AMF, serving the SL positioning client UE, sends the Ranging/SL positioning result to the SL positioning client UE when it receives the result from the GMLC.

Editor’s Note: The procedure for Privacy handling on Ranging and sidelink positioning service exposure needs to be determined by SA3 Group i.e. the authorization of the SL positioning client UE for Ranging and sidelink positioning service invocation of two other UEs.

# * * * * End of changes * * * *