3GPP TSG-WG SA2 Meeting #155	S2-2302905

20 - 24 February 2023, Athens, Greece vision of S2-220xxxx)

Source:	Xiaomi

Title:	Discussion on UE provide information to AMF/MME

Document for:	Approval

Agenda Item:	9.2.2

Work Item / Release:	5GSAT_Ph2 / Rel-18

Abstract: the paper discusses solution optimization of UE providing information to AMF/MME.

- Overall

The CR agreed in S2-2301769 at SA2-154e-AH contains the following contents:

If the UE is able to determine its own UE out-of-coverage period, and decides to remain in no service, then it can trigger the Mobility Registration Update procedure, with sufficient time to complete the procedure before the start of the unreachability period to either:

a)	request MICO mode parameters, extended DRX in CM-IDLE parameters, or other NAS timers taking the UE out-of-coverage period into account using the relevant procedures. If the UE requests MICO mode or usage of eDRX, the UE shall not include a UE out-of-coverage period; or

b)	inform the network of its UE out-of-coverage period, when it is about to leave satellite coverage and then perform a Mobility Registration Update procedure when it returns to coverage using any access type. The AMF may indicate to the UE that the UE is required to perform this procedure when leaving and returning to coverage. If the AMF does not indicate to the UE that the UE is required to perform this procedure, the usage is UE implementation dependent. If the UE indicates its out-of-coverage period, the UE shall not request any power saving parameters e.g. Active Time or extended DRX in CM-IDLE state.

It shows the UE can send request PSM parameters and NAS timers or its UE out-of-coverage period to the network when it is about to leave satellite coverage. Meanwhile, sending PSM parameters and sending its out-of-coverage period are exclusively during contacting the network.

Also there is an EN says:

Editor’s Note:	How a UE determines to send its out-of-coverage period or requests PSM parameters is FFS and may need further updates.

This paper tries to discuss the information provided by UE to AMF.

2	Discussion

In case of UE is able to determine its own out-of-coverage period, two options can support mobility management and power saving optimization:

Option#1: The UE determines MICO/PSM parameters and NAS timer based on UE out-of-coverage period and sends them to the AMF/MME. The AMF/MME provides the authorized parameters to the UE.

Option#2: The UE provides the UE out-of-coverage period to the AMF/MME. The AMF/MME sets MICO/PSM parameters and NAS timer taking UE out-of-coverage period into account and provides them to the UE.

It’s not clear in what case the UE needs to provide MICO/PSM parameters and NAS timers while in other cases the UE needs to provide its out-of-coverage period to AMF/MME. Or is it possible to use one option to support all the cases. Here is the analysis for the two options.

To support power saving and mobility management under discontinuous coverage, MICO/PSM parameters and NAS timers should be set based on UE out-of-coverage period. The differences between the two options is that in option 1 UE determines them and request AMF to authorize while in option 2 AMF determines them and provides to UE. Setting these parameters/NAS timers is the exist functionality of AMF as per existing standard but is a new functionality to the UE. Although the AMF has the enhancement to support parameters determination based on UE out-of-coverage period in option 2, such enhancement is required as well in the case of AMF getting coverage information from AF/OAM.

Observation#1: Option 2 has the same MICO/PSM parameters/NAS timers determination as the case of AMF getting coverage information from AF/OAM. AMF functionality to determine MICO/PSM parameters/NAS timers which defined in the previous specification can be reused.

The two options have the following enhancement to UE/AMF:

Common enhancement of option 1&2 is:

- UE capability: calculate its out-of-coverage period taking coverage map, its location and UE trajectory into account.

The enhancement of option 1 are:

- UE capability : determine MICO/PSM parameters, NAS timer based on its out-of-coverage period

- NAS enhancement: send MICO/PSM parameters, NAS timer from UE to AMF/MME to request authorization.

- AMF/MME capability: perform MICO/PSM parameters and NAS timers authorization. Set Mobile Reachable timer and implicit deregistration timer based on the request parameters.

The enhancement of option 2 are:

- NAS enhancement: send UE-out-of-coverage from UE to AMF/MME

- AMF/MME capability: generate PSM parameters/NAS timer based on UE out-of-coverage period.

Comparing with option 1, option 2 has less enhancement to UE and AMF. The mechanism of AMF determining MICO/PSM parameters and NAS timer can be reused without the need to enhance the same functionality on UE.

Regarding NAS timer, the Periodic Registration timer is used by UE and is set based on out-of-coverage period. In addition, in order to avoid unintended implicit detach due to coverage gap, Mobile Reachable timer and implicit deregistration timer should be set taking out-of-coverage period into account as well. In option 1, since the AMF has no information about UE out-of-coverage period, the two timers applied to the AMF need to be set by UE and sent them to AMF. This will be a key enhancement for the UE as UE doesn’t need to know the two timers as per existing standards.

Observation 2: Option 2 has less feature enhancement to the UE and AMF than Option 1.

Based on the above, option 2 has less enhancement to the UE and AMF. It can provide the same process of PSM parameters/NAS timer determine as the case of AMF getting coverage information from AF/OAM. In addition, if both options are required to be supported by UE, the UE needs to support providing out-of-coverage information to AMF. With this capability, deriving MICO/PSM parameter and NAS timers by UE is not needed.

3	Proposal

It is proposed using option 2 to optimize information provision to AMF by UE. The optimization is reflected in S2-2302906.