3GPP TSG-SA WG2 Meeting #155	S2-2302989

# Athene, Greece, February 20 - 24, 2023								    (revision of S2-2301572)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0169 | rev | 9 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Path switching between PC5 path and Uu path |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [LG Electronics, MediaTek Inc., Huawei, HiSilicon], Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion for KI#3 "Support direct communication path switching between PC5 and Uu (i.e. non-relay case)" described in clause 8.3 of TR 23.700-33, this feature needs to be specified. It is agreed that UEs may share their IP addresses/prefixes used for Uu path via the existing PC5 connection however, it’s not clear how these IP addresses/prefixes are shared. There is an Editor’s Note to be cleared: Editor's Note: When and how to share the IP address/prefixes is FFS. After path switching negotiation from PC5 to Uu, it is not clear how UEs may determine if the Uu path is ready and when traffic can be switched from PC5 to Uu. The preservation of the PC5 unicast link after path switching to Uu path need to be agreed by both peer UEs. Otherwise, we may end-up in a situation where UE1 would like to preserve the PC5 unicast link but UE2 releases it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.1.3.1 - Add path switching policy §5.x - Add new clause about funtional description on Communication path switching between PC5 and Uu reference points §6.x - Add new clause about procedures for Communication path switching between PC5 and Uu reference points - Specify how and when UE1 and UE2 IP addresses/prefixes used over Uu path are shared. Clear the EN. - Add Path Switching Ack message to indicate that traffic is ready to be switched. - Specify that UEs agree to maintain the PC5 unicast link after path switching to Uu path. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Path switching between PC5 path and Uu path is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.1, 5.x (new), 6.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 5.1.3.1	Policy/Parameter provisioning for 5G ProSe Direct Communication

The following sets of information for 5G ProSe Direct Communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "served by NG-RAN".

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "not served by NG-RAN".

NOTE 1:	In this specification, [When the UE is "served by NG-RAN"] and [When the UE is "not served by NG-RAN"] are relevant to 5G ProSe Direct Communications over NR PC5 reference point.

2)	Groupcast mode 5G ProSe Direct Communication policy/parameters:

-	For each application layer group supported include the parameters that enable the UE to perform Groupcast mode 5G ProSe Direct Communication when provided by PCF or provisioned in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group that the UE belongs to.

-	ProSe Layer-2 Group ID: Destination Layer-2 ID applicable only when the Application Layer Group ID is provided by the application layer

-	ProSe Group IP multicast address

-	Indication whether the UE should use IPv4 or IPv6 for that group

-	For a specific Group configured to operate using IPv4, optionally an IPv4 address to be used by the UE as a source address. If none is provisioned, then the UE shall use Dynamic Configuration of IPv4 Link-Local Addresses RFC 3927 [18] to obtain a link local address for the Group.

3)	Radio parameters when the UE is "not served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Communications over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 2:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

4)	Policy/parameters related to privacy:

-	The list of ProSe services (i.e. ProSe identifiers) with Geographical Area(s) that require privacy support.

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

5)	Policy/parameters when NR PC5 is selected:

-	The mapping of ProSe services (i.e. ProSe identifiers) to radio frequencies with Geographical Area(s).

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for broadcast.

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for groupcast.

-	The mapping of ProSe services (i.e. ProSe identifiers) to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

NOTE 3:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one ProSe service. In the case where different ProSe services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one ProSe service, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

NOTE 4:	Security policies for Unicast mode 5G ProSe Direct Communication can be provisioned by PCF as defined in TS 33.503 [29].

-	The mapping of ProSe services (i.e. ProSe identifiers) to PC5 QoS parameters defined in clause 5.6.1 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

-	The mapping of ProSe services (i.e. ProSe identifiers) to the corresponding NR Tx Profiles for broadcast and groupcast (see TS 38.300 [12] and TS 38.331 [16] for further information).

-	AS layer configurations (see TS 38.331 [16]), i.e. the mapping of PC5 QoS profile(s) to radio bearer(s), when the UE is "not served by NG-RAN".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2 of TS 23.287 [2], and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1 of TS 23.287 [2] and in Table 5.6.1-1.

-	For broadcast, groupcast and initial signalling to establish unicast connection, PC5 DRX configuration (see TS 38.331 [16]), e.g. the mapping of PC5 QoS profile(s) to PC5 DRX cycle(s), default PC5 DRX configuration, when the UE is "not served by NG-RAN".

6)	Path selection policy:

-	The mapping of ProSe services (i.e. ProSe identifiers) to path preference (i.e. PC5 preferred, Uu preferred, or no preference) as defined in clause 5.11.

NOTE 5:	The path selection policy can be a one mapping for all ProSe services, i.e. same path preference for all ProSe services.

7)	Path switching policy:

-	The mapping of ProSe services (i.e. ProSe identifiers) to path switching permission (i.e. which path(s) is permitted for path switching: PC5 permitted, Uu permitted, or both PC5 and Uu permitted) as defined in clause 5.x.

Editor's Note:	Detailed format of 7) and whether 7) and 6) can be combined is FFS.

NOTE 6:	The path switching policy can be a one mapping for all ProSe services, i.e. same path preference for all ProSe services.

8)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Direct Communication.

The above parameter sets from bullet 2) to 8) may be provided or updated to the UE by the ProSe Application Server.

* * * * Start of Next Change * * * *

## 5.x	Communication path switching between PC5 and Uu reference points

The "communication path switching between PC5 and Uu reference points" refers to the procedure on how a UE switches between direct Uu communication path and direct PC5 communication path when it is communicating with another UE as illustrated in Figure 5.x-1. The direct communication path over PC5 reference point means that the communication with another UE is performed by using 5G ProSe Direct Communication only. The direct communication path over Uu reference point means that the communication with another UE is performed via the network.

Figure 5.x-1: Example scenario of communication path switching between PC5 and Uu reference points (i.e. switching between Figure a and Figure b)

Path switching policy is provided to the UE to indicate which path(s) is permitted for path switching for all or specific ProSe services as specified in clause 5.1.3.1.

The UE may use the provisioned path switching policy to determine whether path switching is permitted for all or specific ProSe services. The granularity of path switching between PC5 and Uu reference points is per ProSe service.

The UE may decide to perform path switching from Uu path to PC5 path, e.g. because the peer UE is in proximity or for offloading some traffic from the network. The UE may decide to perform path switching from PC5 path to Uu path, e.g. when the PC5 signal strength of the unicast link with the peer UE becomes weak.

For communication path switching from PC5 to Uu reference point, the Uu QoS parameters of each UE are decided based on PC5 QoS parameters and negotiated via the existing PC5 connection between two UEs. Each UE may perform the UE requested PDU Session Modification as defined in TS 23 502 [5] clause 4.3.3 to request the Uu QoS parameters. The IP addresses/prefixes of the peer UEs over the Uu reference point can be shared via the existing PC5 connection between the UEs to assist them to establish communication with each other via the communication path over the Uu reference point.

When both Uu path and PC5 path are available, a make-before-break mechanism is used when path switching between PC5 path and Uu path. That is, when path switching occurs the UEs prepare the target path to communicate with each other, then perform the path switching from PC5 path to Uu path and vice versa and communicate over the target path, and at which point for the former path may be released.

NOTE:	It is not targeted to support session continuity (e.g. IP address preservation) during path switching between PC5 and Uu reference points.

* * * * Start of Next Change * * * *

## 6.x	Procedures for communication path switching between PC5 and Uu reference points

### 6.x.1	Procedure for communication path switching from Uu reference point to PC5 reference point

Figure 6.x.1-1 depicts the procedure for communication path switching from Uu reference point to PC5 reference point.

Figure 6.x.1-1: Procedure for communication path switching from Uu reference point to PC5 reference point

1.	UE-1 and UE-2 communicate with each other via Uu path, i.e. by using the established PDU Sessions.

2.	UE-1 decides to perform path switching for ProSe service(s) from Uu path to PC5 path, e.g. because UE-1 and UE-2 are in proximity each other or for offloading some traffic from the network.

NOTE 1:	UE-1 may perform ProSe discovery to determine whether UE-2 is in proximity using the User Info of UE-2 provided by the application layer.

UE-1 determines whether and which ProSe service(s) can be switched to PC5 path based on the path switching policy as specified in clause 5.1.3.1.

3.	UE-1 triggers the establishment of a PC5 unicast link by using the Layer-2 link establishment procedure as specified in clause 6.4.3.1. The PC5 unicast link is established including the ProSe service(s) that can be switched to PC5 path. If any Layer-2 link already established between UE-1 and UE-2 can be used for this path switching, the Layer-2 link modification as specified in clause 6.4.3.4 is used.

4.	Traffic for the ProSe service(s) is switched from Uu path to PC5 path.

5a.	UE-1 may release its PDU Session, e.g. if no traffic transmitted over the PDU Session, or modify its PDU Session, e.g. only part of the services is switched.

5b.	UE-2 may release its PDU Session, e.g. if no traffic transmitted over the PDU Session, or modify its PDU Session, e.g. only part of the services is switched.

NOTE 2:	Steps 5a and 5b can be executed in parallel.

NOTE 3: 	As an alternative to steps 5a and 5b, the UP connection of the relevant PDU Sessions can be deactivated as specified in clause 4.3.7 of TS 23.502 [5].

### 6.x.2	Procedure for communication path switching from PC5 reference point to Uu reference point

Figure 6.x.2-1 depicts the procedure for communication path switching from PC5 reference point to Uu reference point.

Figure 6.x.2-1: Procedure for communication path switching from PC5 reference point to Uu reference point

1.	UE-1 and UE-2 communicate with each other via PC5 path, i.e. by using the established PC5 unicast link between them.

2.	UE-1 decides to path switch ProSe service(s) from PC5 path to Uu path, e.g. if the PC5 signal strength of the unicast link with UE-2 reaches certain configured signal strength threshold.

UE-1 determines whether and which ProSe service(s) can be switched to Uu path based on the path switching policy as specified in clause 5.1.3.1.

UE-1 sends a Path Switch Request message to UE-2 to negotiate the ProSe service(s) to be switched. UE-1 includes the ProSe service(s) that can be switched to Uu path in the Path Switch Request message.

Optionally, UE-1 can decide the Uu QoS parameters of each UE based on PC5 QoS parameters for each ProSe service. UE-1 may include Uu QoS parameters used for UE-2's Uu path for the ProSe service(s) that can be switched to a Uu path in the Path Switch Request message.

3.	UE-2 responds with a Path Switch Response message.

UE-2 determines whether and which ProSe service(s) can be switched to Uu path based on e.g., the path switching policy as specified in clause 5.1.3.1, availability of Uu path, etc.

If UE-2 accepts the path switch request from UE-1, UE-2 includes the accepted ProSe service(s) to be switched to Uu path in the Path Switch Response message from those indicated by UE-1. Otherwise, UE-2 rejects the path switch request with the appropriate cause (e.g. path switching to Uu path for the ProSe service(s) is not permitted, Uu path is not available) in the Path Switch Response message.

If available, IP address of UE-2 may be included.

4a.	UE-1 may establish or modify PDU Session by using the PDU session establishment procedure as specified in clause 4.3.2 or the PDU session modification procedure as specified in clause 4.3.3 of TS 23.502 [5]. UE-1 may set the Requested QoS as the Uu QoS parameters used for UE-1's Uu path for the specific accepted ProSe service in the PDU Session Modification Request. The PDU Session needs to support QoS requirements for the ProSe service(s) to be switched from PC5 path.

4b.	UE-2 may establish or modify PDU Session by using the PDU session establishment procedure as specified in clause 4.3.2 or the PDU session modification procedure as specified in clause 4.3.3 of TS 23.502 [5]. UE-2 may set the Requested QoS as the Uu QoS parameters used for UE-2's Uu path for the specific accepted ProSe service in the PDU Session Modification Request. The PDU Session needs to support QoS requirements for the ProSe service(s) to be switched from PC5 path.

NOTE 1:	Steps 4a and 4b can be executed in parallel, and step 4b can be performed before step 3.

5.	UE-1 sends Path Switch Ack message to confirm it has successfully established or modified a PDU session for path switching. Path Switch Ack message may include IP addresses/prefixes of UE-1 and the result of PDU session establishment/modification of step 4a to inform if the result and accepted ProSe service(s) are different.

6.	Traffic for the accept ProSe service(s) is switched from PC5 path to Uu path.

7.	The PC5 unicast link may be released, e.g. if no more ProSe services over the PC5 unicast link.

NOTE 2:	The UEs can agree to maintain the PC5 unicast link after path switching from PC5 path to Uu path. In this case, the UEs can determine to switch back to PC5 path from Uu path based on e.g. the path switching policy, the PC5 signal level of the maintained unicast link.

Editor's Note:	It is FFS how to support the notification of UE when it cannot perform path switch.

* * * * End of Changes * * * *