SA WG2 Meeting #S2-155	S2-2303055

# Athens, Greece, Feb 20 – 24, 2023,										     (revision of S2-23001381, 0487r07)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3919 | rev | 4 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of KI#6 conclusion: uplink-downlink transmission coordination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [OPPO, Tencent, Tencent Cloud, Nokia, Nokia Shanghai Bell, vivo, Huawei], Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusion for uplink-downlink transmission coordination to meet Round-Trip latency requirements has been agreed in TR 23700-60. Remove the EN to support the correlated QoS monitoring: Editor’s Note: How the PCF correlates the QoS Monitoring measurements to derive the round-trip latency is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds uplink-downlink transmission coordination to meet the Round-Trip latency requirements of XRM service based on the TR conclusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to support uplink-downlink transmission coordination to address the Round-Trip latency requirements of XRM service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

5.37.x Uplink-Downlink Transmission to meet RT Latency Requirement

For XR and other interactive media services that require very low Round-Trip (RT) latency, Uplink-Downlink Transmission is supported to meet the RT latency requirement. RT latency requirement is the upper bound for the sum of UL delay and DL delay of an application flow between UE and N6 termination point at the UPF. PCF shall support Uplink-Downlink Transmission to meet RT latency requirement based on the local configuration or an RT latency indication from AF during the AF session with the required QoS procedure as defined in clause 4.15.6.6 of TS 23.502 [3].

The AF can provide an RT latency indication with a single direction delay requirement between the UE and the PSA UPF expressed as the QoS Reference parameter or individual QoS parameter (as defined in clause 6.1.3.22 of TS 23.503 [45]). The RT latency indication indicates the application flow needs to meet the RT latency requirement, that is, the doubling of the single direction delay requirement between the UE and the PSA UPF expressed by the QoS Reference parameter or individual QoS parameter.

PCF determines the application flow's UL PDB and DL PDB based on the RT latency requirement. The UL PDB and DL PDB can be unequal, but their sum shall not exceed the RT latency requirement. The PCF shall generate two separate PCC rules for the application flow, one for the UL direction and one for the DL direction. PCF assigns the 5QIs for these two PCC rules according to the derived UL PDB and DL PDB. To support UL and DL delay tracking, QoS monitoring (as defined in clause 6.1.3.21 of TS 23.503 [45]) shall be triggered for each PCC rule to track the UL and DL packet delay independently. Based on the QoS monitoring results, the PCF may adjust the UL PDB and DL PDB under the consideration of the RT latency requirement to better fit the new situation.

In order to support the correlated QoS monitoring of UL and DL in the similar time, the indication to indicate that the QoS monitoring policies are generated and enforced at the same time, or the period time to trigger the QoS monitoring reports periodically, or the correlation ID for getting the QoS monitoring results at the similar time, can be used. It can be provided by the AF, or determined by the PCF based on the local configuration and operator policy.

* * * End of Changes * * *