3GPP TSG-WG SA2 Meeting #155                                                          S2-2303132

Athens, Greece February 20-24, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4182 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update TS23.501 for PDU Set and PDU Set Importance handling by the UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2022-2-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At S2-154e-AH, 23.501 clause 5.8 was updated to include PDU Set related text for Traffic detection, FARs and PDRs (see S2-2301472). However those changes do not describe how the UPF determines PDU Set parameters (parameter mapping), the dependency of PDU Set parameters on the various service protocols indicated in the PDR Protocol Description, how PDU Set Importance is determined, the needed granularity for PDU Set Importance or how the UPF obtains the information it needs to do mapping to the GTP-U header. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.8.2.x is introduced to describe PDU Set Handling by the UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification of user plane handling for PDU Sets is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change (all new next) * * * *

## 5.8.2.x		PDU Set Handling for XRM

The PSA UPF identifies PDUs that belong to PDU Sets and determines PDU Set Information which it sends to the RAN in the GTP-U header of each PDU identified as belonging to a PDU Set. The identification can be done by UPF implementation or by detecting RTP/SRTP header or payload as described in clause 5.8.2.4.2. When RTP/SRTP header or payload is used to identify PDUs that belong to PDU Sets, the Service Protocol indicated by the PDR “Protocol Description” attribute (see clause 5.8.2.11.3), specifies the RTP/SRTP header and payload used by the service data flow. The UPF applies rules for PDU Set Identification (see clause 5.37.x.1) based on the RTP/SRTP header and payload.

NOTE: Since more than one SDF may be mapped to a QoS Flow, the SMF may specify more than one Service Protocol to be detected within a QoS Flow.

Rules for PDU Set identification are specific to each Service Protocol supported by a UPF. These rules, which are provided by local configuration at the UPF, specify a mapping from Service Protocol specific header and payload fields to PDU Set Information sent in the GTP-U header. The Service Protocols supported include the RTP/SRTP header and payload as defined by:

- RFC 3550:  RTP [X1]

- RFC 3711:  SRTP [X2]

- RFC 6184:   RTP Payload Format for H.264 Video [X3]

- RFC 7798:  RTP Payload Format for High Efficiency Video Coding [X4]

- draft-ietf-avtcore-rtp-vvc:  RTP Payload Format for Versatile Video Coding [X10]

- draft-ietf-avtext-framemarking: Frame Marking RTP Header Extension [X7]

- AV1 RTP payload format: RTP payload format for the AV1 video codec [X11]

Within a QoS Flow, PDU Set Importance is the only information conveyed in the GTP-U header that differentiates PDU Sets that have different application layer characteristics (e.g. an Independent vs Dependent video frame). For each supported Service protocol, local configuration at the UPF may specify mapping rules from Service Protocol specific RTP/SRTP header and payload attributes to PDU Set Importance values. For example, in the case of the draft-ietf-avtext-framemarking [X7] Service Protocol, fields in the Long RTP Extension for Scalable Streams indicate whether a PDU coveys a frame that is independent, discardable, part of a spatial base layer or one or more spatial enhancement layers (8 bits), or part of a temporal base layer or one or more temporal enhancement layers (3 bits). Local configuration for this Service Protocol determines how values populated by the application in one or more of these fields are mapped to PDU Set Importance values ranging from 0 (most important) to 15 (least important).

When PDU Set Information Marking is indicated in the FAR, a Service Protocol is indicated in the PDR and mapping rules have been configured for the indicated Service Protocol, the UPF determines the PDUs received on N6 that match the PDR, and applies the mapping rules to determine the PDU Set parameters sent to the RAN in the GTP-U header.

Mapping rules are UPF implementation dependent and may be based on PDU Set identification for different service protocols as described in Annex X.

In addition to determining the PDU Set Importance based on specific Service protocols, the UPF may also do it in any other implementation specific manner upon local configuration. One example would be using IP DiffServ Code Point (DSCP) values.

# * * * * End of change * * * *