3GPP TSG-WG SA2 Meeting #155 	S2-2303186

February 20 – 24, 2023, Athens, GR	(revision of S2-2302898)

Source:	Sony, Huawei, HiSilicon, Interdigital, vivo

Title:	Introduction of Network assisted Sidelink Positioning

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper introduces the feature Network assisted Sidelink Positioning in TS 23.586.

# 1. Introduction/Discussion

This paper proposes to add the feature Network assisted Sidelink Positioning based on the KI#5 conclusion in TR 23.700-86.

# 2. Text Proposal

It is proposed to capture the following changes in TS 23.586.

# * * * * First change* * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[6]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[7]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[x]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

# * * * * Second change* * * *

## 5.x	Network assisted Sidelink Positioning

### 5.x.1 General

Editor’s Note: The final name of the term "Network assisted Sidelink Positioning" is FFS.

This feature is used to estimate the location of a UE with the assistance of the network by using the location of a Located UE and the distance and direction between the UE and the Located UE.

The Network assisted SL Positioning feature has two cases:

-	When the UE can establish a NAS signalling connection, or

-	When the UE cannot establish a NAS signalling connection

For Ranging, UE assisted mode, UE based mode or standalone mode can be used to estimate the distance and direction between the UE and the Located UE(s). These modes are defined in TS 23.273 [8].

UE assisted mode, UE based mode or Network assisted positioning can be used to estimate the location of the Located UE as specified in TS 23.273 [8].

In this release, Network Assisted SL Positioning is only supported when Target UE and Located UE are registered in the same PLMN i.e., no support for inter PLMN Network Assisted SL Positioning when the Target UE and Located UE are registered in different PLMNs. Roaming is supported when Target UE and Located UE are registered in the same PLMN, e.g., Target UE is registered in a VPLMN and Located UE is registered in its HPLMN.

### 5.x.2 Network assisted SL Positioning of UE with NAS connection

A UE with a NAS signalling connection is in CM-Connected state. The UE enters CM-Connected state by performing UE triggered Service Request for MO-LR or performing Network triggered Service Request for NI-LR or MT-LR. As the Target UE can establish a NAS signalling connection with the AMF, the functionality specified in TS 23.273 [8] for location services can be reused including e.g. MO-LR, MT-LR and NI-LR with the additional functionality captured in this clause.

-	The Location Service is triggered via the AMF serving the Target UE. The location request comes either from an AF/external client via the GMLC, a 5G NF, or the Target UE.

-	The LTE positioning protocol (LPP) as specified in TS 37.355 [x] is used between the Target UE and the LMF. When LCS procedures are used to estimate the location of the Located UE, LPP is used between Located UE and LMF.

Editor’s Note: RAN WGs will determine whether and what enhancements and the subset functionalities of LPP are needed to support Network assisted SL positioning including an MT-LR, MO-LR and NI-LR.

-	Either the Target UE or LMF determines if network assisted SL positioning will be applied. When LMF determines that network assisted SL positioning is used, LMF may trigger the Target UE to perform the discovery of Located UE(s).

-	The Target UE discovers Located UE(s) for network assisted SL positioning. Optionally, to assist the Target UE the LMF may provide to the Target UE a list of candidate Located UE(s). In this case the LMF maintains the candidate list of Located UE(s) including e.g. the capability and location information of the Located UE(s).

-	The Target UE and Located UE(s) perform ranging/SL positioning. The Target UE includes the UE identity of the Located UE(s) to the LMF together with the Ranging measurement data or estimation result. The LMF may interact with GMLC to get the location of Located UE.

Editor’s Note: It is FFS what UE identity of the Located UE that the Target UE provides to the LMF. Depending on which UE identity, the LMF or the GMLC may need to resolve this by query another NF. The security impact will be coordinated with SA3.

-	The LMF uses the location of Located UE(s) together with the ranging/SL positioning measurement data or estimation results reported by Target UE and optionally also by Located UEs to estimate the location of the Target UE.

- 	In case the Located UE is a non-stationary UE, the LMF interacts with GMLC to get the location of Located UE, and the Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same time, as specified in TS 23.273 [8], to improve the Target UE positioning accuracy.

NOTE:	The LMF may need to compensate the time difference of the positioning of Target UE and Located UE with additional information.

-	The LMF provides the UE location estimate to the Target UE over LPP or the AMF forwards the UE location to the GMLC, or to the 5GC NF according to legacy functionality.

# * * * * End of changes * * * *