3GPP TSG-WG SA2 Meeting #155 	S2-2303229

February 20 – 24, 2023, Athens, Greece	(revision of S2-2303187/S2-2202745)

Source:	vivo, Interdigital, Sony

Title:	TS 23.586: Network assisted SL positioning without NAS connection

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper implements the conclusions in KI#5 Network assisted SL Positioning about the Target UE has no NAS connection.

1. Introduction/Discussion

This paper implements the following conclusions in KI#5 Network assisted SL Positioning.

-	If Target UE cannot establish NAS connection with AMF, for MO-LR or pending MT-LR (e.g. deferred MT-LR), the following procedures can be used:

Option a):

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE may request for the Location of the Located UE(s).

-	If the Located UE(s) needs network assistance to retrieve its location, the Located UE(s) may trigger a MO-LR. This assumes that the Located UE(s) has a NAS connection to an AMF.

-	The Target UE uses the location returned by Located UE(s) together with the ranging/SL positioning measurement data or result to estimate its own location. The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same time (according to TS 23.273 [11]) to improve the Target UE positioning accuracy.

Option b):

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE may transmit its ranging measurements/results to the Located UE(s).

-	Located UE(s) may report the ranging/SL positioning measurement result to the LMF. This may include ranging measurements/results received from the Target UE. The endpoints for LPP messages can be the LMF and the Located UE(s) in this case.

-	The LMF may use the received information to calculate the location of the Target UE, and provide the resulting location via the Located UE to the Target UE or via 5G NF to the LCS client or the Application Server.

NOTE 6:	Security aspects require confirmation from SA WG3.

NOTE 7:	Supporting of location report via the Located UE to the LMF needs to be confirmed by RAN.

NOTE 8:		It will be determined during Normative phase that when AMF does not maintain target UE's context or GMLC cannot get target UE AMF address because of deregistration, whether and how location report from Located UE on behalf of Target UE is supported.

Above conclusion is initiated by Target UE itself and there isn’t NAS connection between Target UE and AMF, these above related processes are not suitable to be classified to the Network assisted Ranging. Option A and B can have no network involved in, at most the network is involved in to calculated the Ranging result and at most it can be called Network based. So for above conclusions, it is proposed to use a sperate clause to differentiate the Network Assisted Ranging, which may be more suitable.

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

5.X.3	Network assisted SL positioning without NAS connection

When Target UE cannot establish the NAS connection with AMF, for 5GC-MO-LR or pending 5GC-MT-LR (e.g. deferred 5GC-MT-LR), the following principles are applied:

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE may transmit its ranging measurements/results to the Located UE(s).

-	Located UE(s) may report the ranging/SL positioning measurement result to the LMF. This may include ranging measurements/results received from the Target UE. The endpoints for LPP messages are the LMF and the Located UE(s).

-	The LMF may use the received information to calculate the location of the Target UE, and provide the resulting location via the Located UE to the Target UE or via 5G NF to the LCS client or the Application Server.

Editor’s Note: It is FFS if an enhanced 5GC-MO-LR message is sent by the Located UE.

Editor’s Note: Security aspects require confirmation from SA WG3.

Editor’s Note: It is FFS when AMF does not maintain target UE's context or GMLC cannot get target UE AMF address because of deregistration, whether and how location report from Located UE on behalf of Target UE is supported.

Editor’s Note: How LMF associates the ranging/SL positioning measurement or estimation result from different Located UEs is FFS.

Editor’s Note: It is FFS whether the pending 5GC-MT-LR also includes the 5GC-MT-LR.

# * * * * End of changes * * * *