# 3GPP TSG-WG SA2 Meeting #155	S2-2303269

# Athens, Greece, February 20 - 24,  2023 		(revision of S2-2302817)

| # CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | # 23.502 | # CR | # 3919 | # rev | # 1 | # Current version: | # 18.0.0 |  |
| # For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| # Proposed change affects: | # UICC apps |  | # ME |  | # Radio Access Network |  | # Core Network | # X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| # Title: | # Event exposure for bulk subscription from the PCF to the AF in TS 23.502 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Source to WG: | # China Mobile, Nokia, Nokia Shanghai Bell, ZTE |  |  |  |  |  |  |  |  |  |
| # Source to TSG: | # SA2 |  |  |  |  |  |  |  |  |  |
| # Work item code: | # TEI18_ADEE |  |  |  |  |  | # Date: |  |  | # 2023-01-31 |
| # Category: | # B |  |  |  |  |  | # Release: |  |  | # Rel-18 |
|  | # Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) # Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | # Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| # Reason for change: |  | # The event exposure for bulk subscription from the PCF to the AF directly or via NEF may be useful for the AF to enforce application layer logic and the related description is also specified in TS 23.503 as following: # +++ # If the AF requests the PCF to report on the outcome of the service area coverage change, the PCF reports the outcome of the service area coverage change to the AF and notifies the current service area coverage to the AF. The outcome is the result of the execution of the request of service coverage change at the PCF; the outcome is successful if the request was executed, and includes the current service area coverage that may be the same or different from the service area coverage provided by the AF. The subscription may also be implicit. In this case there may be bulk subscription, either for an Internal-Group-Id or for any UE. In order to prevent massive notifications to the AF, the request for any UE is associated to a specific Application Identifier or DNN, S-NSSAI. For bulk subscription, when the AF request includes an expiration time, the PCF stops reporting to the AF when the expiration time is reached. # +++ # But how the procedure works is still not clear in TS 23.502, therefore, it is proposed to adding the procedure about the event exposure for bulk subscription from the PCF to the AF directly or via NEF. |  |  |  |  |  |  |  |  |
| # Summary of change: |  | - Add the procedure that the AF subscribes to notifications of events from the PCF with the bulk subscription directly or via NEF. - Fix the consumer of the corresponding service operation. |  |  |  |  |  |  |  |  |
| # Consequences if not approved: |  | # How the AF subscribes to notifications of events from the PCF with the bulk subscription is missing. |  |  |  |  |  |  |  |  |
| # Clauses affected: |  | # 4.15.3.2.4, 5.2.5.1, 4.5.3.1 |  |  |  |  |  |  |  |  |
|  |  | # Y | # N |  |  |  |  |  |  |  |
| # Other specs |  |  | # X | # Other core specifications |  |  |  | # TS/TR ... CR ... |  |  |
| # affected: |  |  | # X | # Test specifications |  |  |  | # TS/TR ... CR ... |  |  |
| # (show related CRs) |  |  | # X | # O&M Specifications |  |  |  | # TS/TR ... CR ... |  |  |
| # Other comments: |  |  |  |  |  |  |  |  |  |  |
| # This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 4.15.3.2.4	Exposure with bulk subscription

Based on operator configuration NEF may perform bulk subscription with the NFs that provides necessary services. This feature is controlled by local policies of the NEF that control which events (set of Event ID(s)) and UE(s) are target of a bulk subscription.

When the NEF performs bulk subscription (subscribes for any UE (i.e. all UEs), group of UE(s) (e.g. identifying a certain type of UEs such as IoT UEs)), it subscribes to all the NFs that provide the necessary services (e.g. In a given PLMN, NEF may subscribe to all AMFs that support reachability notification for IoT UEs). Upon receiving bulk subscription from the NEF, the NFs store this information. Whenever the corresponding event(s) occur for the requested UE(s) as in bulk subscription request, NFs notify the NEF with the requested information.

The following call flow shows how network exposure can happen for one UE, groups of UE(s) (e.g. identifying a certain type of UEs such as IoT UEs) or any UE.

Figure 4.15.3.2.4-1: NF registration/status notification and Exposure with bulk subscription

1.	NEF subscribes to the NRF for any newly registered NF along with its NF services.

2.	When an NF instantiates, it registers itself along with the supported NF services with the NRF.

3.	NRF acknowledges the registration

4.	NRF notifies the NEF with the newly registered NF along with the supported NF services.

5.	NEF evaluates the NF and NF services supported against the pre-configured events within NEF. Based on that, NEF subscribes with the corresponding NF either for a single UE, group of UE(s) (e.g. identifying a certain type of UEs such as IoT UEs), any UE. NF acknowledges the subscription with the NEF.

6 - 7.	When the event trigger happens, NF notifies the requested information towards the subscribing NEF along with the time stamp. NEF may store the information in the UDR along with the time stamp using either Nudr_DM_Create or Nudr_DM_Update service operation as appropriate.

8.	Application registers with the NEF for a certain event identified by event filters. If the registration for the event is authorized by the NEF, the NEF records the association of the event and the requester identity.

9 - 10.	When the event trigger happens, NF notifies the requested information towards the subscribing NEF. NEF may store the information in the UDR using either Nudr_DM_Create or Nudr_DM_Update service operation as appropriate.

11a-b.	NEF reads from UDR with Nudr_DM_Query and notifies the application along with the time stamp for the corresponding subscribed events.

The application detection event exposure from PCF may be used by an AF for the detection of application start or stop event for group of UE or any UE. In this case, the AF may subscribe to notifications of events from the PCF with the bulk subscription as described in clause 6.1.3.18 of TS 23.503 [20].. The corresponding call flow is as follows:

Figure 4.15.3.2.4-2: bulk subscription to notifications of events from the PCF by the AF

- AF may send request to NEF to subscribe to notifications of events via Nnef_EventExposure_Subscribe/Unsubscribe(Event ID, Event Filter, Event Reporting information, Target of Event Reporting, A Notification Target Address (+ Notification Correlation ID)). The Event ID indicates the Application Detection. The Event Filters includes the Flow description information or External Application Identifier of the application to be detected. The Target of Event Reporting may indicate a group of UE(s) or any UE (i.e. all UEs). The AF may also provides the S-NSSAI and DNN or AF-identifier to the NEF to identify the PDU Session.

2.	The NEFresolves the the External Group Identifier into the Internal Group Identifier using Nudm_SDM_Get (Group Identifier Translation, External Group Identifier).

3.	The NEF dicover the existing PCFs supporting the S-NSSAI and DNN.

4.	The NEF registers with the NRF for any newly registered PCF along with its NF services.

5.	When a new PCF instantiates, it registers itself along with the supported NF services with the NRF.

6.	NRF notifies the NEF with the newly registered PCF along with the supported NF services.

7.	NEF evaluates the PCF supported against the bulk subscription to notification of application detect events. Based on that, NEF subscribes with the group of UE(s) or any UE. PCF acknowledges the subscription with the NEF.

8.	The AF may send request directly to existing PCF to subscribe to notifications of events via Npcf_EventExposure_Subscribe/Unsubscribe with the group of UE(s) or any UE. The PCF acknowledges the subscription with the AF.

9.	When a PDU session is established, the serving SMF may trigger the SM Policy Association Establishment procedure as specified in clause 4.16.4, and the PCF may provide to the SMF the related PCRT on the notifications of events.

Additionally, the PCF may also update the policy information of those PDU Sessions that are already established for users of the group to subscribe to the SMF on the notifications of events.

10.	When the event trigger happens, for example, the SMF receives the report from UPF about detecting the start or the stop of application traffic, SMF triggers the SM Policy Association Modification procedure as specified in clause 4.16.5.1, to report the requested information towards the PCF.

1112.	PCF notifies AF of the requested information via NEF.

PCF may remove the PCRT from the SMF when the PCF receives the notification of the event for the first time.

13.	PCF notifies AF of the requested information.

# * * * * Second change * * * *

5.2.5.1	General

The following table illustrates the PCF Services.

Table 5.2.5.1-1: NF services provided by PCF

| Service Name |  |  | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- | --- | --- |
| Npcf_AMPolicyControl |  |  | Create | Request/Response | AMF |
|  |  |  | Update | Request/Response | AMF |
|  |  |  | UpdateNotify | Subscribe/Notify | AMF |
|  |  |  | Delete | Request/Response | AMF |
| Npcf_Policy Authorization |  |  | Create | Request/Response | AF, NEF, TSCTSF |
| (NOTE) |  |  | Update | Request/Response | AF, NEF, TSCTSF |
|  |  |  | Delete | Request/Response | AF, NEF, TSCTSF |
|  |  |  | Notify | Subscribe/Notify | AF, NEF, NWDAF, PCF, TSCTSF |
|  |  |  | Subscribe |  | AF, NEF, NWDAF, PCF, TSCTSF |
|  |  |  | Unsubscribe |  | AF, NEF, NWDAF, PCF, TSCTSF |
| Npcf_SMPolicyControl |  |  | Create | Request/Response | SMF |
|  |  |  | UpdateNotify | Subscribe/Notify | SMF |
|  |  |  | Update | Request/Response | SMF |
|  |  |  | Delete | Request/Response | SMF |
| Npcf_BDTPolicyControl |  |  | Create | Request/Response | NEF |
|  |  |  | Update | Request/Response | NEF |
|  |  |  | Notify |  | NEF |
| Npcf_UEPolicyControl |  |  | Create | Request/Response | AMF, V-PCF |
|  |  |  | Update | Request/Response | AMF, V-PCF |
|  |  |  | UpdateNotify | Subscribe/Notify | AMF, V-PCF |
|  |  |  | Delete | Request/Response | AMF, V-PCF |
| Npcf_EventExposure |  |  | Subscribe | Subscribe/Notify | AF, NEF, NWDAF |
|  |  |  | Unsubscribe |  |  |
|  |  |  | Notify |  |  |
| Npcf_AMPolicyAuthorization |  |  | Create | Request/Response | AF, NEF, TSCTSF |
|  |  |  | Update | Request/Response | AF, NEF, TSCTSF |
|  |  |  | Delete | Request/Response | AF, NEF, TSCTSF |
|  |  |  | Notify | Subscribe/Notify | AF, NEF, 5G DDNMF |
|  |  |  | Subscribe |  | AF, NEF, 5G DDNMF |
|  |  |  | Unsubscribe |  | AF, NEF, 5G DDNMF |
| Npcf_PDTQPolicyControl |  |  | Create | Request/Response | NEF |
|  |  |  | Update | Request/Response | NEF |
|  |  |  | Notify |  | NEF |
|  | NOTE: In the Npcf_PolicyAuthorization operations, PCF is a consumer when the PCF for the UE and the PCF for the PDU session are different. |  |  |  |  |

# * * * * Third change * * * *

4.15.3.1	Monitoring Events

The Monitoring Events feature is intended for monitoring of specific events in 3GPP system and making such monitoring events information reported via the NEF. It is comprised of means that allow NFs in 5GS for configuring the specific events, the event detection and the event reporting to the requested party.

To support monitoring features in roaming scenarios, a roaming agreement needs to be made between the HPLMN and the VPLMN. If the AMF/SMF in the VPLMN determine that normalisation of an event report is required, the AMF/SMF normalises the event report before sending it to the NEF.

The set of capabilities required for monitoring shall be accessible via NEF to NFs in 5GS. Monitoring Events via the UDM, the AMF, the SMF, the NSACF and the GMLC enables NEF to configure a given Monitor Event at UDM, AMF, SMF, NSACF or GMLC and reporting of the event via UDM and/or AMF, SMF, NSACF or GMLC. Depending on the specific monitoring event or information, it is the AMF, GMLC, NSACF or the UDM that is aware of the monitoring event or information and makes it reported via the NEF.

The following table enumerates the monitoring events and their detection criteria:

Table 4.15.3.1-1: List of events for monitoring capability

| Event | Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Loss of Connectivity | Network detects that the UE is no longer reachable for either signalling or user plane communication (see NOTE 4). The AF may provide a Maximum Detection Time, which indicates the maximum period of time without any communication with the UE after which the AF is to be informed that the UE is considered to be unreachable (see NOTE 7). If Unavailability Period Duration has been provided by the UE, the AMF uses the remaining value of it to determine the foreseen Loss of Connectivity time as described in clause 5.4.1.4 of TS 23.501 [2]. | AMF |
| UE reachability | Detected when the UE transitions to CM-CONNECTED state or when the UE will become reachable for paging, e.g. Periodic Registration Update timer. It indicates when the UE becomes reachable for sending downlink data to the UE. The AF may provide the following parameters: 1) Maximum Latency; 2) Maximum Response Time; 3) Suggested number of downlink packets. (see NOTE 5 and NOTE 7). This event requires the Reachability Filter set to UE reachable for DL traffic" (see clause 5.2.2.3.1-1). For the usage of this event, see clauses 4.2.5.2 and 4.2.5.3. When requesting UE reachability monitoring, the AF may in addition request Idle Status Indication to be included in the UE reachability event reporting. | AMF, UDM |
| Location Reporting | This event is detected based on the Event Reporting Information Parameters that were received in the Monitoring Request (one-time reporting, maximum number of reports, maximum duration of reporting, periodicity, etc. as specified in clause 4.15.1). It reports either the Current Location or the Last Known Location of a UE. When AMF is the detecting NF: One-time and Continuous Location Reporting are supported. For Continuous Location Reporting the serving node(s) sends a notification every time it becomes aware of a location change, with the granularity depending on the accepted accuracy of location (see NOTE 1). For One-time Reporting with immediate reporting flag set, AMF reports the Last Known Location immediately. When AMF is the detecting NF: If the immediate reporting flag is not set, the AMF reports the UE Current Location (In case the AMF does not have the UE current location in the granularity as requested by the location report, the AMF retrieves the information via NG-RAN Location reporting procedure as defined in clause 4.10). When GMLC is the detecting NF: Immediate and Deferred Location Reporting is supported. For Deferred Location Reporting the event types UE availability, Area, Periodic Location and Motion are supported. | AMF, GMLC |
| Change of SUPI-PEI association | This event is detected when the association between PEI and subscription (SUPI) changes (USIM change). | UDM |
| Roaming status | This event is detected based on the UE's current roaming status (the serving PLMN and/or whether the UE is in its HPLMN) and notification is sent when that status changes. (see NOTE 2). If the UE is registered via both 3GPP and N3GPP Access Type, then both instances of Roaming status are included. | UDM |
| Communication failure | This event is detected when RAN or NAS level failure is detected based on connection release and it identifies RAN/NAS release code. | AMF |
| Availability after Downlink Data Notification failure | This event is detected when the UE becomes reachable again after downlink data delivery failure. When requesting Availability after Downlink Data Notification failure monitoring, the AF may in addition request Idle Status Indication to be included in the UE reachability event reporting. | AMF |
| PDU Session Status | This event is detected when PDU session is established or released. (see NOTE 6) | SMF |
| Number of UEs present in a geographical area | This event is detected based on the Event Reporting Information Parameters that were received in the Monitoring Request (Level of aggregation, Sampling ratio, see clause 4.15.1). It indicates the number of UEs that are in the geographical area described by the AF. The AF may ask for the UEs that the system knows by its normal operation to be within the area (Last Known Location) or the AF may request the system to also actively look for the UEs within the area (Current Location). | AMF |
| CN Type change | The event is detected when the UE moves between EPC and 5GC. It indicates the current CN type for a UE or a group of UEs when detecting that the UE switches between being served by a MME and an AMF or when accepting the event subscription. (see NOTE 3) | UDM |
| Downlink data delivery status | It indicates the downlink data delivery status in the core network. Events are reported at the first occurrence of packets being buffered, transmitted or discarded, including: - Downlink data in extended buffering, including: - First data packet buffered event - Estimated buffering time, as per clause 4.2.3.3 - First downlink data transmitted event - First downlink data discarded event | SMF |
| UE reachability for SMS delivery | For SMS over NAS, this event is detected when an SMSF is registered for a UE and the UE is reachable as determined by the AMF and the UDM. For SMS over IP, the event is detected when the UE is reachable as determined by the AMF and the UDM regardless of an SMSF being registered. This enables the UE to receive an SMS. See clauses 4.2.5.2 and 4.2.5.3 (see NOTE 8). | UDM |
| UE memory available for SMS | This event is detected when the UE memory is available for SMS as determined by the AMF and the UDM. This enables to trigger Alert for SMS. See clause 5.1.8 of TS 23.540 [84] | UDM |
| Number of registered UEs or established PDU Sessions | It indicates the current number of registered UEs or established PDU Sessions for a network slice that is subject to NSAC. For One-time Reporting with Immediate Reporting Flag set, NSACF reports the number of registered UEs or established PDU Sessions immediately. | NSACF |
| Area Of Interest | It indicates change of the UE presence in the Area Of Interest. | AMF, GMLC |
| Group Member List Change | It indicates the changes on the members of the group. This event apply to a group of UEs (identified by an External Group ID), such as 5G VN group (see NOTE 9) or other groups. | UDM |
| Group-MBR monitoring | The bit rates of a set of specific QoS flows of a group of UEs' PDU sessions are reported by the UEs' respective serving PSA UPFs to NEF. The collected bit rates reported by UPFs are aggregated by the NEF. The aggregated bit rate is then compared against the AF provisioned Group-MBR threshold. If the aggregated bit rate exceeds the Group-MBR threshold, the NEF notifies the AF for such NEF event. | AF |
| Application Detection | Detection of application start or stop (See NOTE 10). | PCF |
| NOTE 1: Location granularity for event request, or event report, or both could be at cell level (Cell ID) or TA level. The granularity can also be expressed by other formats such as geodetic uncertainty shapes (e.g. polygons, circles, etc.) or civic addresses (e.g. streets, districts, etc.) which can be mapped by NEF to AMF specific granularity levels. NOTE 2: Roaming status means whether the UE is in HPLMN or VPLMN based on the most recently received registration state in the UDM. NOTE 3: CN type of CN Type change event is defined in clause 5.17.5.1 of TS 23.501 [2]. NOTE 4: In the case of UDM service operation information flow, the UDM should set the subscribed periodic registration timer to a smaller value than the value of Maximum Detection Time, since the value of the mobile reachable timer is larger than the value of the periodic registration timer. NOTE 5: Maximum Latency, Maximum Response Time and Suggested number of downlink packets are defined in clause 4.15.6.3a. NOTE 6: The NEF makes a mapping between the 5GS internal event "PDU Session Status" and the T8 API event "PDN Connectivity Status". NOTE 7: The preferred method for provisioning Network Configuration Parameters is External Parameter Provisioning specified in clause 4.15.6.3a. Provisioning event specific parameters as part of Monitoring Request is expected to be used only by the AF that does not support Parameter Provisioning procedure specified in clause 4.15.6.3a. NOTE 8: The NEF maps between the T8 API event "UE reachability" with reachability type SMS and the 5GS internal event "UE reachability for SMS delivery" for SMS over NAS. The event "UE reachability for SMS delivery" for SMS over IP is used by HSS as described in clause 5.5.6.3 of TS 23.632 [68]. NOTE 9: 5G VN group management is defined in the clause 5.29.2 of TS 23.501 [2]. NOTE 10: This event uses bulk subscription/notification in the PCF which may impact the PCF performances |  |  |

# * * * * End of changes * * * *