3GPP TSG-WG SA2 Meeting #155	S2-2303328

# 20 – 24 February, 2023, Athens, Greece		(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0326 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LCS User Plane Functions and Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-010 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR tries to merge the current view on user plane positioning including: - NAS and data channel for the UP connection assistance - The choice of persistent connection by UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Diagram for the user plane procedures The trigger of the related element and call flows |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The user plane options can’t be merged in R18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (no update)* * * *

# * * * * Next change (all new)* * * *

### 6.11.x	User Plane Positioning procedures by user plane notification

Figure 6.11.x-1 shows a procedure which may be used by an LMF to support user plane positioning with UE. The procedure may be using LPP protocol in TS 37.355 [20] between the LMF and UE. Before the user plane procedures, UE should have included its user plane capability in its registration process and it has URSP rule for the PDU session of the user plane connection.

Figure 6.11.x-1: User Plane procedures between LMF and UE

1.	LCS session setup as defined in clause 6.1, 6.2, 6.3 and 6.10.

2.	The AMF invokes the Nlmf_Location_DetermineLocation service operation towards the LMF. To support user plane positioning, AMF may take UE user plane capability, whether UE is authorized to use user plane, the UE’s identity information may also include UE IP and/or SIP address.

NOTE 1: AMF may store these extra parameters in UE context or query UDM/BSF to fetch the information.

3.	LMF makes decision whether to use user plane positioning with UE. In some cases, e, g. some regulatory sessions, LMF may use user plane positioning when the UE is capable but not authorized for user plane.

- Based on the indication of UE’s user plane capability parameter in Nlmf_Location_DetermineLocation,  LMF may decide to use user plane positioning before any LPP sessions with the UE.

- Based on LPP capability exchange between the UE, LMF may also decide to use user plane positioning after some LPP sessions with the UE.

NOTE 2:	It is implementation specific when and how LMF decides to use user plane during the LCS session (between step 2 and step 8).

4.	LMF notifies the UE with assistance information to setup user plane connection. The assistance information may take the LMF address and security attributes/parameters for secure connection. When UE is pre-configured with sufficient information to setup/reuse the secure user plane connections, the notification message could have empty parameters (i.e., without LMF address or security attributes).

4.a	LMF could send notification over UDP/IP or SIP message to UE. This could be applicable in cases (like PNI NPN) that information to setup the secure user plane should not be revealed outside the local area data network.

NOTE 3: Step 5 can be skipped if UE is preconfigured sufficient information to setup the secure user plane connection.

5. When assistance information is available to UE:

5.a.1~5.a.2	UE may use DNS to connect to different (e.g., LMF in public or local network) LMF. For instance, EASDF/DNS resolver may reply LMF instance in public or local data network according to UE’s public or local address as per clause 6.2 of TS 23.548.

NOTE 4: as defined in clause 5.2.1 in TS 23.548, UE may also subscribe to DNS resolver for the update of user plane service address.

5.b		UE could also be pre-configured or assisted with direct LMF (e.g., IP) address, UE may choose an existing (previously setup) user plane connection, or setup a new PDU session and/or user plane connection based on URSP and/or related settings.

NOTE 5:	Even after the user plane decision, LMF may still reject the user plane connection during the secure user plane connection setup phase. It is implementation specific, for instance, LMF may reject UE connection from certain source addresses.

6. UE chooses an existing (previously setup) user plane connection, or setup a new PDU session and/or user plane connection based on URSP and/or related settings

7. LMF performs LPP session with UE.

NOTE 6: LMF may also perform NRPPa session with RAN node(s) alongside the LPP session.

NOTE 7: LMF may report location result towards (local) GMLC using Nlmf_Location_EventNotify service operation.

8. LMF replies the Nlmf_Location_DetermineLocation request as defined in clause 6.1, 6.2, 6.3 and 6.10.

NOTE 8: it is implementation specific that UE and LMF may keep the user plane connection to be reused in successive sessions or tear it down after step 8.

Editor's note:	Suitable security attributes/parameters needs to be determined by SA WG3. The type of user plane protocol needs to be determined at stage 3.

# * * * * End of changes * * * *