3GPP TSG-SA WG2 Meeting #155		S2-2303353

# Athens, GR, Februrary 20 - 24, 2023					 (rev of S2-2302326, 1453, 00331r05)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0160 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing functionality of resource efficiency in MOCN Network Sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson , ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | #1 In TR 23.700-47 v2.0.0, KI#4 MOCN Network Sharing has been concluded as follows to achieve resource efficiency: 8.2 Key Issue #2: MOCN network sharing For conclusions, the following aspects will be considered: - For solutions where the broadcast MBS sessions for different PLMNs are established towards a NG-RAN node, the NG-RAN node shall be able to identify the same MBS service and avoid multiple deliveries over radio. - A solution compatible with Rel-17 UEs is preferred. - A solution compatible with Rel-17 NG-RAN is preferred. - The AF may provide associated session identifier (SSM used by AF) additionally to the NG-RAN nodes via 5GC so that the shared NG-RAN nodes can determine that the multiple broadcast MBS sessions are transmitting same content for the same MBS service (i.e., Soln#2 and Soln#7 SSM option), or - The association of MBS session identifiers may be configured in NG-RAN, where there is no requirement on AF to provide associated session identifier. - It should be possible not to establish all the shared delivery tunnels to the same NG RAN from different PLMNs for the same MBS service. - The solution should support the scenario where all NG-RAN nodes are shared by PLMNs and the scenario where only part of the NG-RAN nodes are shared by PLMNs. This CR propose the update to TS 23.247 based on the conclusion above and focus on the update to the following: clause 5.3 for enhancement NF description, Clause 6.9.1 for enhancement in MBS Session Context clause 6.5.x for new Associated Session ID, clause 7.3.x, a new subclause for “Broadcast MBS Session Transport in Network Sharing”, and Clause 9 for service operations. NOTE that TS 23.247 CR0176 focuses on 6.x for description of the new function, and TS 23.247 CR0165 focuses on enhancement to procedures except for “Broadcast MBS Session Transport in Network Sharing”. #2 For location dependent service, especially when multiple area sessions are served by one NG-RAN, it is not clear how to handle. #3 void #4 Editorial update in Table 9.1.1-1: NF services provided by MB-SMF Changes in SA2#155 (to rev 1 (S2-2301453) agreed in SA2#154AHe): Resolving EN how to name the function by using “Resource sharing across broadcast MBS Sessions during network sharing”. Corrections/improvements are needed to make the changes consistent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #1 5.3.2.7 add description for NG-RAN to support the resource efficiency in network sharing 6.5.x new sub-clause for Associated Session ID 6.9.1 Add the Associated Session Id in MBS Session Context 7.3.x new clause for transport change in network sharing. 9.1.3.6, 9.3.2.2 and 9.4.3.2, add the new Associated Session ID in the related service operations #2 void #3 void. #4 9.1.1 editorial update |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.7, 6.5.X (new), 6.9.1, 7.3.X (new), 9.1.1, 9.1.3.6, 9.3.2.2, 9.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS 23.247 CR0176 and CR0165 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | New clause 6.X referenced in this CR is introduced in TS 23.247 CR0176 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

#### 5.3.2.7	NG-RAN

In addition to the functions defined in TS 23.501 [5], the NG-RAN performs the following functions to support MBS:

-	Management of MBS QoS flows via N2.

-	Delivery of MBS data packets for multiple UEs over radio using PTM or PTP.

-	Configuration of UE for MBS QoS flow reception at AS layer.

-	Control switching between PTM and PTP delivery per UE.

-	Support for multicast session service continuity during Xn Handover and N2 Handover.

-	Support group paging at multicast session activation over radio toward UEs in CM-IDLE state and CM-CONNECTED with RRC INACTIVE state.

-	Reception of MBS data packets from 5GC via shared MBS traffic delivery.

-	Support for efficient radio resource utilization for multiple broadcast MBS Sessions via multiple CNs to deliver the same broadcast content in the case of network sharing.

* * * * Next Changes * * * *

## 6.5	Identifiers

//No change//

### 6.5.x	Associated Session ID

In the case of network sharing, an Associated Session ID may be used as specified in clause 6.X. When the AF creates multiple broadcast MBS Sessions via different CNs to deliver the same content, it may provide the Associated Session ID which enables the NG-RAN to identify the multiple MBS Sessions delivering the same content.

Source Specific IP Multicast Address specified in clause 6.5.3 may be used as Associated Session ID..

Editor’s Note: For location dependent broadcast service, whether the Associated Session ID is sufficient for the NG-RAN to identify multiple broadcast MBS Sessions via different CNs delivering the same content is FFS.

* * * * Next Changes * * * *

### 6.9.1	MBS Session Context

The MBS Session Context contains all information describing a particular MBS session in the 5GS and is created in each node involved in the delivery of the MBS data.

The content of the Multicast MBS Session Context is described in Table 6.9.1-1.

Table 6.9.1-1: Multicast MBS Session Context

| Parameter | Description | NG-RAN | AMF | SMF | MB-SMF |
| --- | --- | --- | --- | --- | --- |
| State | State of MBS session ('Active' or 'Inactive' or 'Configured') | X (note 2) |  | X (note 2) | X |
| SSM (source specific IP multicast address) | IP multicast address identifying the MBS session. |  |  | X (note 1) | X (note 1) |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | x | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X | X (note 1) | X (note 1) |
| MB-SMF | The MB-SMF that handles the MBS session. |  | X | X |  |
| QoS information | QoS information of the MBS session. | X |  | X | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X (note 1) |  | X (note 1) | X (note 1) |
| NG-RAN Node ID(s) | NG-RAN nodes which are involved in the Multicast MBS session |  | X |  |  |
| AMF | The AMF(s) which are selected for the MBS session | X |  |  | X |
| IP multicast and source address for data distribution | IP addresses identifying the SSM user plane transport for shared delivery between MB-UPF and NG-RAN and for individual delivery between MB-UPF and UPF when the IP multicast transport is used. | X (note 1) |  | X (note 1) | X (note 1) |
| SMF | The SMF(s) that manages the associated PDU session. |  |  |  | X |
| UE ID | ID identifying the UE that successfully join the Multicast MBS session. For NG-RAN it is NGAP UE ID and for SMF it is SUPI. | X (note 3) |  | X (note 3) |  |
| IP address for distribution | The IP addresses and TEID of NG-RAN used for the user plane between NG-RAN and MB-UPF and between MB-UPF and UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) | X (note 1) |
| TEID for data distribution | The tunnel ID used for receiving the multicast data for shared delivery by NG-RAN and for individual delivery by UPF | X |  | X | X |
| PCF | The MB-PCF that provides policy control for the MBS session. |  |  |  | X (note 1) |
| NOTE 1: It is an optional parameter. NOTE 2: The value 'Configured' is not applicable for NG-RAN and SMF. NOTE 3: the UE ID is available within the UE Context which contains the MBS information. |  |  |  |  |  |

In Broadcast MBMS mode, an MBS Session Context is created in the NG-RAN, AMF, MB-SMF and MBSF as a result of the MBS Session Start procedure.

The content of the Broadcast MBS Session Context is described in Table 6.9.1-2.

Table 6.9.1-2: Broadcast MBS Session context

| Parameter | Description | NG-RAN | AMF | MB-SMF |
| --- | --- | --- | --- | --- |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) |
| AMF | The AMF(s) which are selected for the MBS session | X |  | X |
| MB-SMF | The MB-SMF that handles the MBS session. |  | X |  |
| QoS information | QoS information for the MBS Session, including the QoS parameters of QoS flows. | X |  | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X | X | X |
| NG-RAN Node ID(s) | NG-RAN nodes which are selected for the Broadcast MBS session |  | X |  |
| IP multicast address for data distribution | IP addresses identifying the user plane transport used for shared delivery between MB-UPF and NG-RAN when the IP multicast transport is used. | X (note 1) |  | X (note 1) |
| NG-RAN IP Address for data distribution | The IP address of NG-RAN used for the user plane between NG-RAN and MB-UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) |
| TEID for data distribution | The tunnel ID used for receiving the broadcast data for shared delivery by NG-RAN | X |  | X |
| PCF | The PCF that provides policy control for the MBS session. |  |  | X (note 1) |
| MBS FSA ID | MBS Frequency Selection Area (FSA) ID is used for broadcast MBS sessions to guide the frequency selection of the UE. | X |  | X |
| Associated Session ID | Associated Session ID is used by NG-RAN in network sharing to identify MBS sessions via different CNs transmitting the same content | X (note 1) |  | X (note 1) |
| NOTE 1: It is an optional parameter. |  |  |  |  |

* * * * Next Changes * * * *

### 7.3.X	Transport change for resource sharing across broadcast MBS Sessions during network sharing

Editor´s note: The procedures in this clause may be updated based on NG-RAN decisions.

The procedure in this clause is performed when one of the following events occurs under the condition that the resource sharing across broadcast MBS Sessions in network sharing was previously applied and the NG-RAN determined to receive only a single copy of MBS data as specified in clause 6.X:

-	A broadcast MBS session, from which the shared NG-RAN receives DL data stream, is released, and no DL data stream is received in the NG-RAN from the remaining broadcast MBS session(s) with the same Associated Session ID.

- 	When the shared NG-RAN fails to receive DL data stream from the CN (e.g., due to failure in the user plane), the NG-RAN attempts to get the DL data stream via another CN’s user plane.

Figure 7.3.X-1: Transport change for resource sharing across broadcast MBS Sessions during network sharing

1.	The NG-RAN selects another broadcast MBS Session that is delivering the same content via another CN to establish user plane, utilizing the broadcast MBS session context stored in the NG-RAN.

If unicast transport of N3mb applies, continue at step 3.

NOTE x: Which CN is to be selected is up to NG-RAN implementation.

2.	If multicast transport of N3mb applies, the NG-RAN joins the multicast group towards the LL SSM provided by the CN and continues at step 8.

3.	If unicast transport of N3mb applies, the NG-RAN allocates N3mb DL Tunnel Info, and sends N2 message (e.g., BROADCAST SESSION TRANSPORT REQUEST) to the AMF, including the MBS Session ID and the N3mb DL Tunnel Info.

4.	The AMF transfers the Namf_MBSBroadcast_ContextStatusNotify request to the MB-SMF, which contains the N2 message.

5.	The MB-SMF sends an N4mb Session Modification Request to the MB-UPF to allocate the N3mb point-to-point transport tunnel for a replicated MBS stream for the MBS Session. The MB-UPF sends N4mb Session Modification Response to the MB-SMF.

6.	The AMF forwards the received N2 information in N2 message (e.g., BROADCAST SESSION TRANSPORT RESPONSE) to the NG-RAN.

7.	The NG-RAN receives the media stream from the MB-UPF via N3mb multicast transport or unicast transport.

8.	The NG-RAN sends the received packets using the existing radio resource.

* * * * Next Changes * * * *

### 9.1.1	General

The following table illustrates the MB-SMF Services for MBS.

Table 9.1.1-1: NF services provided by MB-SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Nmbsmf_TMGI | Allocate | Request/Response | NEF, MBSF, AF |
|  | Deallocate | Request/Response | NEF, MBSF, AF |
|  | ContextStatusSubscribe |  | SMF |
| Nmbsmf_MBSSession | ContextStatusUnsubscribe | Subscribe/Notify | SMF |
|  | ContextStatusNotify |  | SMF |
|  | ContextUpdate | Request/Response | AMF, SMF |
|  | StatusSubscribe | Subscribe/Notify | MBSF, NEF, AF |
|  | StatusUnsubscribe |  | MBSF, NEF, AF |
|  | StatusNotify |  | MBSF, NEF, AF |
|  | Create | Request/Response | MBSF, NEF, AF |
|  | Update | Request/Response | MBSF, NEF, AF |
|  | Delete | Request/Response | MBSF, NEF, AF |

* * * * Next Changes * * * *

#### 9.1.3.6	Nmbsmf_MBSSession_Create service operation

Service operation name: Nmbsmf_MBSSession_Create

Description: Create a new multicast session or broadcast session, or for a location dependent MBS session, the part of the MBS session within a service area. Optionally subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (SSM or TMGI) or TMGI allocation request, MBS Service Type (multicast or broadcast).

Input, Optional: DNN, S-NSSAI, MBS service area, MBS Service Information (as defined in clause 6.14), Input Transport Address Request, MBS start time, MBS termination time. For a multicast session, indication that any UE may join, multicast session security context. For a broadcast session, MBS FSA ID(s). For subscription to notifications event ID(s), Notification Target Address, Request for location dependent MBS session, Associated Session ID for resource sharing across broadcast MBS Sessions during network sharing.

Output, Required: Result Indication.

Output, Optional: TMGI, NID, Expiry Time of the TMGI, Cause, MB-UPF tunnel info, MBS FSA ID(s), Area Session ID.

* * * * Next Changes * * * *

#### 9.3.2.2	Namf_MBSBroadcast_ContextCreate service operation

Service operation name: Namf_MBSBroadcast_ContextCreate

Description: This service operation is used to create the broadcast session context towards the AMF.

Inputs, Required: MBS Session ID, Broadcast service area, N2 container (e.g. MBS Session ID, MBS QoS profile, Broadcast service area).

Inputs, Optional: Area Session ID, Optional parameters in the N2 container (MBS IP Multicast Tunnel Info, Area Session ID, MBS FSA ID(s), Associated Session ID for resource sharing across broadcast MBS Sessions during network sharing).

Outputs, Required: Result Indication.

Outputs, Optional: N2 container (NG-RAN MBS Tunnel Info).

* * * * Next Changes * * * *

#### 9.4.3.2	Nnef_MBSSession_Create service operation

Service operation name: Nnef_MBSSession_Create

Description: Create a new multicast session or broadcast session, or for a location dependent MBS session, the part of the MBS session within a service area. Optionally implicitly subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (SSM or TMGI) or TMGI allocation request, MBS Service Type (broadcast or multicast).

Input, Optional: MBS service area (mandatory for broadcast. Can be Cell ID list and/or TAI list, or geographical area information, or civic address information), MBS start time, MBS stop time, MBS Service Information (as defined in clause 6.14), Input Transport Address Request, session state (Active/Inactive), for subscription to notifications notification target address. For a multicast session, indication that any UE may join the multicast session. Request for location dependent MBS session. For a broadcast session, MBS FSA ID(s), Associated Session ID for resource sharing across broadcast MBS Sessions during network sharing.

Output, Required: Result Indication.

Output, Optional: TMGI, NID, Cause, MB-UPF tunnel info, MBS FSA ID(s), Area Session ID.

* * * * End of Changes * * * *