3GPP TSG-WG SA2 Meeting #155	S2-2303472

# Athens, February 20 – 24, 2023		(revision of S2-2302580)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0675 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New NWDAF service supporting for AnLF assisted MTLF in accuracy monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph3 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce new NWDAF service to supprot AnLF assisted MTLF in accuracy monitoring, based on the conclusions for eNA_ph3 KI#1 in TR 23.700-81 and the agreed prposal S2-2301980 in SA2#154AHE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add general service in clause 7.1; Add general description about Nnwdaf_ MLModelMonitor Servie in clause 7.X.1; Add Nnwdaf_ MLModelMonitor_Subscribe service operation in clause 7.X.2; Add Nnwdaf_ MLModelMonitor_Unsubscribe service operation in clause 7.X.3; Add Nnwdaf_ MLModelMonitor_Notify service operation in clause 7.X.4; Add Nnwdaf_MLModelMonitor_Register service operation in clause 7.X.5; Add Nnwdaf_MLModelMonitor_Deregister service operation in clause 7.X.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality agreed based on conclusions in TR 23.700-81 and missing service on procedure for AnLF-assisted MTLF ML Models Accuracy Monitoring. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.X (NEW), 7.X.1(NEW), 7.X.2(NEW), 7.X.3(NEW), 7.X.4(NEW), 7.X.5(NEW), 7.X.6(NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 7	Nnwdaf Services Description

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_ MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Deregister |  | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |

# * * * * Next change * * * *

## 7.X	Nnwdaf_ MLModelMonitor Service

### 7.X.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe for ML model accuracy information monitored by an NWDAF (i.e., NWDAF containing AnLF). The service also enables the NWDAF containing AnLF registers the use and monitoring capability for an ML model into the model provider NWDAF, i.e., NWDAF containing MTLF.

### 7.X.2	Nnwdaf_ MLModelMonitor_Subscribe service operation

Service operation name: Nnwdaf_MLModelMonitor_Subscribe.

Description: Subscribes to NWDAF for ML model accuracy information with specific parameters.

Inputs, Required: (set of) Unique ML Model identifier(s), Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the ML model monitor subscription), Accuracy metrics to indicate the metrics to calculate the accuracy information, ML Model accuracy information period to indicate the reporting periodicity in which the information can be reported, Accuracy reporting threshold to indicate the reporting condition above which the accuracy information needs to be reported.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

### 7.X.3	Nnwdaf_ MLModelMonitor_Unsubscribe service operation

Service operation name: Nnwdaf_MLModelMonitor_Unsubscribe.

Description: The NF consumer unsubscribes to the NWDAF for ML Model accuracy information.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 7.X.4	Nnwdaf_MLModelMonitor_Notify service operation

Service operation name: Nnwdaf_MLModelMonitor_Notify.

Description: NWDAF notifies the ML Model accuracy information to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Notification Correlation Information, Set of

- the tuple (Unique ML model identifier, ML Model accuracy information).

ML Model accuracy information may include a deviation value which indicates the deviation of the predictions generated using the ML model(s) from the ground truth data, Network data when the deviation occurs (which can be used by the NWDAF containing MTLF for possible ML model retraining), Accuracy metrics as requested in Subscribe service operation.

Inputs, Optional: Validity period.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.X.5	Nnwdaf_MLModelMonitor_Register

Service operation name: Nnwdaf_MLModelMonitor_Register

Description: The consumer registers the use and monitoring capability for an ML model at an NWDAF containing MTLF

Inputs, Required: Consumer NF ID, Unique ML Model identifier

Inputs, Optional: Endpoint address of the Nnwdaf_MLModelMonitor_Subscribe service operation

Outputs, Required: ML model monitoring registration ID

Outputs, Optional: None

7.X.6	Nnwdaf_MLModelMonitor_Deregister

Service operation name: Nnwdaf_MLModelMonitor_Deregister

Description: The consumer deregisters, from an NWDAF containing MTLF, a previous MLModelMonitor registration, e.g., when the consumer is no longer using or monitoring the accuracy of the analytics generated using the ML model.

Inputs, Required: ML model monitoring registration ID

Inputs, Optional: None

Outputs, Required: None

Outputs, Optional: None

# * * * * End of changes * * * *