3GPP TSG-SA WG2 Meeting #155	S2-2303489

# February 20 – 24, 2023, Athens, Greece											 (revision of S2-2302749)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3913 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Parameters update for URSP in VPLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-85 clause 8.1.3 "Conclusions on which PLMN determines the VPLMN specific URSP Rules" includes the following conclusions for KI#1 - URSP in VPLMN. The scenario where the network provides the UE with URSP rules applicable in the VPLMN It is proposed to adopt the following interim conclusion principles: - The H-PCF provides VPLMN specific URSP Rules to the UE. - The H-PCF generates VPLMN specific URSP rules by taking Service Parameters from V-PCF or the V-AF into account. - The Service Parameters provided by the V-AF are either provided to: - V-NEF and stored in V-UDR so that the V-PCF can obtain the Service Parameters from V-UDR. - H-NEF and stored in the H-UDR. H-PCF retrieves the service parameters directly from the H-UDR. - The Service Parameters are AF guidance for URSP Rule determination for subscribers of a certain PLMN. Above conclusions has been reflected in the agreed CR 3712 for TS 23.502 during the UE Policy Association Establishment procedure, but it is possible that the Service Parameters will be updated after the UE Policy Association Establishment procedure, so it is reasonable to add the flow that V-PCF provides the updated Service Parameters to the HPCF for HPCF generating URSP rules applicable in the VPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §4.16.12.2 - Add description about Service Parameters provided by V-PCF to H-PCF during the the UE Policy Association Modification initiated by the VPCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No description is captured to influence the HPCF generating URSP rules applicable in the VPLMN, when the service parameters in VUDR is updated after the UE Policy Association Establishment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.16.12.2	UE Policy Association Modification initiated by the PCF

This procedure is used to update UE policy and/or UE policy triggers.

Figure 4.16.12.2-1: UE Policy Association Modification initiated by the PCF

This procedure concerns both roaming and non-roaming scenarios.

In the non-roaming case the V-PCF is not involved and the role of the H-PCF is performed by the PCF. In the roaming case, the H-PCF provides UE policy decision and provides the policy to the AMF via V-PCF.

1a and 1b.	If (H-)PCF subscribed to notification of subscriber´s policy data change or 5G VN Group Configuration (5G VN group data, 5G VN group membership) change and a change is detected, the UDR notifies that the subscriber´s policy data of a UE or 5G VN Group Configuration (5G VN group data, 5G VN group membership) has been changed.

The UDR notifies the (H-)PCF of the updated policy control subscription information profile via Nudr_DM_Notify (Notification correlation Id, Policy Data, either UE context policy control data or Policy Set Entry data or both, SUPI), or

The UDR notifies the (H-)PCF of the updated 5G VN Group Configuration (5G VN group data, 5G VN group membership) via Nudr_DM_Notify (Notification correlation Id, 5G VN Group Configuration, Internal-Group-Identifier), or

The (V-)UDR notifies the (V-)PCF of the updated policy control subscription information profile via Nudr_DM_Notify (Notification correlation Id, Policy Data, PolicySetEntry Data. PLMN ID).

The (V-)UDR notifies the (V-)PCF of the updated Service Parameters via Nudr_DM_Notify.

1c and 1d.	PCF determines locally that UE policy information needs to be sent to the UE.

2a and 2b.	The PCF makes the policy decision. If the group data is updated, the (H-) PCF checks the UE Policy Associations for those SUPIs within the Internal-Group-Id and may need to perform step 3 to step 9 for each UE Policy Association that needs to be updated with new UE Policies sent to each UE. In the non-roaming case, the PCF may subscribe to Analytics from NWDAF as defined in clause 6.1.1.3 of TS 23.503 [20].

2c and 2d.	In the roaming case, the V-PCF may provide the updated Service Parameters that is notified by the V-UDR as specified in clause 4.15.6.10 to the H-PCF using the Npcf_UEPolicyControl_Update Request. The H-PCF sends a response to the V-PCF.

3.	The (H-)PCF may create the UE policy container including UE policy information as defined in clause 6.1.2.2.2 of TS 23.503 [20]. In the case of roaming, the H-PCF may send the UE policy container in the Npcf_UEPolicyControl UpdateNotify Request. The H-PCF may provide updated policy control triggers for the UE policy association. If there is the received Service Parameters from the V-PCF in step 2, the H-PCF may take the Service Parameters obtained from V-PCF to generate URSP rules applicable in the VPLMN as specified in clause 4.15.6.10.

4.	The V-PCF sends a response to H-PCF using Npcf_UEPolicyControl UpdateNotify Response.

5.	The (V-)PCF provides the Policy Control Request Trigger parameters in the Npcf_UEPolicyControl UpdateNotify Request to the AMF. In the case of roaming, the V-PCF may also provide UE policy information to the UE. The V-PCF may also provide updated policy control triggers for the UE policy association to the AMF.

6.	The AMF sends a response to (V-)PCF.

Steps 7, 8 and 9 are the same as steps 8, 9 and 10 of procedure UE Policy Association Establishment in clause 4.16.11.

* * * * End of Changes * * * *