3GPP TSG-WG SA2 Meeting #155	S2-2303556

# Athens, Greece, February 20 – 24, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0927 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for URSP awareness |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Deutsche Telekom, Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-02-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in TR 23.700-85: If the UE can indicate the capability of reporting URSP rule enforcement, the 5GC indicates to the UE to report URSP rule enforcement to network, and if the UE URSP rule includes Connection Capabilities contained in the TD, when newly-appeared application traffic is matched to the TD during URSP evaluation, the UE reports the Connection Capabilities contained in the TD when the URSP rule is matched), then to the PCF for the PDU Session for policy control decisions. But the process to generate URSP Rules to the UE and corresponding PCC rules for the traffic matching the Traffic Description in the URSP Rule does not require the UE to send the URSP ID or the App ID. The PCF for the UE can derive the candidate RSD components using the PDU Session parameters sent by the UE, if no URSP Rule ID or App ID is sent as defined in Sol#9. The PCF knows the list of URSP Rules sent to the UE and its RSDs, this allows the PCF to generate PCC Rules to apply policies for a PDU Session established with preconfigured URSP Rules. Besides, the PCF for a PDU Session can notify the PCF for a UE about the detected unexpected traffic. Based on this information, the PCF for a UE can adjust the URSP rules. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add policy control decisions based on awareness of URSP rule enforcement for an application by using policy control decision based on awareness of URSP rule enforcement with UE assistance and policy control decision based on awareness of URSP rule enforcement without UE assistance as described above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PCC decisions based on awareness of URSP rule enforcement is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.x (new), 6.1.x.1 (new), 6.1.x.2 (new), 6.1.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.x	Policy control decisions based on awareness of URSP rule enforcement

6.1.x.1	General

Based on operator policy, the PCF may make policy control decisions based on awareness of URSP rule enforcement for an application by using the following mechanisms:

-	Policy control decision based on awareness of URSP rule enforcement with UE assistance: PCF may make PCC decision based on UE reported Connection Capabilities as illustrated in clause 6.1.x.2;

-	Policy control decision based on awareness of URSP rule enforcement without UE assistance: PCF may make PCC decision without involving UE based on application detection as illustrated in clause 6.1.x.3.

Editors’ notes: the description on PCC rule generation based on pre-configured URSP rules is FFS.

6.1.x.2	Policy control decisions based on awareness of URSP rule enforcement with UE assistance

The PCF may obtain UE reporting of URSP rule enforcement as described in clause 6.6.2.x.

Based on the received URSP rule enforcement information, the PCF for the PDU Session may interact with the PCF for the UE. The PCF for the UE may adjust the URSP rules based on the notification from the PCF for a PDU Session e.g. when the PCF for the UE determines that the UE does not have up-to-date URSP rules.

NOTE 1: The PCF can combine UE reporting of URSP rule enforcement with analytics information (see clause 6.1.x.3)

6.1.x.3	Policy control decisions based on awareness of URSP rule enforcement without UE assistance

The PCF for the UE may subscribe to Statistics for traffic monitoring of known traffic according to provisioned URSP rule(s). If the PCF for a UE is notified with traffic which is not expected according to a URSP rule as described in TS 23.288 [24], the PCF for a UE may adjust the URSP rules when unexpected application traffic is detected.

Editors’ notes: Other details of policy control decision for awareness of URSP rule enforcement is FFS.

# * * * * End of changes * * * *