3GPP TSG-SA2 Meeting #155	S2-2303595

# 20th – 24th February, 2023, Athens, Greece								   revision of S2-2302837

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3899 | rev | 5 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN identifiers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Interdigital, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in TR 23.700-88, PIN ID needs to be supported by 5G. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe identifers for PIN. The 5G System already supports management of 5G VN Group identification and membership through External Group ID and Internal Group ID. A PIN also has exactly the same requirements (that can be already fullfilled by 5G VN management): - Managing memebership of PIN - 5G LAN type services for communication needs between different PEGCs and PEMCs that are part of a PIN - Need for dynamic management of PIN membership and identification - Need for dynamic management of policies and QoS authorization for the group members It is proposed to use the External Group ID and Internal Group ID to identify a PIN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Identifiers, including PIN ID, are missing for supporting PIN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 5.X.d |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-5 provides below changes: - The External PIN ID changed to PIN ID - The Internal PIN ID is removed and clarified that PIN ID is managed in the same way as the External Group ID and the 5GC determines the Internal Group ID from the PIN ID. |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 5.X	Support for Personal IoT Networks

### 5.X.d	Identifiers

A PIN pre-configured or created by an AF is identified by a PIN ID, which is unique in the PLMN. The PIN ID is managed in the same way as the External Group ID. The 5GC determines the Internal Group ID from the PIN ID (i.e. the External Group ID).

# * * * * End of change * * * *