3GPP TSG-SA2 Meeting #155	S2-2303652

# Athens, Greece, 20th Feb 2023 - 24th Feb 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0616 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#7 - 23.288 CR for adding UE Transmission Latency Performance Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, OPPO, SK Telecom, Samsung, CATT, ETRI, NTT DOCOMO, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Proposal to update TS 23.288 to add a new analytic as concluded in TR 23.700-80 KI#7 5GS Assistance to Federated Learning Operation, where the conclusions state the following: AF can request NWDAF (via NEF if needed) to provide existing, enhanced or new analytics to assist with federated learning operation. NWDAF Analytics services will be enhanced as follows: - New latency performance analytics are introduced for NWDAF. NOTE 1: The specific input and output parameters for the latency performance analytics among the group of UEs to assist the Application AI/ML FL operation will be determined during the normative phase. It is concluded that the AF as a consumer can request NWDAF to get a new Analytics ID on UE latency performance analytics. In revision 2, considering this editor’s note “It is FFS if this analytics needs more input data.” and as “The UE latency performance refers to a time delay for completing the transmission of a specific data volume from UE to AF, or from AF to UE.” the following inputs from AF are proposed to measure the time delay needed for completing the transmission of a specific data volume: - Transmitted data volume - Transmission time (This may be needed for the model training) The relevant EN is removed. And, since it seems that there will an analytic/statistic per time-slot, then the “UE latency performance (1...max)” is redundant output, so it is removed and the corresponding EN is removed. In addition, the EN saying “It is FFS how NWDAF will use those parameters: expected number of repeating data transmissions and expected time interval between data transmissions” is removed with following clarification: - If those parameters are not contained in the request, NWDAF analyses the UE latency performance considering only one time transmission of the given UL/DL data volume over the given analytics target periods (e.g. the UE latency performance would be 1ms to transmit 10MB traffic anytime between 10:00-10:10, and it would be 2ms to transmit 10MB traffic anytime between 11:00-11:10) - If one of those parameters is contained in the request, NWDAF analyses the average value of the UE latency performance of each transmission considering the number of repeating transmissions for the given UL/DL data volume over the given analytics target periods. (e.g. the average UE latency performance would be 1.3ms to transmit 10 times of 10MB traffic during 10:00-10:10, it would be 2.2ms to transmit 10 times of 10MB traffic during 11:00-11:10) The EN saying “It is FFS if Aggregate latency performances can be output of this analytics” is removed with the following clarification: the latency performance of the UE group or a subset of UEs in the group will be essential to support the member selection and the AIML operation. This can help the application server determine how best to apply federated learning within the UEs in the group (e.g. synchronous FL or asynchronous FL). Therefore, the classified UE latency performance will assist the NEF and AF in member selection and performance of the federated learning more efficiently. The classification should be done by the NWDAF. To make it more clarified, ‘Aggregate latency performances’ in Table 6.x.3-1 and Table 6.x.3-2 is renamed to ‘Classified latency performances for a list of UEs’. The list of UEs are indicated in the request of service consumer for federated learning. The classification is with respect to one or more reporting thresholds. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new clause under 6 to define a new analytic on UE latency performance analytics |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions agreed in TR 23.700-88 (AIMLsys) KI#7 cannot be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.x.1 (new), 6.x.2 (new), 6.x.3 (new), 6.x.4 (new), 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_AIMLsys |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In revision 2, the remaining editor’s notes are resolved |  |  |  |  |  |  |  |  |

********** Start of Changes ***************

********** Next Change (all new text) ***************

### 6.x E2E data volume transfer time analytics

#### 6.x.1 General

The clause 6x describes how NWDAF can provide E2E data volume transfer time analytics, in the form of statistics or predictions or both, to a service consumer. NWDAF collects E2E data volume transfer time related input data from 5GC NFs, OAM and AF. The consumer can either subscribe to analytics notifications (i.e., a Subscribe-Notify model) or request a single notification (i.e., a Request-Response model).

The E2E data volume transfer time refers to a time delay for completing the transmission of a specific data volume from UE to AF, or from AF to UE. If a target number of repeating data transmissions or a target time interval between data transmissions is given, the E2E data volume transfer time can be provided as an average value of every data volume transmission time performance within the Analytics target period. The E2E data volume transfer time analytics may be used to assist an AF hosting AI/ML-based services, e.g., for member selection of federated learning.

The E2E data volume transfer time analytics may be provided as defined in clause 6. x.3 for a UE individually or a list of UEs.

More than one E2E data volume transfer time classes might be assigned by operator or AF for a group of UEs. The UEs might be classified to high-, medium-, and low-latency classes with respect to the threshold(s) of the corresponding class.

The service consumer may be an NF (e.g. AF, or NEF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = "E2E data volume transfer time ".

-	Target of Analytics Reporting: a single UE (SUPI/GPSI) or a group of UEs (a list of SUPIs/GPSIs).

-	Analytics Filter Information optionally including:

-	DNN;

-	S-NSSAI;

-	Application ID;

-	Area of Interest (AOI(s)): restricts the scope of the E2E data volume transfer time analytics to the provided area;

-	An optional list of analytics subsets that are requested (see clause 6. x.3);

-	Data Volume UL/DL: indicates a specific data volume transmitted once from UE to AF and/or from AF to UE;

-	QoS requirements (e.g., 5QI, QoS Characteristics);

-	Optionally, either a target number of repeating data transmissions or a target time interval between data transmissions within the Analytics target period;

-	Optionally, a request for geographical distribution (i.e., the AoIs) of the UEs.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, preferred level of accuracy of the analytics.

-	Optional preferred order of results for the list of E2E data volume transfer time:

-	ordering criterion: "E2E data volume transfer time",

-	order: ascending or descending.

-	Optionally, Reporting Thresholds, which indicate conditions on the level to be reached for respective analytics subsets (see clause 6.X.3)

-	Optionally, maximum number of UEs.

#### 6.x.2 Input Data

The NWDAF supporting analytics on E2E data volume transfer time shall be able to collect UE performance information from AF, OAM and 5GC NFs.

More detailed information collected by the NWDAF from the OAM is defined in the Table 6.x.2-1, from relevant 5GC NFs (i.e. UPF, SMF, AMF) is defined in Table 6.x.2-2, and from AF is defined in Table 6.x.2-3.

Table 6.x.2-1: Input data from OAM related to E2E data volume transfer time

| Information | Source | Description |
| --- | --- | --- |
| RAN part delay | OAM TS 28.554 [10] | Average packet transmission delay through the RAN part to the UE, per timeslot, per 5QI and per S-NSSAI. |
| Timestamp | OAM | A time stamp associated with the collected information. |
| Reference Signal Received Power | OAM (see NOTE 1) | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM (see NOTE 1) | The per UE measurement of the received radio quality in a network cell the specified area where UE(s) will be located, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM (see NOTE 1) | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |
| RAN Throughput for DL and UL | OAM (see NOTE 1) | The per UE measurement of the throughput for DL and UL as specified in clauses 5.2.1.1 and 5.4.1.1 of TS 37.320 [20] |
| RAN Packet delay for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet delay for DL and UL, including per QCI per UE packet delay as specified in clauses 5.2.1.1 of TS 37.320 [20] and per DRB per UE packet delay as specified in clauses 5.4.1.1 of TS 37.320 [20] |
| RAN Packet loss rate for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet loss rate for DL and UL, including the per QCI per UE packet loss rate as specified in clauses 5.2.1.1 of TS 37.320 [20] and the per DRB per UE packet loss rate as specified in clauses 5.4.1.1 of TS 37.320 [20] |
| Distribution of UL/DL/round trip packed delay between PSA UPF and UE | OAM | The distribution of UL/DL/round trip packet delay between PSA UPF and UE as captured in TS 28.552 [8]. |
| NOTE 1: Per UE measurement for a specific UE from OAM (via MDT), is as captured in clause 6.2.3.1. |  |  |

NWDAF subscribes the network data from OAM in the Table 6.X.2-1 by using the services provided by OAM as described in clause 6.2.3.

NOTE 1:	Whether the UE(s) is supporting a Slice or not can be checked by retrieving the registered AMF details from UDM or by asking AMF about what Slice is used by the UE(s) at the current registration (Alternatively, if NSACF is deployed, NSACF can provide a report on what slices are used by the UE(s))

NOTE 2:	User consent checking from UDM can apply for these analytics.

Table 6.x.2-2: Service Data from 5GC NFs for E2E data volume transfer time analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| UE location | AMF, GMLC | Location of the UE(s) needs to be selected via AMF if the application needs to be started at the same time. If the AoI indicated by the AF is a finer granularity area than the Cell level, the current location of the UE(s) needs to be selected via GMLC instead. |
| UE ID | AMF | (list of) SUPI(s). |
| QFI | SMF | QoS Flow Identifier. |
| QoS flow Packet Delay | SMF, UPF | The observed Packet delay for UL/DL/round trip directions; |

Table 6.x.2-3: Service Data from AF for E2E data volume transfer time analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | AF | Timestamp of the collected information. |
| Application ID | AF | Identifier of the application at the AF. |
| UE ID(s) | AF | Internal or External UE IDs (i.e. SUPI or GPSI, respectively). |
| Transmitted data volume | AF | The volume of the transmitted data |
| Transmission time duration | AF | The time duration (start and end time) needed for sending a volume of data |
| Server location | AF | The location where the server (AF) is located |

Editor's note:	Which AF event can be used to collect Server location is FFS.

#### 6.x.3 Output Analytics

The NWDAF supporting E2E data volume transfer time analytics provides the analytics results to consumer NFs, e.g., AF, or NEF. The analytics results provided by the NWDAF could be E2E data volume transfer time statistics as defined in Table 6.X.3-1 or predictions as defined in table 6.x.3-2.

Table 6. x.3-1: E2E data volume transfer time statistics

| Information | Description |  |  |
| --- | --- | --- | --- |
| UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. a list of UEs for which the statistic applies. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |  |  |
| > Time slot start | Time slot start within the Analytics target period. |  |  |
| > Duration | Duration of the time slot. |  |  |
| > E2E data volume transfer times (1…max) | List of E2E data volume transfer time per UE Max. is the number of UEs, if applicable. |  |  |
| >> Application ID | Identifies the application in use during the time slot |  |  |
| >> DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |  |  |
| >> UE location | Indicating the UE location information when the UE service is delivered. |  |  |
| >> DNN | DNN for the PDU Session which contains the QoS flow. |  |  |
| >> S-NSSAI | Identifies the Network Slice used to access the Application. |  |  |
| >> Validity period | The validity period for the E2E data volume transfer time statistics as defined in clause 6.1.3. |  |  |
| >> Spatial validity | Area where the E2E data volume transfer time statistics applies |  |  |
| >> E2E data volume transfer time UL | E2E data volume transfer time UL indicators Statistics of E2E data volume transfer time for uplink over the Analytics target period (e.g. average, variance). |  |  |
| >>> Data volume UL | Indicates uplink data volume used to derive E2E data volume transfer time UL |  |
| >> E2E data volume transfer time DL | E2E data volume transfer time DL indicators Statistics of E2E data volume transfer time for downlink over the Analytics target period (e.g. average, variance). |  |
| >>> Data volume DL | Indicates downlink data volume used to derive E2E data volume transfer time DL |  |
| > Classified E2E data volume transfer times for a list of UEs (NOTE 1) | Classified E2E data volume transfer time statistics for multiple UEs with respect to one or more reporting thresholds (e.g., NWDAF may provide the ratio of UEs that have reached certain Reporting Threshold(s)). The list of UEs is indicated in the request of service consumer. |  |  |
| >> E2E data volume transfer time classes (1…max) (NOTE 2) | List with group of UEs classified by ranges of E2E data volume transfer time |  |  |
| >>> UE ID(s) | Identifies the UE(s) in the latency class with respect to the threshold of the corresponding latency class. |
| >>> Ratio of UEs per E2E data volume transfer time class | Ratio of UEs |  |  |
| >> Validity period | The validity period for the E2E data volume transfer time statistics as defined in clause 6.1.3. |  |  |
| >> Spatial Validity Condition | Area where the E2E data volume transfer time analytics applies. |  |  |
| > Geographical distribution of the UE(s) |  | If requested, a list of UEs per location information |  |  |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The number of latency classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |  |  |

Table 6.x.3-2: E2E data volume transfer time predictions

| Information |  | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| UE ID or list of UE IDs (1..SUPImax) |  |  | Identifies a UE or a group of UEs, e.g. a list of UEs for which the statistic applies. |  |  |
| Time slot entry (1..max) |  | List of time slots during the Analytics target period. |  |  |  |
| > Time slot start |  | Time slot start within the Analytics target period. |  |  |  |
| > Duration |  | Duration of the time slot. |  |  |  |
| > E2E data volume transfer times (1…max) |  | List of E2E data volume transfer time per UE. Max. is the number of UEs, if applicable. |  |  |  |
| >> Application ID |  | Identifies the application in use during the time slot |  |  |  |
| >> DNAI |  | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |  |  |  |
| >> UE location |  | Indicating the UE location information when the UE service is delivered. |  |  |  |
| >> DNN |  | DNN for the PDU Session which contains the QoS flow. |  |  |  |
| >> S-NSSAI |  | Identifies the Network Slice used to access the Application. |  |  |  |
| >> Validity period |  | The validity period for the E2E data volume transfer time prediction as defined in clause 6.1.3. |  |  |  |
| >> Spatial validity |  | Area where the E2E data volume transfer time prediction applies |  |  |  |
| >> E2E data volume transfer time UL |  | E2E data volume transfer time indicators Predictions of E2E data volume transfer time for uplink over the Analytics target period (e.g. average, variance). |  |  |  |
| >>> Data volume UL |  |  |  | Indicates uplink data volume used to derive E2E data volume transfer time UL |  |
| >> E2E data volume transfer time DL |  |  |  | E2E data volume transfer time DL indicators Predictions of E2E data volume transfer time for downlink over the Analytics target period (e.g. average, variance). |  |
| >>> Data volume DL | Indicates downlink data volume used to derive E2E data volume transfer time DL |
| > Classified E2E data volume transfer times for a list of UEs (NOTE 1) |  | Classified E2E data volume transfer time prediction for multiple UEs with respect to one or more reporting thresholds (e.g., NWDAF may provide the ratio of UEs that have reached certain Reporting Threshold(s)). The list of UEs is indicated in the request of service consumer. |  |  |  |
| >> E2E data volume transfer time classes (1…max) (NOTE 2) |  | List with group of UEs classified by ranges of E2E data volume transfer time |  |  |  |
| >>> UE ID(s) |  |  | Identifies the UE(s) in the latency class with respect to the threshold of the corresponding latency class. . |  |  |
| >>> Ratio of UEs per E2E data volume transfer time class |  | Ratio of UEs |  |  |  |
| >> Validity period |  | The validity period for the E2E data volume transfer time prediction as defined in clause 6.1.3. |  |  |  |
| >> Spatial Validity Condition |  | Area where the E2E data volume transfer time analytics applies. |  |  |  |
| > Geographical distribution of the UE(s) |  |  | If requested, a list of UEs per location information |  |  |
| Confidence |  | Confidence of this prediction. |  |  |  |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The number of latency classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |  |  |  |  |

NOTE:	When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics latency performance result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE.

#### 6.x.4 Procedures

The NWDAF can provide E2E data volume transfer time analytics to a 5GC NF (e.g., AF, or NEF).

Figure 6.x.4-1: Procedure for E2E data volume transfer time  analytics

1.	The Consumer NF, e.g., AF, or NEF, requests or subscribes to analytics for E2E data volume transfer time analytics from NWDAF (possibly via NEF in case the consumer NF is AF) and provides the input information as specified in 6.x.1 to 5GC.

2a-b.	The NWDAF subscribes the service data from AMF in Table 6.x.2-2 using Namf_EventExposure_Subscribe service for collecting UE location(s) for a UE or a group of UEs.

NOTE:	If NWDAF requires UE location information with finer granularity than TA/cell, then NWDAF collects the location data from GMLC instead of AMF.

2c.	NWDAF subscribes to service data from SMF in Table 6.x.2-2 by invoking Nsmf_EventExposure_Subscribe (Event ID, SUPI(s) or Application ID).

In order to provide the requested analytics, the NWDAF subscribes to information of the UE and may subscribe to N4 Session related input data from SMFs as defined in Table 6.x.2-2.

2d-e.	N4 related input data is provided by UPF to SMF.

2f.	SMF provides the requested input data to NWDAF.

2g-h.	The NWDAF may subscribe to input data in Table 6.x.2-1 from the OAM according to the data collection principles from the OAM described in clause 6.2.3.

2i-j			The NWDAF may subscribe the service data from AF in the Table 6.x-3 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service as defined in TS 23.502 [3].

3.	The NWDAF derives requested analytics, in the form of latency performance statistics or predictions or both.

4.	The NWDAF provides requested latency performance analytics to the NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-7.	If the NF subscribed to E2E data volume transfer time analytics at step 1, when the NWDAF generates new analytics, it notifies the new generated analytics to the consumer.

********** Next Change ***************

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| E2E data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |

********** End of Changes ***************